3GPP TSG-RAN2 Meeting #109bis Electronic 	R2- 2002559

# 20 April – 24 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR |  | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SR Trigger for Sidelink CSI Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When both data from SCCH and Sidelink CSI Reporting MAC CE is available for transmission, data from SCCH is prioritised. So if SL resources are available for new transmission corresponding to a destination, sidelink CSI Reporting MAC CE for SL-CSI reporting of a destination may or may not be mutliplexed in the MAC PDU of that destination Even if the Sidelink CSI Reporting MAC CE of that destination is mutliplexed in the MAC PDU, the transmission in available SL resource may overlap with UL and SL transmission may be dropped due to prioritisation rule. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the MAC entity has SL resources allocated for new transmission and SL resources can accommodate the Sidelink CSI Reporting MAC CE plus its suheader as a result of logical channel prioritization and SL transmission in the SL resources is not dropped due to prioritisation between SL and UL: Sidelink CSI Reporting MAC CE is generated and triggered SL-CSI reporting is cancelled |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Sidelink CSI Reporting will be delayed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.22.1.7	CSI Reporting

The Sidelink Channel State Information (SL-CSI) reporting procedure is used to provide a peer UE with sidelink channel state information as specified in clause 8.5 of TS 38.214 [7].

The MAC entity shall for each pair of the Source Layer-2 ID and the Destination Layer-2 ID:

1>	if the SL-CSI reporting has been triggered by a SCI and not cancelled:

2>	if the MAC entity has SL resources allocated for new transmission and SL resources can accommodate the Sidelink CSI Reporting MAC CE plus its suheader as a result of logical channel prioritization and SL transmission in the SL resources is not deperioritized as a result of prioritisation between SL and UL (as specified in 5.22.1.3.1):

3>	instruct the Multiplexing and Assembly procedure to generate a Sidelink CSI Reporting MAC CE as defined in clause 6.1.3.35;

3>	cancel the triggered SL-CSI reporting.

2>	else if the MAC entity has been configured by RRC to transmit using a SL-RNTI or SLCS-RNTI:

3>	trigger a Scheduling Request.

END OF CHANGES