3GPP TSG RAN WG2 Meeting #109bis-e                                                    R2-2002601

# Electronic meeting, 20th – 30th April 2020

Agenda item:		6.11.1

Source:			Intel Corporation

Title:		Report of email discussion [Post109e#42][PowSav] UE capabilities

Document for:	 	Discussion and decision

# Introduction

This contribution discusses the new UE capabilities required to specify the features defined for Rel-16 UE power saving WI considering the related discussion paper [1] and CR [2] submitted in RAN2#109-e meeting. For simplicity, the baseline CR for discussion in this email discussion is also included in Annex A.

- [Post109e#42][PowSav] UE capabilities (Intel)

Discuss RAN2 related UE capabilities for Power Saving

Intended outcome: Set of agreeable proposals.  Running CR capturing the potential agreements can be provided by Intel

The deadline of this email discussion #42 is 2020-04-08 23:59 PST.

Note that this email discussion will focus on RAN2-driven features not to overlap with the ongoing RAN1 email discussion [100e-NR-Rel-16-UEFeatures] where the following features are discussed:

| Index | Feature group | Components |
| --- | --- | --- |
| 19-1 | DRX Adaptation | - PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected outside Active time - Configured P-CSI report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON |
| 19-1a | DRX Adaptation | - Indication of UE dormancy-like behaviour by DCI format 2_6 |
| 19-2 | Cross Slot Scheduling | - Dynamic indication of minimum scheduling offset by DCI format 0_1 and 1_1 - minimumSchedulingOffset configuration for PDSCH and aperiodic CSI-RS triggering offset - minimumSchedulingOffset configuration for PUSCH |
| 19-3 | Maximum MIMO Layer Adaptation | - Maximum number of MIMO layer configuration per DL BWP |
| 19-4a | UE assistance information | - Preferred k0/k2 - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots |

# Pase-1 discussion

## New UE capabilities on RAN2 driven features for Rel-16 PWS WI

### Discussion point 1

- For new UE assistance information that provides UE’s preference to the C-DRX configuration, a new UE capability (referred e.g. as drx-Preference) is defined to indicate its preference on DRX parameters for power saving in RRC_CONNECTED. Do you agree with previous statement?

- Yes. If so, this new capability can be defined in section 4.2.2 “General parameters” of TS 38.306. Please indicate if you prefer specifying it in a different (sub)section.

- No. If so, please explained your suggestion and motivation.

| Company’s name | Option | Company’s comments, if any |
| --- | --- | --- |
| OPPO | Option a | Like the overheating, we can put this parameter in “General parameters” |
| Ericsson | Option a |  |
| Qualcomm | Option a | We prefer to have all power saving related parameters defined in a new section 4.2.x “Power saving parameters”. If that is not agreeable, we prefer to define all of them in Section 4.2.2. |
| Huawei | Option a | Similar as “overheatingInd”. |
| Intel | Option a | We do not see any strong reason to create a new section 4.2.x for power saving parameters and have slightly preference on capturing it on section 4.2.2 where other RAN2 features related to UE assistance were also captured. Moreover, network configures the usage of each UE assistance for PWS independently, therefore different UE capabilities looks also preferable. |
| CATT | Option a | Prefer to put it in “General parameters”. |
| ZTE | Option a |  |
| vivo | Option a | We prefer to put it in “General parameters” as in overheating. |
| Xiaomi | Option a | Yes, we can put it in “General parameters”, like the “ delayBudgetReporting “which is also used by the network for DRX adaptation. |
| Apple | Option a | Prefer to group it under “General Parameters” |

### Discussion point 2

- For new UE assistance information that provides UE’s preference to the maximum aggregated bandwidth, a new UE capability (referred e.g. as maxBW-Preference) is defined to indicate its preference on the maximum aggregated bandwidth for power saving in RRC_CONNECTED. Do you agree with previous statement?

- Yes. If so, this new capability can be defined in section 4.2.7.10 “Phy-Parameters” of TS 38.306. Please indicate if you prefer specifying it in a different (sub)section.

- No. If so, please explained your suggestion and motivation.

| Company’s name | Option | Company’s comments, if any |
| --- | --- | --- |
| OPPO | Option a with preference | We prefer to put it in section 4.2.2 “General parameters”. it would be good that all the UE assistance information related to power saving should be put in the same place. |
| Ericsson | Option a | We do not have strong view, but agree with OPPO that this capability could be placed under “general parameters” similar as overheating |
| Qualcomm | Option a | We prefer to have all power saving related parameters defined in a new section 4.2.x “Power saving parameters”. If that is not agreeable, we prefer to define all of them in Section 4.2.2. |
| Huawei | Option a | Agree with OPPO that all the UE assistance information related to power saving could be put in “general parameters”. |
| Intel | Option a | We are also ok if companies prefer keeping this UE capability in section 4.2.2 “General parameters” |
| CATT | Option a | Slightly prefer to put it in “General parameters”. |
| ZTE | Option a | We prefer to define all the power saving related parameters in section 4.2.2. |
| vivo | Option a | We also agree that all UE assistance information should be put together, in “general parameters”. |
| Xiaomi | Option a | We do not have strong view, but slightly prefer to put it in 4.2.7.10 “Phy-Parameters”. |
| Apple | Option a | Prefer to group it under “General Parameters” |

### Discussion point 3

- For new UE assistance information that provides UE’s preference to the maximum number of secondary component carriers, a new UE capability (referred e.g. as maxCC-Preference) is defined to indicate its preference on the maximum number of secondary component carriers for power saving in RRC_CONNECTED. Do you agree with previous statement?

- Yes. If so, this new capability can be defined in section 4.2.7.10 “Phy-Parameters” of TS 38.306. Please indicate if you prefer specifying it in a different (sub)section.

- No. If so, please explained your suggestion and motivation.

| Company’s name | Option | Company’s comments, if any |
| --- | --- | --- |
| OPPO | Option a with preference | We prefer to put it in section 4.2.2 “General parameters”. it would be good that all the UE assistance information related to power saving should be put in the same place. |
| Ericsson | Option a | We do not have strong view, but agree with OPPO that this capability could be placed under “general parameters” similar as overheating |
| Qualcomm | Option a | We prefer to have all power saving related parameters defined in a new section 4.2.x “Power saving parameters”. If that is not agreeable, we prefer to define all of them in Section 4.2.2. |
| Huawei | Option a | Agree with OPPO that all the UE assistance information related to power saving could be put in “general parameters”. |
| Intel | Option a | We are also ok if companies prefer keeping this UE capability in section 4.2.2 “General parameters” |
| CATT | Option a | Slightly prefer to put it in “General parameters”. |
| ZTE | Option a | We prefer to define all the power saving related parameters in section 4.2.2. |
| vivo | Option a | Similar as above questions. |
| Xiaomi | Option a | We do not have strong view, but slightly prefer to put it in 4.2.7.10 “Phy-Parameters”. |
| Apple | Option a | Prefer to group it under “General Parameters” |

### Discussion point 4

- For new UE assistance information that provides UE’s preference to the maximum number of MIMO layers, a new UE capability (referred e.g. as maxMIMO-LayerPreference) is defined to indicate its preference on the maximum number of MIMO layers for power saving in RRC_CONNECTED. Do you agree with previous statement?

- Yes. If so, this new capability can be defined in section 4.2.7.10 “Phy-Parameters” of TS 38.306. Please indicate if you prefer specifying it in a different (sub)section.

- No. If so, please explained your suggestion and motivation.

| Company’s name | Option | Company’s comments, if any |
| --- | --- | --- |
| OPPO | Option a with preference | We prefer to put it in section 4.2.2 “General parameters”. it would be good that all the UE assistance information related to power saving should be put in the same place. |
| Ericsson | Option a | - We are fine with the proposed capabilities, but if any “bundling” should be discussed, it should perhaps have been bundling of UE assistance for maxCC, maxBW and maxMIMO, i.e. bundle them into a separate UE capability. Especially maxCC and maxBW are two flavours of the same/similar thing. - Again, no strong view an where to put it, but MIMO is perhaps more a PHY feature. |
| Qualcomm | Option a | We prefer to have all power saving related parameters defined in a new section 4.2.x “Power saving parameters”. If that is not agreeable, we prefer to define all of them in Section 4.2.2. |
| Huawei | Option a | Agree with OPPO that all the UE assistance information related to power saving could be put in “general parameters”. |
| Intel | Option a | We are also ok if companies prefer keeping this UE capability in section 4.2.2 “General parameters” |
| CATT | Option a | Slightly prefer to put it in “General parameters”. |
| ZTE | Option a | We prefer to define all the power saving related parameters in section 4.2.2. |
| vivo | Option a | Similar as above questions. |
| Xiaomi | Option a | We do not have strong view, but slightly prefer to put it in 4.2.7.10 “Phy-Parameters”. |
| Apple | Option a | Prefer to group it under “General Parameters” |

### Discussion point 5

- For new UE assistance information that provides UE’s preference to transition out of RRC_CONNECTED, a new UE capability (referred e.g. as release-Preference) is defined to indicate its preference assistance information to transition out of RRC_CONNECTED for power saving. Do you agree with previous statement?

- Yes. If so, this new capability can be defined in section 4.2.2 “General parameters” of TS 38.306. Please indicate if you prefer specifying it in a different (sub)section.

- No. If so, please explained your suggestion and motivation.

| Company’s name | Option | Company’s comments, if any |
| --- | --- | --- |
| OPPO | Option a |  |
| Ericsson | Option a |  |
| Qualcomm | Option a | We prefer to have all power saving related parameters defined in a new section 4.2.x “Power saving parameters”. If that is not agreeable, we prefer to define all of them in Section 4.2.2. |
| Huawei | Option a |  |
| Intel | Option a |  |
| CATT | Option a |  |
| ZTE | Option a |  |
| vivo | Option a | Similar as above questions. |
| Xiaomi | Option a | Putting it in 4.2.2 “General parameters” is OK. |
| Apple | Option a | Prefer to group it under “General Parameters” |

### Discussion point 6

- For the relaxed measurement feature, a new UE capability is defined for the relaxed measurement feature without signalling to the gNB, i.e. gNB will not know whether a UE supports or not this feature. Do you agree with previous statement?

- Yes. If so, this new capability can be defined in section 5 “Optional features without UE radio access capability parameters” of TS 38.306. Please indicate if you prefer specifying it in a different (sub)section.

- No. If so, please explained your suggestion and motivation.

| Company’s name | Option | Company’s comments, if any |
| --- | --- | --- |
| OPPO | Option a |  |
| Ericsson | Option a | - We have been keen to have the option to only configure “low mobility” trigger. It is our understanding that NW does not have to provide IOT test opportunities for “not-at-cell-edge”, because this is an Idle/Inactive. If this is an incorrect assumption, we would be happy to know. - RAN4 is still discussing RRM relaxation methods and it is not clear (to me) if this actually will influence the scope of RRM relaxation capability. - Formally we will have optional/mandatory discussions/agreements later, but by putting this in section 5 it seems to optional. However we are fine with this. |
| Qualcomm | Option a |  |
| Huawei | Option a |  |
| Intel | Option a |  |
| ZTE | Option a |  |
| CATT | Option a |  |
| vivo | Option a | We agree with Option a. Regarding Ericsson’s comments: - We donot see any difference between “low mobility” and “not-at-cell-edge”. - RAN4 have already made the conclusion on the relaxation methods in last RAN4 meeting. |
| Xiaomi | Option a |  |
| Apple | Option a |  |

## Characteristics of the new capabilities on RAN2-driven features for Rel-16 PWS WI

### Discussion point 7

- For the new capabilities discussed in previous discussion points, indicate whether you agree or not with the suggested characteristics (that are added in the 2nd column) associated to the UE, mandatory/optional features, and applicability for FDD/TDD and for FR1/FR2 equally applicable for FR1/FR2. If not, please explained your suggestion and motivation. Note that for simplicity, a 1st general row is added in case you agree with all the suggested values below (in which case you only indicate so there 😊).

| Discussion point # | Characteristic | Agree? | Company’s name and comments, if any |
| --- | --- | --- | --- |
| All inputs in below rows |  | Yes | Intel, CATT, ZTE, Xiaomi |
| 1) drx-Preference | Per: UE | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | M: No | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | FDD-TDD DIFF: No | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | FR1-FR2 DIFF: No | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
| 2) maxBW-Preference | Per: UE | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | M: No | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | FDD-TDD DIFF: No | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | FR1-FR2 DIFF: No | Yes | OPPO, ERI, Huawei, vivo |
|  |  | No | Qualcomm. UE should be allowed to report different capabilities for FR1 and FR2, since separate IEs are used for FR1 and FR2 Apple. UE should be able to distinguish between FR1 and FR2 while reporting maxBW-Preference |
| 3) maxCC-Preference | Per: UE | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | M: No | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | FDD-TDD DIFF: No | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | FR1-FR2 DIFF: No | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
| 4) maxMIMO-LayerPreference | Per: UE | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | M: No | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | FDD-TDD DIFF: No | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | FR1-FR2 DIFF: No | Yes | OPPO, ERI, Huawei |
|  |  | No | Qualcomm. UE should be allowed to report different capabilities for FR1 and FR2, since separate IEs are used for FR1 and FR2 vivo. We share the same view as Qualcomm. It should allow UE to report different capability for FR1 and FR2. Apple : Agree with Qualcomm and Vivo |
| 5) release-Preference | Per: UE | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | M: No | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | FDD-TDD DIFF: No | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |
|  | FR1-FR2 DIFF: No | Yes | OPPO, ERI, Qualcomm, Huawei, vivo, Apple |
|  |  | No |  |

## Miscellaneous for Rel-16 PWS WI

### Discussion point 8

- Companies are invited to provide their views on other UE capabilities for Rel-16 UE power saving WI not discussed in previous discussion points.

| Company’s name | Company’s comments, if any |
| --- | --- |
| OPPO | Do we need to introduce a capability bit to indicate whether UE supports power saving related assistance information? |
| Ericsson | 1. UE assistance capability: - If the UE indicates that it supports a certain UE assistance feature, it is our understanding that the UE then indicates that it can, i.e. in principle is able, to provide UE assistance for all preferred parameters. For 38.331 we then have an additional discussion what it means when certain optional parameters are omitted and not signalled by the UE. However this story is not completely satisfactory when from the 38.331 it is concluded that omission means “no preference”. For example, for the release preference, it is possible (and likely in my view) that the UE only provides an indication that it would like to be released, and no preferred RRC state where to go to. The NW has to be able to process preferred RRC state, and ability to act on it, from the beginning the feature is deployed, but it can never be sure when to receive any preferred RRC state from the UE. We do not think it is a good answer that NW can implement preferred RRC state signalling, when UE starts sending it. 2. Optional, mandatory, conditionally mandatory: - In our understanding we will have discussion about the optional and mandatory aspects of the UE capabilities later, i.e. they seem to be “hidden” in the draft stage 3 below, however we agree that all the NR UE power saving features should be optional. But perhaps some conditional mandatory discussions may apply. For example UE shall indicate support for maxLayersMIMO-Indication, when it indicates support for maxMIMO-LayerPreference. |
| Qualcomm | We do not think it is necessary to have a capability just for UE assistance itself, if we already have capability bits for individual parameters. |
| Huawei | There is a potential linkage between Rel-15 capability maxLayersMIMO-Indication and Rel-16 capability maxMIMO-LayerPreference, it would be clearer to capture the relationship between these two fields in the spec. |
| Intel | - To OPPO’s question, we do not think that a general capability for power saving related assistance information is needed. - To Ericsson’s comment, our understanding is that all these new features would be optional from UE point of view. Nevertheless, discussion point 7) address this when asking if companies are ok that each capability is marked in TS 38.306 as “M: No” (where the M refers to whether we are ok with the feature being mandatory (if M: Yes) or optional (if M: No). - To Qualcomm’s comment, we also share the same view as explained in previous discussion point 1). - To Huawei’s comment, we do not have any strong view on whether that linkage is needed for UE assistance purpose; however, if companies prefer doing so, we wonder if similar reference should be added for the related parameter defined as part of overheating UE assistance. |
| CATT | We don’t think it is necessary to have a general capability for power saving related assistance information. And we don’t think we need to define the linkage between power saving related assistance info and overheating UE assistance. We need to wait for the final agreements for the field descriptions for power saving related assistance info. |
| vivo | We also don’t think a general capability for assistance information in power saving is needed. Otherwise, there will be a list of issues on “no preference”. For the linkage between power saving assistance information and overheating assistance information, it is very interesting issue. We think it may need some discussion. |
| Xiaomi | As we can see, many people would prefer to put all the UE assistance information related to power saving in section 4.2.2 “General parameters” like the overhearingInd. However since we already have separate capability bits for individual parameters, it is not necessarily to do so. Obviously the maxCC, maxBW and maxMIMO are perhaps more related to PHY parameters. |

# Report summary and phase-2 discussion

This section provides a summary of the inputs provided by the 10 companies that participated in this email discussion (OPPO, Ericsson, Qualcomm, Huawei, Intel, CATT, ZTE, vivo, Xiaomi and Apple), and few follow up discussion points for companies to further provide their views in phase 2.

## Discussion point 1

On the UE assistance information that provides UE’s preference to the C-DRX configuration:

All 10 companies support option (a) on defining a new UE capability (referred e.g. as drx-Preference) is to indicate its preference on DRX parameters for power saving in RRC_CONNECTED.

- 9 companies (OPPO, Ericsson, Huawei, Intel, CATT, ZTE, vivo, Xiaomi and Apple) prefer defining it in section 4.2.2 “General parameters” of TS 38.306.

- 1 companies (Qualcomm) prefers defining it in a new section 4.2.x “Power saving parameters” to be defined in TS 38.306.

- [10 out of 10] A new UE capability (referred e.g. as drx-Preference) is defined to indicate its preference on DRX parameters for power saving in RRC_CONNECTED. [9 out of 10] It is included in section 4.2.2 “General parameters” of TS 38.306.

## Discussion point 2

On UE’s preference to the maximum aggregated bandwidth:

All 10 companies support option (a) on defining a new UE capability (referred e.g. as maxBW-Preference) is defined to indicate its preference on the maximum aggregated bandwidth for power saving in RRC_CONNECTED.

- 8 companies (OPPO, Ericsson, Huawei, Intel, CATT, ZTE, vivo, and Apple) prefer defining it in section 4.2.2 “General parameters” of TS 38.306.

- 2 companies (Intel, and Xiaomi) prefer defining it in section 4.2.7.10 “Phy-Parameters” of TS 38.306.

- 1 companies (Qualcomm) prefers defining it in a new section 4.2.x “Power saving parameters” to be defined in TS 38.306.

- [10 out of 10] A new UE capability (referred e.g. as maxBW-Preference) is defined to indicate its preference on the maximum aggregated bandwidth for power saving in RRC_CONNECTED. [8 out of 10] It is included in section 4.2.2 “General parameters” of TS 38.306.

## Discussion point 3

On UE’s preference to the maximum number of secondary component carriers:

All 10 companies support option (a) on defining a new UE capability (referred e.g. as maxCC-Preference) is defined to indicate its preference on the maximum number of secondary component carriers for power saving in RRC_CONNECTED.

- 8 companies (OPPO, Ericsson, Huawei, Intel, CATT, ZTE, vivo, and Apple) prefer defining it in section 4.2.2 “General parameters” of TS 38.306.

- 2 companies (Intel, and Xiaomi) prefer defining it in section 4.2.7.10 “Phy-Parameters” of TS 38.306.

- 1 companies (Qualcomm) prefers defining it in a new section 4.2.x “Power saving parameters” to be defined in TS 38.306.

- [10 out of 10] A new UE capability (referred e.g. as maxCC-Preference) is defined to indicate its preference on the maximum number of secondary component carriers for power saving in RRC_CONNECTED. [8 out of 10] It is included in section 4.2.2 “General parameters” of TS 38.306.

## Discussion point 4

On UE’s preference to the maximum number of MIMO layers:

All 10 companies support option (a) on defining a new UE capability (referred e.g. as maxMIMO-LayerPreference) is defined to indicate its preference on the maximum number of MIMO layers for power saving in RRC_CONNECTED.

- 8 companies (OPPO, Ericsson, Huawei, Intel, CATT, ZTE, vivo, and Apple) prefer defining it in section 4.2.2 “General parameters” of TS 38.306.

- 2 companies (Intel, and Xiaomi) prefer defining it in section 4.2.7.10 “Phy-Parameters” of TS 38.306.

- 1 companies (Qualcomm) prefers defining it in a new section 4.2.x “Power saving parameters” to be defined in TS 38.306.

1 company (Ericsson) suggest the discussion on whether the bundling of UE assistance for maxCC, maxBW and maxMIMO, i.e. bundle them into a separate UE capability. As rapporteur, we suggest adding a new discussion point 9 below to understand companies’ view on this proposal.

- [10 out of 10] A new UE capability (referred e.g. as maxMIMO-LayerPreference) is defined to indicate its preference on the maximum number of MIMO layers for power saving in RRC_CONNECTED. [8 out of 10] It is included in section 4.2.2 “General parameters” of TS 38.306.

### Discussion point 9

- Do you prefer bundling into a separate new UE capability the support for UE’s assistance of the maxCC, maxBW and maxMIMO?

- No.

- Yes. If so, please explain your understanding on “bundling of maxCC, maxBW, and maxMIMO”. In addition, provide your views on whether this new capability could be defined in section 4.2.2 “General parameters” of TS 38.306 and could be referred e.g. as maxConfig-PwsPreference.

| Company’s name | Option | Company’s comments, if any |
| --- | --- | --- |
| Ericsson | a or b | We do not have a strong view about this, i.e. either way is fine for us. |
| Intel | a | We do not see strong view to bundle those preferences into the same UE capability when they are configured/treated independently during UE assistance mechanism. |
| Qualcomm | a | Implementation related to number MIMO layer involve different hardware from implementation related to max number of carriers or max aggregated bandwidth. Hence they should have separate capabilities. |
| vivo | a | We also think they should be configured independently in UE assistance mechanism. |
| Huawei | a | They are different mechanisms and it would be better to associate with different capabilities. |
| OPPO | a |  |
| CATT | a | Same view as above companies |
| Apple | a | It is better to have separate capabilities to allow for independent configurability. |

7 companies (Qualcomm, vivo, Huawei, OPPO, CATT, Apple and Intel) support option (a) of not defining a new capability that bundles the support for maxCC, maxBW and maxMIMO, and 1 company (Ericsson) was ok either way.

- [8 out of 8] It is not agreed to bundle into a separate new UE capability the support for UE’s assistance of the maxCC, maxBW and maxMIMO.

## Discussion point 5

On UE’s preference to transition out of RRC_CONNECTED:

All 10 companies support option (a) on defining a new UE capability (referred e.g. as release-Preference) is defined to indicate its preference assistance information to transition out of RRC_CONNECTED for power saving.

- 9 companies (OPPO, Ericsson, Huawei, Intel, CATT, ZTE, vivo, Xiaomi and Apple) prefer defining it in section 4.2.2 “General parameters” of TS 38.306.

- 1 companies (Qualcomm) prefers defining it in a new section 4.2.x “Power saving parameters” to be defined in TS 38.306.

- [10 out of 10] A new UE capability (referred e.g. as release-Preference) is defined to indicate its preference assistance information to transition out of RRC_CONNECTED for power saving. [9 out of 10] It is included in section 4.2.2 “General parameters” of TS 38.306.

## Discussion point 6

For the relaxed measurement feature:

All 10 companies support option (a) on defining a new UE capability is defined for the relaxed measurement feature without signalling to the gNB, i.e. gNB will not know whether a UE supports or not this feature.

- All companies prefer defining it in section section 5 “Optional features without UE radio access capability parameters” of TS 38.306.

1 company (Ericsson) explained that in their view, network could only configure “low mobility” trigger (although network does not have to provide IOT test opportunities for “not-at-cell-edge” because this feature is mainly used in an Idle/Inactive). 1 company (vivo) already responded that they do not see any difference between “low mobility” or for “not-at-cell-edge” trigger. As rapporteur, we suggest adding a new discussion point 10 below to understand companies’ view on this proposal.

1 company (Ericsson) indicated that it is FFS if RAN4 outcome on RRM relaxation methods may have any impact on the scope of this UE capability. 1 company (vivo) indicated that RAN4 already made their conclusions. As rapporteur, we suggest not to discuss this in RAN2 until RAN4 sends a formal LS with their inputs on this, if any.

1 company (Ericsson) pointed that optional/mandatory discussions is required, although this is addressed in discussion point 5.

- [10 out of 10] new UE capability is defined for the relaxed measurement feature without signalling to the gNB, i.e. gNB will not know whether a UE supports or not this feature. [10 out of 10] It is included in section 5 “Optional features without UE radio access capability parameters” of TS 38.306.

### Discussion point 10

- Do you share the view that network could only configure the “low mobility” trigger although network does not have to provide IOT test opportunities for “not-at-cell-edge” (because this feature is mainly used is an Idle/Inactive)?

- No.

- Yes. If so, please indicate if you prefer adding any specific TP on this.

| Company’s name | Option | Company’s comments, if any |
| --- | --- | --- |
| Ericsson | B | The “low mobility” and “not-at-cell-edge” can be configured independently, as discussed and agreed in RAN2: Agreements: - Network can configure the triggering criteria independently (i.e. either cell-edge or low mobility or both) It is not in the spirit of this agreement, if the NW would have to provide IOT test opportunities for “not-at-cell-edge” when it does not want to deploy and configure this trigger condition. We are not sure if RAN2 needs to capture anything about this, i.e. perhaps in RAN5, but we are not sure. PS: we do not think that it is a good idea to split the UE capability, nor would that resolve the NW issue. In case there would be two capabilities, we would need to discuss what to do when NW configures both triggers(via AND), and UE only supports one of them. |
| Intel | - | In our understanding this topic is not related to UE capability discussion but to the way relaxed measurement criteria may be configured by the network. |
| Qualcomm | - | We have the same understanding as Intel. |
| vivo | - | First, we donot see any difference between “low mobility” and “not-at-cell-edge”. Besides, we share the same view as Intel and Qualcomm, as it is related to network configuration. |
| Huawei | - | We share the same view with Intel that it is not related to UE capability. |
| OPPO |  | Share the same view as Intel |
| CATT | - | As others, we are not sure to get the point. There seems to be several issues raised. For IOT aspects, we believe it belongs to RAN5 to decide. And we don’t see a contradiction with supporting per-criterion configuration granularity and having a single capability for both. That is, NW configuring either one or other criterion not knowing if/which UE(s) support it. A UE not capable of any given criterion, will just not relax its measurements if this criterion is configured by the NW. |
| Apple | - | Same view as Intel |

1 company (Ericsson) supports that network could only configure the “low mobility” trigger without having to provide IOT test opportunities for “not-at-cell-edge”. Moreover, they wonder if the NW would have to provide IOT test opportunities for “not-at-cell-edge” when it does not want to deploy and configure this trigger condition; and if so, whether RAN2 or RAN5 should capture anything on this regard. On the other hand, 7 companies (Qualcomm, vivo, Huawei, OPPO, CATT, Apple and Intel) understand that this topic is not related to UE capability discussion but to the way relaxed measurement criteria may be configured by the network. As rapporteur, we suggest not to discuss further this topic as part of this email discussion.

## Discussion point 7

For drx-Preference, maxCC-Preference, release-Preference:

- All 10 companies support defining its characteristics as “Per: UE” (to the characteristic associated to the UE), “M: No” (to the characteristic associated with mandatory/optional features), “FDD-TDD DIFF: No” (to the characteristic associated with the applicability for FDD/TDD) and “FR1-FR2 DIFF: No” (to the characteristic associated to FR1/FR2 equally applicable for FR1/FR2).

For maxMIMO-LayerPreference:

- All 10 companies support defining its characteristics as “Per: UE” (to the characteristic associated to the UE), “M: No” (to the characteristic associated with mandatory/optional features), “FDD-TDD DIFF: No” (to the characteristic associated with the applicability for FDD/TDD).

- 8 companies (OPPO, Ericsson, Huawei, Intel, CATT, ZTE, vivo, Xiaomi) support defining its characteristic as “FR1-FR2 DIFF: No” (to the characteristic associated to FR1/FR2 equally applicable for FR1/FR2), vs 2 ones (Qualcomm, Apple) that support it as “FR1-FR2 DIFF: Yes” to allow to report different capabilities for FR1 and FR2, since separate IEs are used for FR1 and FR2.

For maxBW-Preference:

- All 10 companies support defining its characteristics as “Per: UE” (to the characteristic associated to the UE), “M: No” (to the characteristic associated with mandatory/optional features), “FDD-TDD DIFF: No” (to the characteristic associated with the applicability for FDD/TDD).

- 7 companies (OPPO, Ericsson, Huawei, Intel, CATT, ZTE, vivo, Xiaomi) support defining its characteristic as “FR1-FR2 DIFF: No” (to the characteristic associated to FR1/FR2 equally applicable for FR1/FR2), vs 3 ones (Qualcomm, vivo, Apple) that support it as “FR1-FR2 DIFF: Yes” to allow to report different capabilities for FR1 and FR2, since separate IEs are used for FR1 and FR2.

- [10 out of 10] For drx-Preference, maxCC-Preference, release-Preference, maxMIMO-LayerPreference, and maxBW-Preference, its characteristics are defined as as “Per: UE” (to the characteristic associated to the UE), “M: No” (to the characteristic associated with mandatory/optional features), “FDD-TDD DIFF: No” (to the characteristic associated with the applicability for FDD/TDD).

- [10 out of 10] For drx-Preference, maxCC-Preference, and release-Preference, its characteristics are defined as “FR1-FR2 DIFF: No” (to the characteristic associated to FR1/FR2 equally applicable for FR1/FR2).

- [8 or 7 out of 10] For maxMIMO-LayerPreference, and maxBW-Preference, its characteristics are defined as “FR1-FR2 DIFF: No” (to the characteristic associated to FR1/FR2 equally applicable for FR1/FR2).

## Discussion point 8

The following new topics were raised and shortly discussed by some companies:

- 1 company (OPPO) suggests introducing a capability bit to indicate whether UE supports power saving related assistance information. 4 companies (Qualcomm, Intel, CATT, vivo) does not think this new UE capability is needed. As rapporteur, we suggest not to discuss this topic as there are already 4 companies no supporting this new UE capability.

- 1 company (Ericsson) suggests discussing details regarding the omission of providing a preference for a given parameter or feature of UE assistance information. As rapporteur, we suggest not to discuss this topic here as it is already address on email discussion Post109e#43.

- 1 company (Ericsson) suggests discussing the optional and mandatory aspects of the UE capabilities. 1 company (Intel) indicate that this is already addressed as part of discussion point 7. As rapporteur, we suggest not to re-discuss this point again.

- 1 company (Huawei) suggest indicating the linkage between Rel-15 capability maxLayersMIMO-Indication and Rel-16 capability maxMIMO-LayerPreference. If this were added, 1 company (Intel) raised the question whether this similar statement should also be considered for the related parameter defined as part of overheating UE assistance. 1 company (CATT) does not see the need to define this linkage and another one (vivo) prefers to further discuss this topic. As rapporteur, we suggest discussing this in the following discussion point 11.

- 1 company (Xiaomi) prefers having UE’s preference on maxCC, maxBW and maxMIMO defined as part of PHY parameters (instead of the general parameters). As rapporteur, we suggest not re-discussing this topic as it is already covered in previous discussion point (and there was a clear majority view).

### Discussion point 11

- Do you prefer indicating the linkage between Rel-15 capability maxLayersMIMO-Indication and Rel-16 capability maxMIMO-LayerPreference?

- No.

- Yes. If so, should this also be considered for the related parameter defined as part of Rel-15 overheating UE assistance information.

| Company’s name | Option | Company’s comments, if any |
| --- | --- | --- |
| Ericsson | Yes | In our understanding the maxLayersMIMO-Indication was introduced late in REL-15, to solve potential ambiguity in the configuration of MIMO when the UE indicates different MIMO capability for band combinations. This is an optional REL-15 feature, but in our understanding a UE is “strongly advised” to support this feature. After some further thinking we were not sure if there should be a coupling with UE assistance or with configuration. In our understanding a REL-16 UE is required to support per BWP MIMO configuration. We could consider to specify that a REL-16 UE shall support maxLayersMIMO-Indication. |
| Intel | Neutral | We understand that there is a relation between them and indeed UE should support Rel-15 capability maxLayersMIMO-Indication for the network to act upon reception of Rel-16 capability maxMIMO-LayerPreference. On other hand, we do not see it critical to specify this similarly as it is done for the similar preference included in overheating. |
| Qualcomm | Option C | Our understanding is that maxLayersMIMO-Indication is for UE to indicate the maximum number of MIMO layers that it can support. On the other hand, maxMIMO-LayerPreference for the purpose of power saving is for UE to indicate whether it supports per-BWP configuration of max MIMO layer. They are different capabilities. |
| Vivo | Slightly Yes | We think there is somewhat linkage between them. Rel-15 capability should be supported for supporting rel-16 capability. Otherwise, Rel-16 capability maxMIMO-LayerPreference is not useful for the network. But we are not so sure about this linkage…. |
| Huawei | - | Thank rapporteur and colleagues for taking care of our comments, and thank rapporteur for raising a good point that there is similar situation for overheating. Our initial thinking is that if maxLayersMIMO-Indication is not supported or the Rel-16 “per-BWP MIMO layer configuration” is not supported by the UE, no power saving gain would be obtained even if UE supports maxMIMO-LayerPreference. But actually we don’t intend to make it complicated and it would be better to avoid any complicated changes on legacy Rel-15 mechanism. Considering this, we are a bit hesitant to introduce this linkage if more (e.g. Rel-15 overheating) will be impacted. If companies are aligned with the understanding that: power saving gain is obtained when both maxLayersMIMO-Indication or “per-BWP MIMO layer configuration” is supported by the UE, we are fine if nothing is specified as we do for overheating, the supported UE capabilities is up to UE itself. |
| OPPO |  | We agree the observation from Huawei “Our initial thinking is that if maxLayersMIMO-Indication is not supported or the Rel-16 “per-BWP MIMO layer configuration” is not supported by the UE, no power saving gain would be obtained even if UE supports maxMIMO-LayerPreference ”. But we also don't think a “linkage” should be introduced. |
| CATT | Option C | We agree there is a linkage between Rel-15 capability maxLayersMIMO-Indication and Rel-16 capability maxMIMO-LayerPreference. But we also agree with Intel and Huawei that we could live with it for overheating, so we don't need to introduce anything new for power saving. |

2 companies (Ericsson, vivo) prefer indicating the linkage between Rel-15 capability maxLayersMIMO-Indication and Rel-16 capability maxMIMO-LayerPreference, vs 4 companies (Qualcomm, Huawei, OPPO, CATT) that prefer not to do so and 1 company (Intel) that is neutral with slightly preference not to do so either. As rapporteur, we suggest not to specify any linkage between Rel-15 capability maxLayersMIMO-Indication and Rel-16 capability maxMIMO-LayerPreference.

- [5 out of 7] Rel-16 capability maxMIMO-LayerPreference is not linked to Rel-15 capability maxLayersMIMO-Indication.

# Conclusion

The proposals captured are the following:

Proposal 1.	[10 out of 10] A new UE capability (referred e.g. as drx-Preference) is defined to indicate its preference on DRX parameters for power saving in RRC_CONNECTED. [9 out of 10] It is included in section 4.2.2 “General parameters” of TS 38.306.

Proposal 2.	[10 out of 10] A new UE capability (referred e.g. as maxBW-Preference) is defined to indicate its preference on the maximum aggregated bandwidth for power saving in RRC_CONNECTED. [8 out of 10] It is included in section 4.2.2 “General parameters” of TS 38.306.

Proposal 3.	[10 out of 10] A new UE capability (referred e.g. as maxCC-Preference) is defined to indicate its preference on the maximum number of secondary component carriers for power saving in RRC_CONNECTED. [8 out of 10] It is included in section 4.2.2 “General parameters” of TS 38.306.

Proposal 4.	[10 out of 10] A new UE capability (referred e.g. as maxMIMO-LayerPreference) is defined to indicate its preference on the maximum number of MIMO layers for power saving in RRC_CONNECTED. [8 out of 10] It is included in section 4.2.2 “General parameters” of TS 38.306.

Proposal 5.	[8 out of 8] It is not agreed to bundle into a separate new UE capability the support for UE’s assistance of the maxCC, maxBW and maxMIMO.

Proposal 6.	[10 out of 10] A new UE capability (referred e.g. as release-Preference) is defined to indicate its preference assistance information to transition out of RRC_CONNECTED for power saving. [9 out of 10] It is included in section 4.2.2 “General parameters” of TS 38.306.

Proposal 7.	[10 out of 10] new UE capability is defined for the relaxed measurement feature without signalling to the gNB, i.e. gNB will not know whether a UE supports or not this feature. [10 out of 10] It is included in section 5 “Optional features without UE radio access capability parameters” of TS 38.306.

Proposal 8.	[10 out of 10] For drx-Preference, maxCC-Preference, release-Preference, maxMIMO-LayerPreference, and maxBW-Preference, its characteristics are defined as as “Per: UE” (to the characteristic associated to the UE), “M: No” (to the characteristic associated with mandatory/optional features), “FDD-TDD DIFF: No” (to the characteristic associated with the applicability for FDD/TDD).

Proposal 9.	[10 out of 10] For drx-Preference, maxCC-Preference, and release-Preference, its characteristics are defined as “FR1-FR2 DIFF: No” (to the characteristic associated to FR1/FR2 equally applicable for FR1/FR2).

Proposal 10.	[8 or 7 out of 10] For maxMIMO-LayerPreference, and maxBW-Preference, its characteristics are defined as “FR1-FR2 DIFF: No” (to the characteristic associated to FR1/FR2 equally applicable for FR1/FR2).

Proposal 11.	[5 out of 7] Rel-16 capability maxMIMO-LayerPreference is not linked to Rel-15 capability maxLayersMIMO-Indication.

# References

- R2-2000452, UE capabilities for R16 UE power saving WI, Intel Corporation, February 2020.

- R2-2000453, UE capabilities for R16 UE power saving WI, Intel Corporation, February 2020, Rel-16, TS 38.306, v15.8.0, CR#0231, category B, NR_UE_pow_sav.

# Annex A

Baseline CR to TS 38.306 showing the TP based on the proposal suggested in the different discussion points of this email discussion considering [1][2].

### 4.2.2	General parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| accessStratumRelease Indicates the access stratum release the UE supports as specified in TS 38.331 [9]. | UE | Yes | No | No |
| delayBudgetReporting Indicates whether the UE supports delay budget reporting as specified in TS 38.331 [9]. | UE | No | No | No |
| drx-Preference Indicates whether the UE supports providing its preference on DRX parameters for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| inactiveState Indicates whether the UE supports RRC_inactive as specified in TS 38.331 [9]. | UE | Yes | No | No |
| maxBW-Preference Indicate whether the UE supports providing its preference on the maximum aggregated bandwidth for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| maxCC-Preference Indicate whether the UE supports providing its preference on the maximum number of secondary component carriers for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| maxMIMO-LayerPreference Indicate whether the UE supports providing its preference on the maximum number of MIMO layers for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| overheatingInd Indicates whether the UE supports overheating assistance information. | UE | No | No | No |
| reducedCP-Latency Indicates whether the UE supports reduced control plane latency as defined in TS 38.331 [9] | UE | No | No | No |
| release-Preference Indicates whether the UE supports providing its preference assistance information to transition out of RRC_CONNECTED for power saving, as specified in TS 38.331 [9]. | UE | No | No | No |
| splitSRB-WithOneUL-Path Indicates whether the UE supports UL transmission via MCG path and DL reception via either MCG path or SCG path, as specified for the split SRB in TS 37.340 [7]. The UE shall only set the bit in UE-MRDC-Capability -> generalParametersMRDC. It shall not set the FDD/TDD specific fields. | UE | No | No | No |
| splitDRB-withUL-Both-MCG-SCG Indicates whether the UE supports UL transmission via both MCG path and SCG path for the split DRB as specified in TS 37.340 [7]. The UE shall only set the bit in UE-MRDC-Capability -> generalParametersMRDC. It shall not set the FDD/TDD specific fields. | UE | No | No | No |
| srb3 Indicates whether the UE supports direct SRB between the SN and the UE as specified in TS 37.340 [7]. The UE shall only set the bit in UE-MRDC-Capability -> generalParametersMRDC. It shall not set the FDD/TDD specific fields. This field is not applied to NE-DC. | UE | Yes | No | No |
| v2x-EUTRA Indicates whether the UE supports EUTRA V2X according to UE-EUTRA-Capability as defined in TS 36.331 [17], independent of the configured EN-DC band combination. This field is only applied to EN-DC. In UE-NR-Capability, this field is not used, and UE does not include the field. | UE | No | No | No |

### 4.2.7	Physical layer parameters

<Omitted text>

#### 4.2.7.10	Phy-Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| absoluteTPC-Command Indicates whether the UE supports absolute TPC command mode. | UE | No | No | Yes |
| almostContiguousCP-OFDM-UL Indicates whether the UE supports almost contiguous UL CP-OFDM transmissions as defined in clause 6.2 of TS 38.101-1 [2]. | UE | No | No | Yes |
| bwp-SwitchingDelay Defines whether the UE supports DCI and timer based active BWP switching delay type1 or type2 specified in clause 8.6.2 of TS 38.133 [5]. It is mandatory to report type 1 or type 2. | UE | Yes | No | No |
| cbg-FlushIndication-DL Indicates whether the UE supports CBG-based (re)transmission for DL using CBG flushing out information (CBGFI) as specified in TS 38.214 [12]. | UE | No | No | No |
| cbg-TransIndication-DL Indicates whether the UE supports CBG-based (re)transmission for DL using CBG transmission information (CBGTI) as specified in TS 38.214 [12]. | UE | No | No | No |
| cbg-TransIndication-UL Indicates whether the UE supports CBG-based (re)transmission for UL using CBG transmission information (CBGTI) as specified in TS 38.214 [12]. | UE | No | No | No |
| configuredUL-GrantType1 Indicates whether the UE supports Type 1 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value of one. | UE | No | No | No |
| configuredUL-GrantType2 Indicates whether the UE supports Type 2 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value of one. | UE | No | No | No |
| cqi-TableAlt Indicates whether UE supports the CQI table with target BLER of 10^-5. | UE | No | No | Yes |
| csi-ReportFramework See csi-ReportFramework in 4.2.7.2. For a band combination comprised of FR1 and FR2 bands, this parameter, if present, limits the corresponding parameter in MIMO-ParametersPerBand. | Band or UE | Yes | No | No |
| csi-ReportWithoutCQI Indicates whether UE supports CSI reporting with report quantity set to 'CRI/RI/i1' as defined in clause 5.2.1.4 of TS 38.214 [12]. | UE | No | No | Yes |
| csi-ReportWithoutPMI Indicates whether UE supports CSI reporting with report quantity set to 'CRI/RI/CQI' as defined in clause 5.2.1.4 of TS 38.214 [12]. | UE | No | No | Yes |
| csi-RS-CFRA-ForHO Indicates whether the UE can perform reconfiguration with sync using a contention free random access on PRACH resources that are associated with CSI-RS resources of the target cell. | UE | No | No | No |
| csi-RS-IM-ReceptionForFeedback See csi-RS-IM-ReceptionForFeedback in 4.2.7.2. For a band combination comprised of FR1 and FR2 bands, this parameter, if present, limits the corresponding parameter in MIMO-ParametersPerBand. | Band or UE | Yes | No | No |
| csi-RS-ProcFrameworkForSRS See csi-RS-ProcFrameworkForSRS in 4.2.7.2. For a band combination comprised of FR1 and FR2 bands, this parameter, if present, limits the corresponding parameter in MIMO-ParametersPerBand. | Band or UE | No | No | No |
| dl-64QAM-MCS-TableAlt Indicates whether the UE supports the alternative 64QAM MCS table for PDSCH. | UE | No | No | Yes |
| dl-SchedulingOffset-PDSCH-TypeA Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A. | UE | Yes | Yes | Yes |
| dl-SchedulingOffset-PDSCH-TypeB Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B. | UE | Yes | Yes | Yes |
| downlinkSPS Indicates whether the UE supports PDSCH reception based on semi-persistent scheduling. | UE | No | No | No |
| dynamicBetaOffsetInd-HARQ-ACK-CSI Indicates whether the UE supports indicating beta-offset (UCI repetition factor onto PUSCH) for HARQ-ACK and/or CSI via DCI among the RRC configured beta-offsets. | UE | No | No | No |
| dynamicHARQ-ACK-Codebook Indicates whether the UE supports HARQ-ACK codebook dynamically constructed by DCI(s). This field shall be set to supported. | UE | Yes | No | No |
| dynamicHARQ-ACK-CodeB-CBG-Retx-DL Indicates whether the UE supports HARQ-ACK codebook size for CBG-based (re)transmission based on the DAI-based solution as specified in TS 38.213 [11]. | UE | No | No | No |
| dynamicPRB-BundlingDL Indicates whether UE supports DCI-based indication of the PRG size for PDSCH reception. | UE | No | No | No |
| dynamicSFI Indicates whether the UE supports monitoring for DCI format 2_0 and determination of slot formats via DCI format 2_0. | UE | No | Yes | Yes |
| dynamicSwitchRA-Type0-1-PDSCH Indicates whether the UE supports dynamic switching between resource allocation Types 0 and 1 for PDSCH as specified in TS 38.212 [10]. | UE | No | No | No |
| dynamicSwitchRA-Type0-1-PUSCH Indicates whether the UE supports dynamic switching between resource allocation Types 0 and 1 for PUSCH as specified in TS 38.212 [10]. | UE | No | No | No |
| pucch-F0-2WithoutFH Indicates whether the UE supports transmission of a PUCCH format 0 or 2 without frequency hopping. When included, the UE does not support PUCCH formats 0 and 2 without frequency hopping. When not included, the UE supports the PUCCH formats 0 and 2 without frequency hopping. | UE | Yes | No | Yes |
| pucch-F1-3-4WithoutFH Indicates whether the UE supports transmission of a PUCCH format 1, 3 or 4 without frequency hopping. When included, the UE does not support PUCCH formats 1, 3 and 4 without frequency hopping. When not included, the UE supports the PUCCH formats 1, 3 and 4 without frequency hopping. | UE | Yes | No | Yes |
| interleavingVRB-ToPRB-PDSCH Indicates whether the UE supports receiving PDSCH with interleaved VRB-to-PRB mapping as specified in TS 38.211 [6]. | UE | Yes | No | No |
| interSlotFreqHopping-PUSCH Indicates whether the UE supports inter-slot frequency hopping for PUSCH transmissions. | UE | No | No | No |
| intraSlotFreqHopping-PUSCH Indicates whether the UE supports intra-slot frequency hopping for PUSCH transmission, except for PUSCH scheduled by PDCCH in the Type1-PDCCH common search space before RRC connection establishment. | UE | Yes | No | Yes |
| maxLayersMIMO-Indication Indicates whether the UE supports the network configuration of maxMIMO-Layers as specified in TS 38.331 [9]. | UE | Yes | No | No |
| maxNumberSearchSpaces Indicates whether the UE supports up to 10 search spaces in a SCell per BWP. | UE | No | No | No |
| multipleCORESET Indicates whether the UE supports configuration of more than one PDCCH CORESET per BWP in addition to the CORESET with CORESET-ID 0 in the BWP. It is mandatory with capability signaling for FR2 and optional for FR1. | UE | CY | No | Yes |
| mux-HARQ-ACK-PUSCH-DiffSymbol Indicates whether the UE supports HARQ-ACK piggyback on a PUSCH with/without aperiodic CSI once per slot when the starting OFDM symbol of the PUSCH is different from the starting OFDM symbols of the PUCCH resource that HARQ-ACK would have been transmitted on. | UE | Yes | No | Yes |
| mux-MultipleGroupCtrlCH-Overlap Indicates whether the UE supports more than one group of overlapping PUCCHs and PUSCHs per slot per PUCCH cell group for control multiplexing. | UE | No | No | Yes |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot Indicates whether the UE supports multiplexing SR, HARQ-ACK and CSI on a PUCCH or piggybacking on a PUSCH more than once per slot when SR, HARQ-ACK and CSI are supposed to be sent with the same or different starting symbol in a slot. | UE | No | No | Yes |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot sameSymbol indicates the UE supports multiplexing SR, HARQ-ACK and CSI on a PUCCH or piggybacking on a PUSCH once per slot, when SR, HARQ-ACK and CSI are supposed to be sent with the same starting symbols on the PUCCH resources in a slot. diffSymbol indicates the UE supports multiplexing SR, HARQ-ACK and CSI on a PUCCH or piggybacking on a PUSCH once per slot, when SR, HARQ-ACK and CSI are supposed to be sent with the different starting symbols in a slot. The UE is mandated to support the multiplexing and piggybacking features indicated by sameSymbol while the UE is optional to support the multiplexing and piggybacking features indicated by diffSymbol. If the UE indicates sameSymbol in this field and does not support mux-HARQ-ACK-PUSCH-DiffSymbol, the UE supports HARQ-ACK/CSI piggyback on PUSCH once per slot, when the starting OFDM symbol of the PUSCH is the same as the starting OFDM symbols of the PUCCH resource(s) that would have been transmitted on. If the UE indicates sameSymbol in this field and supports mux-HARQ-ACK-PUSCH-DiffSymbol, the UE supports HARQ-ACK/CSI piggyback on PUSCH once per slot for which case the starting OFDM symbol of the PUSCH is the different from the starting OFDM symbols of the PUCCH resource(s) that would have been transmitted on. | UE | FD | No | Yes |
| mux-SR-HARQ-ACK-PUCCH Indicates whether the UE supports multiplexing SR and HARQ-ACK on a PUCCH or piggybacking on a PUSCH once per slot, when SR and HARQ-ACK are supposed to be sent with the different starting symbols in a slot. | UE | No | No | Yes |
| nzp-CSI-RS-IntefMgmt Indicates whether the UE supports interference measurements using NZP CSI-RS. | UE | No | No | No |
| oneFL-DMRS-ThreeAdditionalDMRS-UL Defines whether the UE supports DM-RS pattern for UL transmission with 1 symbol front-loaded DM-RS with three additional DM-RS symbols. | UE | No | No | Yes |
| oneFL-DMRS-TwoAdditionalDMRS-UL Defines support of DM-RS pattern for UL transmission with 1 symbol front-loaded DM-RS with 2 additional DM-RS symbols and more than 1 antenna ports. | UE | Yes | No | Yes |
| onePortsPTRS Defines whether UE supports PT-RS with 1 antenna port in DL reception and/or UL transmission. It is mandatory with UE capability signalling for FR2 and optional for FR1. The left most in the bitmap corresponds to DL reception and the right most bit in the bitmap corresponds to UL transmission. | UE | CY | No | Yes |
| onePUCCH-LongAndShortFormat Indicates whether the UE supports transmission of one long PUCCH format and one short PUCCH format in TDM in the same slot. | UE | No | No | Yes |
| pCell-FR2 Indicates whether the UE supports PCell operation on FR2. | UE | Yes | No | FR2 only |
| pdcch-MonitoringSingleOccasion Indicates whether the UE supports receiving PDCCH scrambled with C-RNTI or CS-RNTI in a search space configured to be monitored within a single span of any three contiguous OFDM symbols in a slot with the capability of supporting at least 44 blind decodes in a slot for 15 kHz subcarrier spacing. | UE | No | No | FR1 only |
| pdcch-BlindDetectionCA Indicates PDCCH blind decoding capabilities supported by the UE for CA with more than 4 CCs as specified in TS 38.213 [11]. The field value is from 4 to 16. NOTE: FR1-FR2 differentiation is not allowed in this release, although the capability signalling is supported for FR1-FR2 differentiation. | UE | No | No | No |
| pdcch-BlindDetectionMCG-UE Indicates PDCCH blind decoding capabilities supported for MCG when in NR DC. The field value is from 1 to 15. The UE sets the value in accordance with the constraints specified in TS 38.213 [11]. Additionally, if the UE does not report pdcch-BlindDetectionCA, and if X is the maximum number of CCs supported by the UE across all NR-DC band combinations then there is at least one parameter pair (X1, X2) such that X1 + X2 = X and the UE supports at least one NR-DC band combination with X1 CCs in MCG and X2 CCs in SCG and for which X1 <= pdcch-BlindDetectionMCG-UE and X2 <= pdcch-BlindDetectionSCG-UE. | UE | No | No | Yes |
| pdcch-BlindDetectionSCG-UE Indicates PDCCH blind decoding capabilities supported for SCG when in NR DC. The field value is from 1 to 15. The UE sets the value in accordance with the constraints specified in TS 38.213 [11]. Additionally, if the UE does not report pdcch-BlindDetectionCA, and if X is the maximum number of CCs supported by the UE across all NR-DC band combinations then there is at least one parameter pair (X1, X2) such that X1 + X2 = X and the UE supports at least one NR-DC band combination with X1 CCs in MCG and X2 CCs in SCG and for which X1 <= pdcch-BlindDetectionMCG-UE and X2 <= pdcch-BlindDetectionSCG-UE. | UE | No | No | Yes |
| pdsch-256QAM-FR1 Indicates whether the UE supports 256QAM modulation scheme for PDSCH for FR1 as defined in 7.3.1.2 of TS 38.211 [6]. | UE | Yes | No | FR1 only |
| pdsch-MappingTypeA Indicates whether the UE supports receiving PDSCH using PDSCH mapping type A with less than seven symbols. This field shall be set to supported. | UE | Yes | No | No |
| pdsch-MappingTypeB Indicates whether the UE supports receiving PDSCH using PDSCH mapping type B. | UE | Yes | No | No |
| pdsch-RepetitionMultiSlots Indicates whether the UE supports receiving PDSCH scheduled by DCI format 1_1 when configured with higher layer parameter pdsch-AggregationFactor > 1, as defined in 5.1.2.1 of TS 38.214 [12]. | UE | No | No | No |
| pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot Indicates the maximum number of supported PDSCH Resource Element (RE) mapping patterns for FR1, each described as a resource (including NZP/ZP CSI-RS, CRS, CORESET and SSB) or bitmap. The number of patterns coinciding in a symbol in a CC and in a slot in a CCare limited by the respective capability parameters. Value n10 means 10 RE mapping patterns and n16 means 16 RE mapping patterns, and so on. | UE | Yes | No | FR1 only |
| pdsch-RE-MappingFR2-PerSymbol/pdsch-RE-MappingFR2-PerSlot Indicates the maximum number of supported PDSCH Resource Element (RE) mapping patterns for FR2, each described as a resource (including NZP/ZP CSI-RS, CORESET and SSB) or bitmap. The number of patterns coinciding in a symbol in a CC and in a slot in a CC are limited by the respective capability parameters. Value n6 means 6 RE mapping patterns and n16 means 16 RE mapping patterns, and so on. | UE | Yes | No | FR2 only |
| precoderGranularityCORESET Indicates whether the UE supports receiving PDCCH in CORESETs configured with CORESET-precoder-granularity equal to the size of the CORESET in the frequency domain as specified in TS 38.211 [6]. | UE | No | No | No |
| pre-EmptIndication-DL Indicates whether the UE supports interrupted transmission indication for PDSCH reception based on reception of DCI format 2_1 as defined in TS 38.213 [11]. | UE | No | No | No |
| pucch-F2-WithFH Indicates whether the UE supports transmission of a PUCCH format 2 (2 OFDM symbols in total) with frequency hopping in a slot. This field shall be set to supported. | UE | Yes | No | Yes |
| pucch-F3-WithFH Indicates whether the UE supports transmission of a PUCCH format 3 (4~14 OFDM symbols in total) with frequency hopping in a slot. This field shall be set to supported. | UE | Yes | No | Yes |
| pucch-F3-4-HalfPi-BPSK Indicates whether the UE supports pi/2-BPSK for PUCCH format 3/4 as defined in 6.3.2.6 of TS 38.211 [6]. It is optional for FR1 and mandatory with capability signalling for FR2. | UE | CY | No | Yes |
| pucch-F4-WithFH Indicates whether the UE supports transmission of a PUCCH format 4 (4~14 OFDM symbols in total) with frequency hopping in a slot. | UE | Yes | No | Yes |
| pusch-RepetitionMultiSlots Indicates whether the UE supports transmitting PUSCH scheduled by DCI format 0_1 when configured with higher layer parameter pusch-AggregationFactor > 1, as defined in clause 6.1.2.1 of TS 38.214 [12]. | UE | Yes | No | No |
| pucch-Repetition-F1-3-4 Indicates whether the UE supports transmission of a PUCCH format 1 or 3 or 4 over multiple slots with the repetition factor 2, 4 or 8. | UE | Yes | No | No |
| pusch-HalfPi-BPSK Indicates whether the UE supports pi/2-BPSK modulation scheme for PUSCH as defined in 6.3.1.2 of TS 38.211 [6]. It is optional for FR1 and mandatory with capability signalling for FR2. | UE | CY | No | Yes |
| pusch-LBRM Indicates whether the UE supports limited buffer rate matching in UL as specified in TS 38.212 [10]. | UE | No | No | Yes |
| ra-Type0-PUSCH Indicates whether the UE supports resource allocation Type 0 for PUSCH as specified in TS 38.214 [12]. | UE | No | No | No |
| rateMatchingCtrlResrcSetDynamic Indicates whether the UE supports dynamic rate matching for DL control resource set. | UE | Yes | No | No |
| rateMatchingResrcSetDynamic Indicates whether the UE supports receiving PDSCH with resource mapping that excludes the REs corresponding to resource sets configured with RB-symbol level granularity based on dynamic indication in the scheduling DCI as specified in TS 38.214 [12]. | UE | No | No | No |
| rateMatchingResrcSetSemi-Static Indicates whether the UE supports receiving PDSCH with resource mapping that excludes the REs corresponding to resource sets configured with RB-symbol level granularity following the semi-static configuration as specified in TS 38.214 [12]. | UE | Yes | No | No |
| scs-60kHz Indicates whether the UE supports 60kHz subcarrier spacing for data channel in FR1 as defined in clause 4.2-1 of TS 38.211 [6]. | UE | No | No | FR1 only |
| semiOpenLoopCSI Indicates whether UE supports CSI reporting with report quantity set to 'CRI/RI/i1/CQI ' as defined in clause 5.2.1.4 of TS 38.214 [12]. | UE | No | No | Yes |
| semiStaticHARQ-ACK-Codebook Indicates whether the UE supports HARQ-ACK codebook constructed by semi-static configuration. | UE | Yes | No | No |
| spatialBundlingHARQ-ACK Indicates whether the UE supports spatial bundling of HARQ-ACK bits carried on PUCCH or PUSCH per PUCCH group. With spatial bundling, two HARQ-ACK bits for a DL MIMO data is bundled into a single bit by logical "AND" operation. | UE | Yes | No | No |
| sp-CSI-IM Indicates whether the UE supports semi-persistent CSI-IM. | UE | No | No | Yes |
| sp-CSI-ReportPUCCH Indicates whether UE supports semi-persistent CSI reporting using PUCCH formats 2, 3 and 4. | UE | No | No | No |
| sp-CSI-ReportPUSCH Indicates whether UE supports semi-persistent CSI reporting using PUSCH. | UE | No | No | No |
| sp-CSI-RS Indicates whether the UE supports semi-persistent CSI-RS. | UE | Yes | No | Yes |
| supportedDMRS-TypeDL Defines supported DM-RS configuration types at the UE for DL reception. Type 1 is mandatory with capability signaling. Type 2 is optional. | UE | CY | No | Yes |
| supportedDMRS-TypeUL Defines supported DM-RS configuration types at the UE for UL transmission. Support both type 1 and type 2 are mandatory with capability signalling. | UE | Yes | No | Yes |
| tdd-MultiDL-UL-SwitchPerSlot Indicates whether the UE supports more than one switch points in a slot for actual DL/UL transmission(s). | UE | No | TDD only | Yes |
| tpc-PUCCH-RNTI Indicates whether the UE supports group DCI message based on TPC-PUCCH-RNTI for TPC commands for PUCCH. | UE | No | No | Yes |
| tpc-PUSCH-RNTI Indicates whether the UE supports group DCI message based on TPC-PUSCH-RNTI for TPC commands for PUSCH. | UE | No | No | Yes |
| tpc-SRS-RNTI Indicates whether the UE supports group DCI message based on TPC-SRS-RNTI for TPC commands for SRS. | UE | No | No | Yes |
| twoDifferentTPC-Loop-PUCCH Indicates whether the UE supports two different TPC loops for PUCCH closed loop power control. | UE | Yes | Yes | Yes |
| twoDifferentTPC-Loop-PUSCH Indicates whether the UE supports two different TPC loops for PUSCH closed loop power control. | UE | Yes | Yes | Yes |
| twoFL-DMRS Defines whether the UE supports DM-RS pattern for DL reception and/or UL transmission with 2 symbols front-loaded DM-RS without additional DM-RS symbols. The left most in the bitmap corresponds to DL reception and the right most bit in the bitmap corresponds to UL transmission. | UE | Yes | No | Yes |
| twoFL-DMRS-TwoAdditionalDMRS-UL Defines whether the UE supports DM-RS pattern for UL transmission with 2 symbols front-loaded DM-RS with one additional 2 symbols DM-RS. | UE | Yes | No | Yes |
| twoPUCCH-AnyOthersInSlot Indicates whether the UE supports transmission of two PUCCH formats in TDM in the same slot, which are not covered by twoPUCCH-F0-2-ConsecSymbols and onePUCCH-LongAndShortFormat. | UE | No | No | Yes |
| twoPUCCH-F0-2-ConsecSymbols Indicates whether the UE supports transmission of two PUCCHs of format 0 or 2 in consecutive symbols in a slot. | UE | No | Yes | Yes |
| type1-PUSCH-RepetitionMultiSlots Indicates whether the UE supports Type 1 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value equal to 2, 4, or 8 with a single repetition of the transport block within each slot, and redundancy version pattern as indicated by UL-TWG-RV-rep. A UE supporting this feature shall also support Type 1 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value of one. | UE | No | No | No |
| type2-PUSCH-RepetitionMultiSlots Indicates whether the UE supports Type 2 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value equal to 2, 4, or 8 with a single repetition of the transport block within each slot, and redundancy version pattern as indicated by UL-TWG-RV-rep. A UE supporting this feature shall also support Type 2 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value of one. | UE | No | No | No |
| type2-SP-CSI-Feedback-LongPUCCH Indicates whether UE supports Type II CSI semi-persistent CSI reporting over PUCCH Formats 3 and 4 as defined in clause 5.2.4 of TS 38.214 [12]. | UE | No | No | No |
| uci-CodeBlockSegmentation Indicates whether the UE supports segmenting UCI into multiple code blocks depending on the payload size. | UE | Yes | No | Yes |
| ul-64QAM-MCS-TableAlt Indicates whether the UE supports the alternative 64QAM MCS table for PUSCH with and without transform precoding respectively. | UE | No | No | Yes |
| ul-SchedulingOffset Indicates whether the UE supports UL scheduling slot offset (K2) greater than 12. | UE | Yes | Yes | Yes |

#### 4.2.7.11	Other PHY parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| appliedFreqBandListFilter Mirrors the FreqBandList that the NW provided in the capability enquiry, if any. The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter. | UE | No | No | No |
| appliedFilters Contains all filters requested with UE-CapabilityRequestFilterNR from version 15.6.0 onwards. | UE | No | No | No |
| downlinkSetEUTRA Indicates the features that the UE supports on the DL carriers corresponding to one EUTRA band entry in a band combination by FeatureSetEUTRA-DownlinkId. The FeatureSetEUTRA-DownlinkId = 0 means that the UE does not support a EUTRA DL carrier in this band of a band combination. | Band | N/A | No | No |
| downlinkSetNR Indicates the features that the UE supports on the DL carriers corresponding to one NR band entry in a band combination by FeatureSetDownlinkId. The FeatureSetDownlinkId = 0 means that the UE does not support a DL carrier in this band of a band combination. A fallback per band feature set resulting from the reported DL feature set that has fallback per CC feature set is not signalled but the UE shall support it. | Band | N/A | No | No |
| featureSetCombinations Pools of feature sets that the UE supports on the NR or MR-DC band combinations. | UE | N/A | No | No |
| featureSets Pools of downlink and uplink features sets as well as a pool of FeatureSetCombination elements. A FeatureSetCombination refers to the IDs of the feature set(s) that the UE supports in that FeatureSetCombination. The BandCombination entries in the BandCombinationList then indicate the ID of the FeatureSetCombination that the UE supports fot that band combination. | UE | N/A | No | No |
| naics-Capability-List Indicates that UE in MR-DC supports NAICS as defined in defined in TS 36.331 [17]. | UE | No | No | No |
| supportedBandCombinationList Defines the supported NR and/or MR-DC band combinations by the UE. For each band combination the UE identifies the associated feature set combination by featureSetCombinations index referring to featureSetCombination. A fallback band combination resulting from the reported CA and MR-DC band combination is not signalled but the UE shall support it. For intra-band non-contiguous CA band combinations, the UE only includes one band combination, and exclude the others for which the presence of uplink CA bandwidth class in the band combination entry is different. One band combination entry can also indicate support of any other possible permutations in the presence of uplink CA bandwidth class where a paired downlink CA bandwidth class is the same or where the number of UL CCs is smaller than the one of paired DL CCs expressed by the CA bandwidth class, as specified in TS 36.306 [15]. For these band combinations not included in the capability, the supported feature set is the same as the ones for the band combination included in the UE capability. | UE | Yes | No | No |
| supportedBandCombinationListNEDC-Only Defines the supported NE-DC only type of band combinations by the UE. | UE | Yes | No | No |
| supportedBandListNR Includes the supported NR bands as defined in TS 38.101-1 [2] and TS 38.101-2 [3]. | UE | Yes | No | No |
| uplinkSetEUTRA Indicates the features that the UE supports on the UL carriers corresponding to one EUTRA band entry in a band combination by FeatureSetEUTRA-UplinkId. The FeatureSetUplinkId = 0 means that the UE does not support a UL carrier in this band of a band combination. | Band | N/A | No | No |
| uplinkSetNR Indicates the features that the UE supports on the UL carriers corresponding to one NR band entry in a band combination by FeatureSetUplinkId. The FeatureSetUplinkId = 0 means that the UE does not support a UL carrier in this band of a band combination. A fallback per band feature set resulting from the reported UL feature set that has fallback per CC feature set is not signalled but the UE shall support it. | Band | N/A | No | No |

#### 4.2.7.12	NRDC-Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| sfn-SyncNRDC Indicates the UE supports NR-DC only with SFN and frame synchronization between PCell and PSCell. If not included by the UE supporting NR-DC, the UE supports NR-DC with slot-level synchronization without condition on SFN and frame synchronization. | UE | No | No | No |

# 5	Optional features without UE radio access capability parameters

Definitions for feature

SU-MIMO Interference Mitigation advanced receiver - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas UE supporting the feature is required to meet the Enhanced Receiver Type requirements in TS 38.101-4 [18].

Relaxed measurement

End Text Proposal Change