3GPP TSG-RAN WG2 Meeting #109bis-e	                        R2-2002617

Electronic, 20 April – 30 April 2020

Source:	vivo

Title:          Discussion on gap configuration and request for NR positioning

Agenda Item:	6.8.2.2

Document for:	Discussion and Decision

- Introduction

In this document, details on measurement gap configuration and request for NR positioning are discussed.

- Discussion on gap configuration for NR positioning

In Rel-16 NR positioning, it was agreed that the UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made within a configured measurement gap. However, based on measurement gap configuration specified below in TS38.331[1], the maximum gap length is 6ms, which may only be applicable for continuous measurement with a length of 5ms in FR1 and 5.5ms in FR2.

GapConfig ::= SEQUENCE { gapOffset INTEGER (0..159), mgl ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6}, mgrp ENUMERATED {ms20, ms40, ms80, ms160}, mgta ENUMERATED {ms0, ms0dot25, ms0dot5}, ..., [[ refServCellIndicator ENUMERATED {pCell, pSCell, mcg-FR2} OPTIONAL -- Cond NEDCorNRDC ]] }

From our point of views, the length of measurement gap may cause limitation for DL PRS measurement, since the time duration spanned by continuous PRS can easily exceed 5.5ms. The duration of continuous PRS at least depends on the following parameters:

- DL-PRS-ResourceRepetitionFactor. It defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and takes values TrepPRS∈1,2,4,6,8,16,32.

- DL-PRS-ResourceTimeGap. It defines the offset in number of slots between two repeated instances of a DL PRS resource with the same DL-PRS-ResourceID within a single instance of the DL PRS resource set and takes values TgapPRS∈1,2,4,8,16,32.This parameter together with the parameter ‘DL-PRS-ResourceRepetitionFactor’ can determine the time duration spanned by one instance of a DL PRS resource set.

- DL-PRS-NumSymbols. It defines the number of symbols of the DL PRS resource within a slot where the allowable values are {2,4,6,12}.

- Number of PRS resources within a PRS resource set. We may assume that the maximum number of PRS resources configured for measurement is 4 or 8 for FR1 and 64 for FR2 at most.

- DL-PRS-MutingPattern. It defines a bitmap of the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. In the second option, each bit in the bitmap corresponds to a single repetition index for each of the DL PRS resources. The length of continuous repeated PRS resources depends on the length of consecutive bits in the bitmap .

Then we illustrate some cases in Table1 where the duration of continuous PRS is larger than measurement gap length.

    - Table 1 Illustrations of duration of continuous PRS

| FR/SCS | DL-PRS- NumSymbols | DL-PRS- ResourceRepetitionFactor | DL-PRS- ResourceTimeGap | Number of PRS resources per set | Total duration (ms) |
| --- | --- | --- | --- | --- | --- |
| FR1/15kHz | 12 | 8 | 1 | 1 | 8 |
| FR1/15kHz | 12 | 8 | 4 | 4 | 32 |
| FR1/15kHz | 6 | 4 | 2 | 4 | 8 |
| FR1/15kHz | 6 | 2 | 4 | 8 | 8 |
| FR1/30kHz | 12 | 32 | 1 | 1 | 16 |
| FR2/60kHz | 6 | 4 | 32 | 64 | 24 |
| FR2/120kHz | 6 | 4 | 32 | 64 | 12 |

In each case, the arrangement of multiple PRS resources with repetition in a set can be assumed as shown in Figure 1.

Figure 1 Time span of a PRS resource set (4 resources within a set, repetition factor=4)

It can be seen that, under different SCS configurations in both FR1 and FR2, there are cases where the time span of continuous PRS resources can be larger than 5.5ms. Therefore, we come to the following observation:

Observation1:

- For some specific configurations for NR positioning, the time span of continuous PRS resources within a PRS resource set can exceed the maximum length of measurement gap configuration.

If measurement gap is configured, the UE is able to measure DL PRS with large bandwidth outside the DL active BWP during the time window of gap length. However, if the UE is configured with continuous PRS resources larger than 5.5ms and a limited measurement gap(<=6ms), the accuracy may be only guaranteed within the time window of gap length. While for PRS resources outside the time window, UE may only perform measurement within the DL active BWP, which may degrade the accuracy. Using measurements with different accuracies within/outside the time window of a gap may reduce the overall accuracy for measurement. Therefore, extending measurement gap length for NR positioning measurement should be considered.

In addition to gap length, the applicability of the gap repetition period also needs to be considered. In current specification, PRS period takes values of 2μ{4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240}slots, where μ=0, 1, 2, 3 for SCS=15, 30, 60 and 120kHz respectively.  If the unit is converted to ms, the period of PRS can be configured very large such as {320, 640, 1280, 2560, 5120, 10240}ms.  However, based on current configuration of measurement gap, the maximum repetition period is only 160ms. For some scenarios where the UE measures different signals such as signals for RRM, signals for positioning, signals with different time offsets or period, it may make sense to configure gap repetition period to be smaller than PRS. Nevertheless, when the UE is only expected to measure PRS resources with large periodicity such as 10240ms within a measurement gap, the repetition period of 160ms may not be suitable and a larger repetition period for measurement gap is required. Besides that, introducing larger values of gap repetition period may not increase overhead, whereas add flexibility.

Proposal1:

- Extending measurement gap length and repetition period for NR positioning measurement should be considered.

- Discussion on gap request for NR positioning

Based on the RAN1 conclusion in TS38.214[3], when the UE is expected to measure the PRS outside the active BWP, it may request a measurement gap by a higher layer signaling. Therefore, we propose:

Proposal2:

- RAN2 should design a higher layer signaling for the UE to request a measurement gap for NR positioning.

In TS38.331, this signaling is related to the IE ‘LocationMeasurementInfo’ as below.

LocationMeasurementInfo information element

-- ASN1START

-- TAG-LOCATIONMEASUREMENTINFO-START

LocationMeasurementInfo ::=     CHOICE {

eutra-RSTD                  EUTRA-RSTD-InfoList,

...,

eutra-FineTimingDetection    NULL

}

EUTRA-RSTD-InfoList ::= SEQUENCE (SIZE (1..maxInterRAT-RSTD-Freq)) OF EUTRA-RSTD-Info

EUTRA-RSTD-Info ::= SEQUENCE {

carrierFreq                 ARFCN-ValueEUTRA,

measPRS-Offset              INTEGER (0..39),

...

}

-- TAG-LOCATIONMEASUREMENTINFO-STOP

-- ASN1STOP

This IE is configured for UE to request gap for E-UTRA inter-frequency RSTD measurements. And in this case, only Gap pattern#0 (with 6ms length and 40ms periodicity) can be used, so that the UE does not need to indicate gap pattern ID in this signaling. For NR positioning, based on our analysis from the previous section, flexible configurations are required for different scenarios and only one Gap pattern used by the UE such as Gap pattern#0 may be not applicable. For Rel-16 positioning, we think more gap patterns should be provided by RAN4.

Proposal 3:

- An LS should be sent to RAN4 for gap patterns used for NR positioning.

Furthermore, the UE should provide a recommend measurement gap pattern from multiple gap patterns. In addition, for a certain gap pattern selected by the UE, the related gap offset and gap TA should also be recommended to the Network.

Proposal 4:

- When requesting a measurement gap, the UE should provide a recommend measurement gap pattern from multiple gap patterns.

- The related gap offset and gap TA should also be recommended to the Network.

In addition, considering the UE can be configured with multiple positioning frequency layers and the UE is assumed to perform measurement for a positioning frequency layer in a measurement gap, we think it is better to associate the request signaling IE with positioning frequency layer.

Proposal 5:

- When requesting a measurement gap, support to associate the request signaling IE with positioning frequency layer.

- Conclusion

Based on the discussion in section 2 and 3, we observe and propose the following:

Observation1:

- For some specific configurations for NR positioning, the time span of continuous PRS resources within a PRS resource set can exceed the maximum length of measurement gap configuration.

Proposal1:

- Extending measurement gap length and repetition period for NR positioning measurement should be considered.

Proposal2:

- RAN2 should design a higher layer signaling for the UE to request a measurement gap for NR positioning.

Proposal 3:

- An LS should be sent to RAN4 for gap patterns used for NR positioning.

Proposal 4:

- When requesting a measurement gap, the UE should provide a recommend measurement gap pattern from multiple gap patterns.

- The related gap offset and gap TA should also be recommended to the Network.

Proposal 5:

- When requesting a measurement gap, support to associate the request signaling IE with positioning frequency layer.

- Reference

- TS38.331 “Radio Resource Control (RRC) protocol specification”, v16.0.0, 2020-03

- Report of 3GPP TSG RAN WG2 meeting #109-e, 24 Feburary-06 March, 2020

[3]. TS38.214 “Physical layer procedures for data”, v16.0.0, 2020-03