3GPP TSG-RAN WG2 Meeting #109bis	R2-2002661

Elbonia, 20 April – 24 April 2020

Agenda item:	6.4.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Further issues on the mismatch of UE capabilities in unicast sidelink

WID/SID:	5G_V2X_NRSL-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In the RAN2#109e [1] meeting the AS configuration failure handling was discussed in [2] and the following agreements were included:

RRC: 3. The RRC connected TX UE reports a new failure cause to the NW upon the reception of RRCReconfigurationFailureSidelink from the RX UE 4. In case an AS configuration failure message is received from the RX UE, the TX UE shall not apply the SLRB configuration(s), which were included in the corresponding failed AS configuration message.

This contribution will discuss about the further issues of reporting the UE capability of the TX UE in the context of receiving a AS configuration failure.

# 2	Discussion

Before the discussion on the UE capability report for getting a sidelink configuration from the network, we would like to illustrate the problem that initiating UE-1 (TX UE) and peer UE-2 (RX UE) can have different UE capabilities. Let us take the example in Fig. 1, where the initiating UE-1 (TX UE) is in RRC CONNECTED mode and wants to establish sidelink communication with its peer UE-2 (RX UE). The initiating UE-1 (in resource allocation mode 1) gets the PC5 configuration via dedicated signalling based on its reported UE capability.

Figure 1 Unicast configuration with AS configuration failure due to different UE capabilities at TX UE and RX UE

An overview of the UE capabilities can be found in the UE feature list from RAN1 in [3]. The problem occurs if TX UE and RX UE have different UE capabilities. As a practical example let’s take the parameter 15-10 from section 15 5G_V2X_NRSL and assume two cases in Table 2:

Table 1: UE feature lists (example for NR sidelink)

| Features | Index | Feature Group | Component |
| --- | --- | --- | --- |
| 15 | 15-10 | 256QAM sidelink transmission | 1) UE can transmit PSSCH with 256QAM in NR sidelink |

Table 2: Example of two UEs in sidelink with support/no-support of Parameter 15-10

| case | TX UE supports 256QAM | RX UE supports 256 QAM |
| --- | --- | --- |
| A | Yes | No |
| B | No | Yes |

In both cases if the TX UE would report its own capabilities, the resulting PC5 configuration received from the serving network cannot be fulfilled by the RX UE. The RX UE feedbacks an AS configuration failure. As also [4] observes that “For unicast, the SL configuration/control from RAN depends on the capability of RX-UE. To solve this issue, the capability has to be reported to network.”

The discussion in [2] resulted in the agreement that the AS configuration failure message does not indicate the root cause of the failure. Neither the TX UE receiving the AS configuration message, nor the network receiving the RRCReconfiguration failure from its UE (UE-1) can know that the unicast PC5 connection establishment failed since the peer UE (RX UE) cannot comply with the PC5 configuration from the TX UE.

Observation 1: Different UE capabilities of initiating UE and peer UE cause AS configuration failure.

An apparent solution might that the TX UE reports the capability of its peer UE instead of its own UE capabilities as proposed in [5]. However, that would also cause an configuration failure as the example in Table 2 B shows: If the peer UE supports UE features that the TX UE does not, but the TX UE reports these UE capabilities as its own to the network, the TX UE can not comply with the PC5 configuration. So the option that the TX UE is not truthfully reporting its own UE capability, but rather a another UE’s capability set should be ruled out.

Proposal 1: The initiating UE should not report its peer UEs capabilities to the network, since the network would interpret this as the TX UE capabilities.

So RAN2 needs to define a solution for the reporting of UE capabilities to the network for setting up sidelink unicast link that both UEs can comply with even when TX UE and RX UE have different sets of UE capabilities.

Proposal 2: The network should ensure that the PC5 configuration for sidelink unicast can be applied by both Tx UE and Rx UE.

In order to fulfil proposal 2 the network configuring the unicast sidelink needs to be aware of both UE capabilities – hence one need to come up with a solution to transfer the UE capabilities of both UEs to the serving network.

Proposal 3: The initiating UE (TX UE) that wants to establish unicast sideling shall report its peer UE capability as well as its own UE capabilities.

There are several possibilities how to signal the UE capabilities of both UEs that want to communicate over unicast sidelink.

Alt1: The TX UE reports both UE capabilities to the network via a container (carrying RX UE capability) embedded in the TX UE capability report. The network will select the correct PC5 configuration that can be fulfilled by both UEs.

Alt2: The TX UE reports both UE capabilities to the network in separate UE capability report messages. The network will select the correct PC5 configuration that can be fulfilled by both UEs.

Proposal 4: RAN2 shall discuss how both UE capabilities are reported to the network (i.e. separate messages or container).

Proposal 5: It’s up to the serving network to configure a PC5 unicast connection that both UEs can fulfil.

# 3	Conclusion

Observation 1: Different UE capabilities of initiating UE and peer UE cause AS configuration failure.

Proposal 1: The initiating UE should not report its peer UEs capabilities to the network, since the network would interpret this as the TX UE capabilities.

Proposal 2: The network should ensure that the PC5 configuration for sidelink unicast can be applied by both Tx UE and Rx UE.

Proposal 3: The initiating UE (TX UE) that wants to establish unicast sideling shall report its peer UE capability as well as its own UE capabilities.

Proposal 4: RAN2 shall discuss how both UE capabilities are reported to the network (i.e. separate messages or container).

Proposal 5: It’s up to the serving network to configure a PC5 unicast connection that both UEs can fulfil.

# 4	References

[1] R2-200xxxx 	Report from session on LTE V2X and NR V2X

[2] R2-2001965 	Report for the offline discussion on Category-2 proposals in RRC summary

[3] R1-1911751	RAN1 UE features list for Rel-16 NR

[4] R2-2000193	Left issues on SL capability

[5] R2-2001418 	Remaining issue on sidelink AS configuration