3GPP TSG-RAN WG2 Meeting #109bis	R2-2002662

Elbonia, 20 April – 24 April 2020

Agenda item:	6.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Minor Correction in TS38.300 on SL physical layer measurements

WID/SID:	5G_V2X_NRSL-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In the last RAN2#109e V2X meeting [1] the NR-SL related agreements have been formulated in the running CR TS 38.300 in R2-2002264. Section 5.x.5 contains input from LS R1-2001356 that has been sent from RAN1 to RAN2 and directly put into the TS38.300 document.

This TDoc is correcting a error in section 5.x.5 of TS38.300 that has been inherited from the RAN1 input.

# 2	Discussion

In section 5.x.5 “Physical layer measurement definition” of TS38.300 CR the abbreviation “PSBCH-SL RSRP” contains the term “sidelink“ twice and is inconsistent with the naming convention of the other measurement reports “PSSCH-RSRP” and “PSCCH-RSRP”. Apparently this small typo has been inserted from the RAN1 LS R1-2001356.

Observation 1: “PSBCH-SL RSRP” is an incorrect abbreviation for “Physical Sidelink Broadcast Channel Reference Signal Received Power”.

TS38.215 can serve as a good reference point for the correct usage of the abbreviation “PSBCH-RSRP”

5.1.22.......... PSBCH reference signal received power (PSBCH-RSRP).................................................................. 16

5.1.23.......... PSSCH reference signal received power (PSSCH-RSRP)................................................................... 16

5.1.24.......... PSСCH reference signal received power (PSCCH-RSRP).................................................................. 16

5.1.25.......... Sidelink received signal strength indicator (SL RSSI)......................................................................... 17

5.1.26.......... Sidelink channel occupancy ratio (SL CR).......................................................................................... 17

5.1.27.......... Sidelink channel busy ratio (SL CBR)................................................................................................. 17

### 5.1.22    PSBCH reference signal received power (PSBCH-RSRP)

| Definition | PSBCH Reference Signal Received Power (PSBCH-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry demodulation reference signals associated with physical sidelink broadcast channel (PSBCH). For frequency range 1, the reference point for the PSBCH RSRP shall be the antenna connector of the UE. For frequency range 2, PSBCH-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported PSBCH-RSRP value shall not be lower than the corresponding PSBCH-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED inter-frequency |

Proposal 1: Correct TS38.300 section 5.x.5 and replace “PSBCH-SL RSRP” by “PSBCH-RSRP”.

# 3	Conclusion

Observation 1: “PSBCH-SL RSRP” is an incorrect abbreviation for “Physical Sidelink Broadcast Channel Reference Signal Received Power”.

Proposal 1: Correct TS38.300 section 5.x.5 and replace “PSBCH-SL RSRP” by “PSBCH-RSRP”.

# 4	References

[1] R2-200xxxx	Report from session on LTE V2X and NR V2X (V2X session chair)

[2] R2-2002264 	Running CR TS38.300

[3] R1-2001356	LS on RAN1 input to Rel-16 TS38.300 on V2X, Positioning and MR-DC NR

[4] TS38.215 		NR Physical Layer Measurements