3GPP TSG-RAN WG2 Meeting #109bis-e	               R2-2002717

Electronic, 20 April – 30 April 2020

Agenda Item:	6.1.8

Source:	AT&T

Title:	Views on NR Rel-16 IAB Features

Document for:	Discussion/Approval

# Introduction

This contribution discusses the definition of IAB-MT features.

# Mandatory IAB-MT Features

During RAN#87e the following agreements were reached:

- RAN WGs to investigate which of the mandatory Rel-15 UE features (as defined in TR 38.822) can be optional for basic operation of (and if found useful, for different classes of IAB-MTs as defined by RAN4).

- RAN WGs should strive to minimize specification impact.

Given that an IAB node is a fixed network node in Rel-16, implementation of the IAB-MT function may be simplified compared to a normal access UE, although the IAB-MT inherits the majority of the Rel-15/Rel-16 UE functionality of the Uu link upon which the backhaul link is based.

In many cases Rel-15 mandatory features have capability signaling which can be used by IAB nodes to indicate if a certain feature is not implemented, however this is not possible for mandatory features without capability signaling.

The vast majority of those features are considered as ‘basic’ or ‘essential’ to the operation of the Uu link, however Table 1 lists Rel-15 mandatory features without capability signaling (as defined in 38.822) which can be candidates for RAN2 to consider making optional for IAB-MTs:

Table 1. Mandatory Rel-15 UE Features / Optional Rel-16 MT Features

| 38.822 Index | Feature Group | Note |
| --- | --- | --- |
| 0-3 | DRB | Can be optional for MTs (e.g. to support OAM traffic) |
| 5-1 | QoS | Not needed for MTs since DRB is not for backhaul traffic |
| 5-2 | HD format | Not needed for MTs since DRB is not for backhaul traffic |

Proposal 1: The following Rel-15 mandatory features without capability signaling (as defined in 38.822) are optional for IAB-MTs:

0-3	DRB

5-1	QoS

5-2	HD format

# Conclusion

The following proposals were made:

Proposal 1: The following Rel-15 mandatory features without capability signaling (as defined in 38.822) are optional for IAB-MTs:

0-3	DRB

5-1	QoS

5-2	HD format