3GPP TSG-RAN2 Meeting #109bis-e	R2-2002789

# Electronic, 20 April – 30 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | draftCR | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the Configuration of sCellState [C101] [C102] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is not clear the UE behaviour if the field sCellState is absent, i.e. the need code for the field is not clear. According to the procedure text in 5.3.5.5.9, if the field is absent, the UE will consider the SCell to be deactivated state, which should be need S. If the need code of the field sCellState is need S, the NW should always configure this field upon UE reconfiguration other field if the NW want to keep the SCell to be activated state, which is inconsistency with the present condition, which is specified the field should only be present upon the SCell addition, reconfiguration with sync, and resuming an RRC connection, it is absent otherwise, that is the field will not be present upon the SCell modification. So we propose the field to be need N. if the field is absent, the UE will keep the current state for this SCell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. delete the clause that the UE considered the SCell to be deactivated state if the field is absent upon SCell modification in 5.3.5.5.9. 2. add the need code for the field sCellState to be need N. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SCell state can’t be configured correctly. Due to inconsistency of the procedure text and the present condition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.5.9, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.3.5.5.9	SCell Addition/Modification

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated;

2>	if the sCellState is included and set to activated:

3>	configure lower layers to consider the SCell to be in activated state;

2>	else:

3>	configure lower layers to consider the SCell to be in deactivated state;

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the sCellConfigDedicated;

2>	if the sCellState is included and set to activated:

3>	configure lower layers to consider the SCell to be in activated state;

NEXT OF CHANGE

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells).

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                        SEQUENCE {

cellGroupId                                CellGroupId,

rlc-BearerToAddModList                     SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig                    OPTIONAL,   -- Need N

rlc-BearerToReleaseList                    SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity              OPTIONAL,   -- Need N

mac-CellGroupConfig                        MAC-CellGroupConfig                                                 OPTIONAL,   -- Need M

physicalCellGroupConfig                    PhysicalCellGroupConfig                                             OPTIONAL,   -- Need M

spCellConfig                               SpCellConfig                                                        OPTIONAL,   -- Need M

sCellToAddModList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig                   OPTIONAL,   -- Need N

sCellToReleaseList                         SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex                    OPTIONAL,   -- Need N

...,

[[

reportUplinkTxDirectCurrent                ENUMERATED {true}                                                   OPTIONAL    -- Cond BWP-Reconfig

]],

[[

bap-Address-r16                            BIT STRING (SIZE (10))                                              OPTIONAL,   -- Need M

bh-RLC-ChannelToAddModList-r16             SEQUENCE (SIZE(1..maxLC-ID-Iab-r16)) OF BH-RLC-ChannelConfig-r16    OPTIONAL,   -- Need N

bh-RLC-ChannelToReleaseList-r16            SEQUENCE (SIZE(1..maxLC-ID-Iab-r16)) OF BH-LogicalChannelIdentity-r16 OPTIONAL, -- Need N

dormancySCellGroups                        DormancySCellGroups                                                 OPTIONAL,   -- Need N

simultaneousTCI-UpdateList-r16             SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex    OPTIONAL,   -- Need R

simultaneousTCI-UpdateListSecond-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex    OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList-r16        SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex    OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedListSecond-r16  SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex    OPTIONAL    -- Need R

]]

}

DormancySCellGroups::=               SEQUENCE {

withinActiveTimeToAddModList         SEQUENCE (SIZE (1..maxNrofDormancyGroups)) OF DormancyGroup-r16    OPTIONAL,   -- Need N

withinActiveTimeToReleaseList        SEQUENCE (SIZE (1..maxNrofDormancyGroups)) OF DormancyGroupID-r16  OPTIONAL,   -- Need N

outsideActiveTimeToAddModList        SEQUENCE (SIZE (1..maxNrofDormancyGroups)) OF DormancyGroup-r16    OPTIONAL,   -- Cond DormancyWUS

outsideActiveTimeToReleaseList       SEQUENCE (SIZE (1..maxNrofDormancyGroups)) OF DormancyGroupID-r16  OPTIONAL    -- Need N

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=                        SEQUENCE {

servCellIndex                       ServCellIndex                                               OPTIONAL,   -- Cond SCG

reconfigurationWithSync             ReconfigurationWithSync                                     OPTIONAL,   -- Cond ReconfWithSync

rlf-TimersAndConstants              SetupRelease { RLF-TimersAndConstants }                     OPTIONAL,   -- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                                             OPTIONAL,   -- Need S

spCellConfigDedicated               ServingCellConfig                                           OPTIONAL,   -- Need M

...

}

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                         OPTIONAL,   -- Need M

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                                               OPTIONAL,   -- Need N

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]]

}

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL    -- Need SCellAddSync

]]}

DormancyGroup-r16 ::=               SEQUENCE {

dormancyGroupID-r16                 DormancyGroupID-r16,

dormancySCellList-r16               SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex

}

DormancyGroupID-r16 ::=             INTEGER (0..4)

-- TAG-CELLGROUPCONFIG-STOP

-- ASN1STOP

CellGroupConfig field descriptions

bap-Address BAP address of node that is hosting this cell group.

bh-RLC-ChannelToAddModList Configuration of the MAC Logical Channel, the corresponding backhaul RLC enitities to be added and modified.

bh-RLC-ChannelToReleaseList List of MAC Logical Channel, the corresponding backhaul RLC enitities to be released.

mac-CellGroupConfig MAC parameters applicable for the entire cell group.

rlc-BearerToAddModList Configuration of the MAC Logical Channel, the corresponding RLC entities and association with radio bearers.

reportUplinkTxDirectCurrent Enables reporting of uplink and supplementary uplink Direct Current location information upon BWP configuration and reconfiguration. This field is only present when the BWP configuration is modified or any serving cell is added or removed. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. If UE is configured with SUL carrier, UE reports both UL and SUL Direct Current locations.

rlmInSyncOutOfSyncThreshold BLER threshold pair index for IS/OOS indication generation, see TS 38.133 [14], table 8.1.1-1. n1 corresponds to the value 1. When the field is absent, the UE applies the value 0. Whenever this is reconfigured, UE resets N310 and N311, and stops T310, if running. Network does not include this field.

sCellState Indicates whether the SCell shall be considered to be in activated state upon SCell configuration.

sCellToAddModList List of secondary serving cells (SCells) to be added or modified.

sCellToReleaseList List of secondary serving cells (SCells) to be released.

simultaneousTCI-UpdateList, simultaneousTCI-UpdateListSecond List of serving cells which can be updated simultaneously for TCI relation with a MAC CE. The simultaneousTCI-UpdateList and simultaneousTCI-UpdateListSecond shall not contain same serving cells.

simultaneousSpatial-UpdatedList, simultaneousSpatial-UpdatedListSecond List of serving cells which can be updated simultaneously for spatial relation with a MAC CE. The simultaneousSpatial-UpdatedList and simultaneousSpatial-UpdatedList shall not contain same serving cells.

spCellConfig Parameters for the SpCell of this cell group (PCell of MCG or PSCell of SCG).

DormancyGroup field descriptions

dormancySCellList List of SCells within the same SCell dormancy group.

dormancyGroupID The field indicates an SCell group corresponding to the explicit information field in DCI, i.e., bitmap with 1 bit per DormancyGroup for indicating dormancy/non-dormancy of SCells, as specified in TS 38.213.

DormancySCellGroups field descriptions

outsideActiveTimeToAddModList List of Dormancy outside active time SCell groups to be added or modified. The use of the Dormancy outside active time SCell groups is specified in TS 38.213 [13].

withinActiveTimeToAddModList List of Dormancy within active time SCell groups SCell groups to be added or modified. The use of the Dormancy within active time SCell groups is specified in TS 38.213 [13].

ReconfigurationWithSync field descriptions

rach-ConfigDedicated Random access configuration to be used for the reconfiguration with sync (e.g. handover). The UE performs the RA according to these parameters in the firstActiveUplinkBWP (see UplinkConfig).

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and NR PCell change. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon. For case of NR PCell change, the smtc is based on the timing reference of source PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

SCellConfig field descriptions

smtc The SSB periodicity/offset/duration configuration of target cell for NR SCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in sCellConfigCommon. The smtc is based on the timing of the SpCell of associated cell group. In case of inter-RAT handover to NR, the timing reference is the NR PCell. In case of intra-NR PCell change (standalone NR) or NR PSCell change (EN-DC), the timing reference is the target SpCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

SpCellConfig field descriptions

reconfigurationWithSync Parameters for the synchronous reconfiguration to the target SpCell.

rlf-TimersAndConstants Timers and constants for detecting and triggering cell-level radio link failure. For the SCG, rlf-TimersAndConstants can only be set to setup and is always included at SCG addition.

servCellIndex Serving cell ID of a PSCell. The PCell of the Master Cell Group uses ID = 0.

| Conditional Presence | Explanation |
| --- | --- |
| BWP-Reconfig | The field is optionally present, Need N, if the BWPs are reconfigured or if serving cells are added or removed. Otherwise it is absent. |
| DormancyWUS | The field is optionally present, Need N, if WUS is configured; otherwise it is absent. |
| ReconfWithSync | The field is mandatory present in case of SpCell change, PSCell addition, SCG resume with NR-DC or (NG)EN-DC, update of required SI for PSCell, and AS security key change; otherwise it is optionally present, need M. The field is absent in the masterCellGroup in RRCResume and RRCSetup messages. |
| SCellAdd | The field is mandatory present upon SCell addition; otherwise it is absent, Need M. |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |
| SCellAddSync | The field is optional present in case of SCell addition, reconfiguration with sync, and resuming an RRC connection. It is absent otherwise, need N. |
| SCG | The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise. |

END OF CHANGE