3GPP TSG-RAN WG2 Meeting #109bis	electronic							    R2-2002873

E-Meeting, 20th Apr – 30th Apr, 2020

Source: 	vivo

Title:  	Correction on the SP SRS Activation/Deactivation MAC CE

Agenda Item:	6.16.3

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the issue of the the SP SRS Activation/Deactivation MAC CE due to the change of the value range for the NZP CSI-RS resource in Rel-15. This issue was discussed in Rel-15, and considered to be resolved in the Rel-16 eMIMO WI by many companies.

# Discussion

## Issue on the NZP CSI-RS resource ID

According to the discussion in the RAN2#107 meeting as quoted below,

R2-1911536 Summary of the discussion on CSI-RS resource ID for SP SRS activation vivo - Vivo report from the offline that the proposed statement can be confirmed Conclusion “NZP CSI-RS resource index of the SP SRS Activation/Deactivation MAC CE refers to the nzp-CSI-RS-ResourceId” is agreed. No TS change required

According to 38.321, the SP SRS Activation/Deactivation MAC CE has an 7-bits “Resource ID” field which can indicate up to 128 resource IDs (e.g. “NZP CSI-RS resource index”) as configured by RRC 38.331. However the RRC specification can configure up to 192 NZP CSI-RS resource(s). Then the NZP CSI-RS resource with the ID value of the NZP CSI-RS resource more than 127 cannot be used for the spatial relationship derivation for SRS.

The collided specification texts are quoted as follows:

38.321: Figure 6.1.3.17-1: SP SRS Activation/Deactivation MAC CE - Fi: This field indicates the type of a resource used as a spatial relationship for SRS resource within SP SRS Resource Set indicated with SP SRS Resource Set ID field. F0 refers to the first SRS resource within the resource set, F1 to the second one and so on. The field is set to "1" to indicate NZP CSI-RS resource index is used, it is set to "0" to indicate either SSB index or SRS resource index is used. The length of the field is 1 bit. This field is only present if MAC CE is used for activation, i.e. A/D field is set to "1"; - Resource IDi: This field contains an identifier of the resource used for spatial relationship derivation for SRS resource i. Resource ID0 refers to the first SRS resource within the resource set, Resource ID1 to the second one and so on. If Fi is set to "0" and the first bit of this field is set to "1", then the remainder of this field contains SSB-Index as specified in TS 38.331 [5], if Fi is set to "0" and the first bit of this field is set to "0" then the remainder this field contains SRS-ResourceId as specified in TS 38.331 [5]. The length of the field is 7 bits. This field is only present if MAC CE is used for activation, i.e. A/D field is set to "1";

38.331: CSI-MeasConfig ::= SEQUENCE { nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource OPTIONAL, -- Need N nzp-CSI-RS-ResourceToReleaseList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId OPTIONAL, -- Need N NZP-CSI-RS-Resource ::= SEQUENCE { nzp-CSI-RS-ResourceId NZP-CSI-RS-ResourceId, resourceMapping CSI-RS-ResourceMapping, powerControlOffset INTEGER (-8..15), powerControlOffsetSS ENUMERATED{db-3, db0, db3, db6} OPTIONAL, -- Need R scramblingID ScramblingId, periodicityAndOffset CSI-ResourcePeriodicityAndOffset OPTIONAL, -- Cond PeriodicOrSemiPersistent qcl-InfoPeriodicCSI-RS TCI-StateId OPTIONAL, -- Cond Periodic ... } NZP-CSI-RS-ResourceId ::= INTEGER (0..maxNrofNZP-CSI-RS-Resources-1) maxNrofNZP-CSI-RS-Resources INTEGER ::= 192 -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources maxNrofNZP-CSI-RS-Resources-1 INTEGER ::= 191 -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

The issue is due to previous LS in R1-1801272 which explicitly states that the size of CSI RS resource ID is smaller than or equal to 7 bits. But later on 2018 RAN1 #92 further agreed that the maximum number of Non-Zero-Power (NZP) CSI-RS resources could be 192.

| TS38.214 | 6.2.1 | Semi-persistent SRS activation | Activates a SP SRS resource set and provides the spatial QCL relationship (if activated) | SP SRS Resource Set Id (the size of ID<=[SRS1=4]bits) ¦ SSB ID (the size of ID <=6bits) /SRS resource ID (the size of ID<=[SRS2=5]bits) / CSI-RS resource ID (the size of ID <=7bits) Each activated resource set can have up to S2 SRS resources therefore Total bits<=SRS1+SRS2*7 |
| --- | --- | --- | --- | --- |

The updated RAN1 agreement is as follows:

Agreement (RRC parameter update): maxNrofNZP-CSI-RS-Resources-1 is 191

Observation: The “Resource ID” field can only indicate 128 NZP-CSI-RS-ResourceId which is currently supporting 192.

To correct the observed issue, we consider that the SP SRS Activation/Deactivation MAC CE can be re-designed to support 192 NZP CSI-RS resource(s).

Proposal: To re-design the SP SRS Activation/Deactivation MAC CE to support 192 NZP CSI-RS resource(s).

# Conclusions

Based on the analysis given above, we have the following Observation and Proposal:

Observation: The “Resource ID” field can only indicate 128 NZP-CSI-RS-ResourceId which is currently supporting 192.

Proposal: To re-design the SP SRS Activation/Deactivation MAC CE to support 192 NZP CSI-RS resource(s).

# Reference

[1] RAN2#107 meeting minutes