3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2002904

Online, 20th – 24th April 2020

Agenda Item		:	6.9.4.2

Source		:	LG Electronics Inc.

Title		:	Consideration on CPC Capability

Document for		:	Discussion and Decision

1. Introduction

In this contribution, for Rel-16, we discuss CPC capability issue which sub-functions should be specified especially based on the progress of the CHO capability.

2. Discussion

For Rel-16 CPC, since we expect the basic function of performing CPC can be mostly complete in this meeting. We think RAN2 needs to discuss CPC capability. We consider 4 possible cases of CPC capabilities as like in the discussion of the CHO capabilities.

2.1 Capability for the handling of CPC configuration

This is obvious to indicate whether the UE can support CPC function. This capability is specified not only for the handling of CPC configuration but also including execution condition and candidate cell configuration. The capability information can be similar to the CHO capability. We think this capability should be separated from CHO capability.

Proposal 1: Capability information for the handling of CPC configuration is needed.

2.2 Capability for maximum candidate cells

In the RAN2#109 meeting [1], RAN2 agreed that the UE can support at most 8 candidate cells for CHO configuration. Then, in the current specification, same IE is being used for CHO and CPC, it is likely that the UE also can support at most 8 candidate cells for CPC. However, for CHO, it is still FFS whether the UE should be able to indicate the maximum number of candidate cells it supports, e.g. the value less than 8, RAN2 need to discuss whether the UE should be able to indicate the maximum number of candidate cells it supports for CPC also.

In our view, this capability information for maximum candidate cells is not essential. This is because the UE already measures multiple frequencies and report multiple neighbour cells. Considering the maximum number of inter-frequency to be measured is 8, the UE is capable to evaluate multiple candidate cells even though all candidate cells are on different inter frequencies. Moreover, since the maximum number of cell to be reported is 8, the UE is also already capable to evaluate multiple candidate cells even if all candidate cells are on intra-frequency. With the above reasons, in the current specification, this capability information doesn’t have new things compared to the legacy measurement related capabilities, we think the capability information for maximum candidate cells isn’t needed.

Proposal 2: No capability information for maximum candidate cells is needed.

2.3 Capability for CPC failure handling

In the RAN2#109 meeting [1], RAN2 agreed that the UE sends SCG failure information when CPC is failed. In our view, the UE should wait for the network response to resolve the failure situation and no further CPC is allowed. This is because the UE may not receive the updated configuration if the CPC is being executed while the network modifies the CPC configuration or release the SCG configuration after SCG failure. Therefore, we think the capability information for CPC failure handling isn’t needed in Rel-16.

Proposal 3: No capability information for CPC failure handling is needed in Rel-16.

2.4 Capability for 2 trigger events for one execution condition

We think this feature should be optional. This is because configuring multiple trigger events for CPC is not always required but only configured for limited cases, e.g. when more reliable CPC execution is necessary. Without enabling this multiple trigger configuration, CPC can be configured and executed. That is, this capability is not essential for CPC. Therefore, we think the capability of supporting multiple trigger events should be optional and the support of this capability should be signalled.

Proposal 4: The capability of supporting multiple trigger events for one execution condition is specified as an optional capability with signalling.

3. Conclusion

In this contribution, we have the following conclusion:

Proposal 1: Capability information for the handling of CPC configuration is needed..

Proposal 2: No capability information for maximum candidate cells is needed.

Proposal 3: No capability information for CPC failure handling is needed.

Proposal 4: The capability of supporting multiple trigger events for one execution condition is specified as an optional capability with signalling.

4. Reference

[1] R2-2001661 Minutes LTE legacy, LTE TEI16 and NR/LTE Rel-16 Mobility topics, Vice Chairman (Nokia)

5. Appendix

RAN2 UE feature list for Mobility enhancement is referenced by the discussion of [AT109e][211], R2-2001727. For readability, some parts are merged or removed.

RAN2 UE feature list on NR mobility enhancement

| Features | Index | Feature group | Components | Need for the gNB to know if the feature is supported | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x. Rel-16 NR Mobility Enhancement | X-0 | DAPS | - FFS Support of DAPS handover Consequence if the feature is not supported by the UE: The network cannot configure UE with DAPS HO | Yes | FFSPer BC | Yes | Yes | N/A | This is the general capability to cover default capabilities of DAPS HO, e.g. sync; dual UL, etc | Optional with capability signalling |
|  | x-1 | CHO | - Handling of CHO configuration, including execution condition and candidate cell configuration; - FFS Maximum candidate cells the UE can support; Default value FFS - CHO based failure handling; - FFS support of 2 trigger events for one execution condition Consequence if the feature is not supported by the UE: 1) The network cannot configure UE with CHO. 2) the network cannot configure candidate cells more than the max number the UE supported; 3) the network cannot configure the UE to perform CHO based failure handling 4) the network cannot configure 2 trigger events for one execution condition | Yes | per UE | Yes | Yes | FFS |  | Optional with capability signalling (1 and, 3) |
|  | x-2 | T312 based PCell fast failure recovery | - T312 based PCell fast failure recovery Consequence if the feature is not supported by the UE: The network cannot configure UE with T312 based PCell fast failure recovery. | Yes | per UE | Yes | Yes | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling (1) |
|  | X-3 | T312 based PSCell fast failure recovery | - T312 based PSCell fast failure recovery Consequence if the feature is not supported by the UE: The network cannot configure UE with T312 based PSCell fast failure recovery | Yes | per UE | Yes | Yes | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling (1) |

RAN2 UE feature list on LTE mobility enhancement

| Features | Index | Feature group | Components | Need for the eNB to know if the feature is supported | Type | Need of FDD/TDD differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x. Rel-16 LTE Mobility Enhancement | X-0 | DAPS | - FFS Support of DAPS handover Consequence if the feature is not supported by the UE: The network cannot configure UE with DAPS HO | Yes | FFS Per BC | N/A | This is the general capability to cover default capabilities of DAPS HO, e.g. sync; dual UL, etc | Optional with capability signalling |
|  | x-1 | CHO | - Handling of CHO configuration, including execution condition and candidate cell configuration; - FFS Maximum candidate cells the UE can support; Default value FFS - CHO based failure handling; - FFS support of Multiple trigger events for one execution condition Consequence if the feature is not supported by the UE: 1) The network cannot configure UE with CHO. 2) the network cannot configure candidate cells more than the max number the UE supported; 3) the network cannot configure the UE to perform CHO based failure handling 4) the network cannot configure 2 trigger events for one execution condition | Yes | per UE | Yes |  | Optional with capability signalling (1 and 3) |