3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003065

Electronic Meeting, 20th April – 1st May 2020

Agenda item:	6.8.3

Source:	Huawei, HiSilicon

Title:	Discussion on UL E-CID

Document for:	Discussion and Decision

1	Introduction

In the current running CR for NR E-CID [1][2], only the downlink E-CID is specified while whether uplink E-CID (UL-ECID) should be supported is listed as FFS according to the discussions of RAN2#109e. So in this document, we provide our views on UL-ECID for the stage2 running CR.

2	Support of NR UL E-CID

We believe a general procedure of handling the signalling flow is within RAN2’s scope, since RAN2 is in charge of the stage-2 specification. In particularly, the required signalling between gNB and LMF to allow a positioning method can and should be discussed in RAN2. For example, we have already discussed without RAN3 involvement the procedures to enable UL-only positioning, to enable multi-RTT positioning, and even assistance data delivery for DL-only positioning. That being said, UL E-CID, which exists for LTE, should be supported in stage-2 and the details can be implemented in RAN3 specification.

In terms of details of NR UL E-CID, we would like to clarify that in Rel-16, UL NR E-CID does not require any gNB measurement, so that a general Rel-16 NR E-CID is only based on UE RRM measurements (RSRP/RSRQ), as indicated in the RAN1 LS. Cell portion information similar to NR CID in Rel-15 can be combined with the NR UL E-CID. It is up to RAN3 to decide what NRPPa message will be used to convey the information to support UL E-CID.

The benefit is that UE does not have to support LPP protocol stack, and it is up to network to use what UE already have in Rel-15 RRM to find the location, for e.g. regulatory use cases. The benefit is remarkably significant when it comes to positioning Rel-15 UE that does not have any Rel-16 NR positioning capabilities.

- Proposal 1: Support NR UL E-CID in stage 2 specification, where the signaling flow for NR UL E-CID is similar to LTE E-CID, and the measurement to be transferred between serving gNB and LMF is only the Cell ID information and the RSRP/RSRQ measurement received from the UE.

- Proposal 2: Send an LS to RAN3 to inform them of the decision.

3	Conclusions

In this document, we provide our views on UL-ECID for the stage2 running CR. We have the following proposals:

- Proposal 1: Support NR UL E-CID in stage 2 specification, where the signaling flow for NR UL E-CID is similar to LTE E-CID, and the measurement to be transferred between serving gNB and LMF is only the Cell ID information and the RSRP/RSRQ measurement received from the UE.

- Proposal 2: Send an LS to RAN3 to inform them of the decision.

4	References

- R2-2002243, “Introduction of NR positioning (37.355CR)”, Intel Corporation, Elbonia, 24th February – 6th March 2020.

- R2-2002399, “Introduction of NR positioning (38.305CR)”, Intel Corporation, Elbonia, 24th February – 6th March 2020.