3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003164

Elbonia, 20 – 30 April 2020

Agenda item:	6.12.4

Source:	Nokia, Nokia Shanghai Bell

Title:	N018 Actions upon successful completion of random-access procedure

WID/SID:	NR_SON_MDT-Core - Release 16

Document for:	Discussion and Decision

# Introduction

This contribution addresses issue N018 raised in Rel-16 ASN.1 RIL. The issue requires a correction to TS38.331 to address a misalignment between an agreement from RAN2 #109-e and what is reflected in RRC.

# 2	Discussion

- The UE will stop logging RA info if all 8 entries is filled in RA report, and starts to count the duration. If not fetched within 48 hours, then the whole RA report will be deleted.

In RAN2 #109e the following agreement was made on the UE RACH Report:

However, the following text was captured in TS38.331:

#### 5.7.x1.4	Actions upon successful completion of random-access procedure

The UE shall:

1>	if the number of RA-Report stored in the RA-ReportList is less than 8, then append the following contents associated to the successfully completed random-access procedure as a new entry in the VarRA-Report:

(…)

The UE may discard the random access report information, i.e. release the UE variable VarRA-Report, 48 hours after the last successful random access procedure related information is added to the VarRA-Report.

Observation 1: In TS 38.331 there is no indication on UE behavior when the number of RA-Report stored in RA-ReportList is equal to 8.

Observation 2: In existing TS38.331, the UE releases the UE variable VarRA-Report 48 hours after the last successful random access procedure unconditionally on whether all 8 entries in the RA Report are filled.

Observation 2 contradicts the agreement of RAN2 #109-e, wanting the UE to start counting the duration only when the RA Report is filled.

Proposal: Agree RRC changes to fix the issue described in Observation 2 as in the attached Annex.

# 3	Conclusion

Observation 1: In TS 38.331 there is no indication on UE behavior when the number of RA-Report sorted in RA-ReportList is equal to 8.

Observation 2: The UE releases the UE variable VarRA-Report 48 hours after the last successful random access procedure unconditionally on whether all 8 entries in the RA Report are filled.

Proposal: Agree RRC changes to fix the issue described in Observation 2 as in the attached Annex.

# Annex: 	Text Proposal to TS38.331

Modified Subclause

#### 5.7.x1.4	Actions upon successful completion of random-access procedure

The UE shall:

1>	if the number of RA-Report stored in the RA-ReportList is less than maxRAReport, then append the following contents associated to the successfully completed random-access procedure as a new entry in the VarRA-Report:

2>	if the list of EPLMNs has been stored by the UE:

3>	if the RPLMN is included in plmn-IdentityList stored in VarRA-Report:

4>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	else:

4>	clear the information included in VarRA-Report;

2>	else:

3>	set the plmn-Identity to the PLMN selected by upper layers from the PLMN(s) included in the plmn-IdentityList in SIB1;

2>	set the cellId to the global cell identity and the tracking area code of the cell in which the random-access procedure was performed;

2>	set the absoluteFrequencyPointA to indicate the absolute frequency of the reference resource block associated to the random-access resources;

2>	set the locationAndBandwidth and subcarrierSpacing associated to the UL BWP of the random-access resources;

2>	set the msg1-FrequencyStart, msg1-FDM and msg1-SubcarrierSpacing associated to the random-access resources;

2>	set the raPurpose to include the purpose of triggering the random-access procedure;

2> set the parameters associated to individual random-access attempt in the chronological order of attempts in the perRAInfoList as specified in 5.3.10.3:

- else

2>  consider the VarRA-Report is full and do not log further random access procedures.

The UE may discard the random access report information, i.e. release the UE variable VarRA-Report, 48 hours after the last successful random access procedure related information is added to the VarRA-Report (the VarRA-report is filled with the number of random access procedures reports equal to maxRAReport).