3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003255

Electronic, 20 April – 30 April 2020

Agenda item:	6.13.3

Source:	Qualcomm Incorporated

Title:	Remaining issue on 2-step CFRA

WID/SID:	NR_2step_RACH-Core – Release 16

Document for:	Discussion and Decision

# Introduction

In this paper, we would like to discuss the remaining issues on 2-step CFRA.

# Discussion

The 2-step CBRA UE capability is under discussion in RAN1 L1 feature list. According to the RAN planery guidance, the 2-step CFRA is only performed for handover use case in Rel-16 and should have no RAN1 impact. From RAN2 perspective, the additional work is whether to introduce 2-step CFRA UE capability as a separate capability.

For the legacy Rel-15 CFRA, there is already one UE capability defined for CFRA. If UE already supports this capability, UE should also support contention-free RA for 2-step RACH. Hence the additional capability is not needed for 2-step CFRA

| csi-RS-CFRA-ForHO Indicates whether the UE can perform reconfiguration with sync using a contention free random access on PRACH resources that are associated with CSI-RS resources of the target cell. | UE | No | No | No |
| --- | --- | --- | --- | --- |

Proposal 1: No additional UE capability signaling is introduced for 2-step CFRA.

In the latest Rel-16 RRC spec [1], the msgA-TransMax can be configured via dedicated signaling through rach-ConfigGenericTwoStepRA. For 2-step CFRA UE, it is possible that the target cell configures UE with smaller value of msgA-TransMax to allow UE fast switching to 4-step RACH to avoid taking relative long time on 2-step CFRA attempts. Since 2-step CFRA UE is configured with dedicated preamble and dedicated PUSCH resource, it usually does not need to attempt so many times like the 2-step CBRA case when performing the handover. Hence, the value of msgA-TransMax can be different from that configured in the RACH-ConfigCommonTwoStepRA.

– RACH-ConfigDedicated

CFRA-TwoStep field descriptions

rach-ConfigGenericTwoStepRA Configuration of contention free random access occasions for CFRA 2-step random access type. The UE shall ignore msgA-preambleReceivedTargetPower, preambleTransMax, msgA-powerRampingStep, msgB-ResponseWindow, msgA-TransMax signaled within this field and use the corresponding values provided in RACH-ConfigCommonTwoStepRA.

We propose that UE should ignore the msgA-TransMax configured in the RACH-ConfigCommonTwoStepRA and shall be allowed to use the corresponding value of msgA-TransMax in rach-ConfigGenericTwoStepRA configured in RACH-ConfigDedicated.

Proposal 2: UE should be allowed to configure and use the value of msgA-TransMax provided in the RACH-ConfigDedicated.

# Conclusion

We’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: No additional UE capability signaling is introduced for 2-step CFRA.

Proposal 2: UE should be allowed to configure and use the value of msgA-TransMax provided in the RACH-ConfigDedicated.

# References

- TS 38.331 V16.0.0, NR Radio Resource Control (RRC) protocol specification, 3GPP.

# Annex

– RACH-ConfigDedicated

The IE RACH-ConfigDedicated is used to specify the dedicated random access parameters.

CFRA-TwoStep field descriptions

rach-ConfigGenericTwoStepRA Configuration of contention free random access occasions for CFRA 2-step random access type. The UE shall ignore msgA-preambleReceivedTargetPower, preambleTransMax, msgA-powerRampingStep, msgB-ResponseWindow, msgA-TransMax signaled within this field and use the corresponding values provided in RACH-ConfigCommonTwoStepRA.