3GPP TSG-RAN WG2 #109bis-e	Tdoc R2-2003307

Electronic meeting, 20th - 30th April, 2020

Agenda Item:	5.4.3

Source:	Ericsson

Title:	Bands in supportedBandListNR

Document for:	Discussion, Decision

# 1	Introduction

This document discusses UE inclusion of supported bands in UE capability field supportedBandListNR.

# 2	Discussion

TS 38331 describes how UE shall set band combinations, feature set combinations and feature sets supported by the UE (for NR SA and MR-DC). But it is not specified that UE is also required to indicate all its supported bands in supportedBandListNR.

This means a UE could in principle indicate support of a certain band in supportedBandCombinationList, but omit that band in supportedBandListNR.

In LTE, this is not an issue. In a bandCombination, the bands are indicated by their index (position) in the band list. But in NR, the bands in a band combination are explicitly referenced by the NR frequency band number (FreqBandIndicatorNR).

We note that there are there several fields in BandNR (including its child IE MIMO-ParametersPerBand) that are mandatory for UE to set and support (marked with “Yes” in the M column in TS 38.306, section 4.2.7.2). Hence, we expect that a UE indicates the same bands in supportedBandListNR that it indicates in any band combination.

This leads us to the following:

UE that indicate support for certain band in supportedBandCombinationList (in RF-Parameters or RF-ParametersMRDC) also indicates this band in supportedBandListNR.

# 3	Conclusion

We ask RAN2 to confirm the following:

- UE that indicate support for certain band in supportedBandCombinationList (in RF-Parameters or RF-ParametersMRDC) also indicates this band in supportedBandListNR.

If proposal 1 is agreed, we volunteer to provide the CRs.