3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003322

# Electronic meeting, 20 – 30 April 2020

Agenda item:		6.7.6

Source:	Intel Corporation

Title:	Remaining issues in IIoT UE capability

Document for:		Discussion and Decision

# Introduction

In RAN2#109-e meeting, following was agreed regarding IIoT UE capability:

- UE signals the maximum number of supported EHC contexts as a capability. - A separate UE capability is defined for EHC context continuation. - FFS whether signalling of maximum value of additional SPS periodicities and additional CG periodicities supported by a UE is required. - FFS whether to support allowing CG periodicities of multiple of 2/7 symbols as a separate capability with a cross-slot boundary capability as a pre-requisite. - … - FFS whether LCH based prioritization can be supported without PHY prioritization. It is expected this can be discussed once RAN1 has defined feature/capability related to PHY layer prioritization

In this contribution, we discuss the remaining issues of IIoT UE capability.

# Discussion

## Number of EHC contexts

It is agreed that “UE signals the maximum number of supported EHC contexts as a capability”. The signaled values are not determined yet.

ROHC context capability can be used as a reference. In TS 38.331, the following is defined:

maxNumberROHC-ContextSessions       ENUMERATED {cs2, cs4, cs8, cs12, cs16, cs24, cs32, cs48, cs64,

cs128, cs256, cs512, cs1024, cs16384, spare2, spare1},

As proposed in companion contribution R2-2003321 [1], CID length is 7 and 15 bits. It is proposed that the maximum number of EHC context contains the following values {2, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048, 4096, 8192, 16384, 32768}, with 128 and 32768 corresponding to 7 and 15 bit CID, respectively.

Proposal 1: The signaled number of EHC context capability contains the following values {2, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048, 4096, 8192, 16384, 32768}.

## Periodicities of SPS and CG

One FFS from RAN2#109-e meeting is “whether signalling of maximum value of additional SPS periodicities and additional CG periodicities supported by a UE is required.” In R2-2001290 [2], it was proposed to support the signaling of maximum value of additional SPS and CG periodicities. The arguments are: 1) Testing/verification of all new periodicities can be challenging; 2) While configurations with lower values of new SPS/CG periodicities (e.g., less than 10ms) are known to be useful, practical relevance of vast majority of new CG/SPS periodicities is unknown. We agree with the analysis in R2-2001290 [2], which is also aligned to the analysis in our contribution R2-1914740 [3], which shows the gain of alignment between TSN periodicity and SPS/CG periodicity diminishes when the periodicity increases. Therefore, it is reasonable to support the signaling of maximum value of additional SPS and CG periodicities to minimize testing effort.

Proposal 2: The maximum value of additional SPS periodicities and additional CG periodicities supported by a UE is signaled as capability.

As for the range of the signaling for the maximum value of additional SPS/CG periodicities, it should be noted that current RRC signaling supports the signaling from 1 slot to 640 ms, as below (taking CG as an example):

ConfiguredGrantConfig ::=           SEQUENCE {

...

periodicityExt-r16		INTEGER (1..5120)					OPTIONAL,		-- Need M

...

]]

}

periodicityExt This field is used to calculate the periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5,8.2). If this field is present, the field periodicity is ignored. The following periodicites are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 640. 30 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 1280. 60 kHz with normal CP: periodicityExt*14, where periodicityExt has a value between 1 and 2560. 60 kHz with ECP: periodicityExt*12, where periodicityExt has a value between 1 and 2560. 120 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 5120.

One simple way for the capability signaling is to signal it in unit of ms, e.g. { ms10, ms20, ms40, ms80, ms160, ms320, ms640}. With the capability signaling, UE supports all periodicities which are less than or equal to the signaled value.

Proposal 3: The following is signaled as the maximum value of additional SPS/CG periodicities: { ms10, ms20, ms40, ms80, ms160, ms320, ms640 }.

Another open issue is “whether to support allowing CG periodicities of multiple of 2/7 symbols as a separate capability with a cross-slot boundary capability as a pre-requisite.” Rel-15 already supports 2 and 7 symbol periodicities for CG. Rel-16 further introduces slot granularity periodicity of up to 640 ms for CG. Additional support of CG periodicities of multiple of 2/7 symbols does not bring much gain of alignment between TSN periodicity and SPS/CG periodicity. It is proposed not to consider the additional support of CG periodicities of multiple of 2/7 symbols.

Proposal 4: Support of CG periodicities of multiple of 2/7 symbols is not defined in Rel-16.

# Conclusion

In this contribution, we discuss the remaining issues of IIoT UE capability, and propose the following:

Proposal 1: The signaled number of EHC context capability contains the following values {2, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048, 4096, 8192, 16384, 32768}.

Proposal 2: The maximum value of additional SPS periodicities and additional CG periodicities supported by a UE is signaled as capability.

Proposal 3: The following is signaled as the maximum value of additional SPS/CG periodicities: { ms10, ms20, ms40, ms80, ms160, ms320, ms640 }.

Proposal 4: Support of CG periodicities of multiple of 2/7 symbols is not defined in Rel-16.

# References

[1] R2-2003321, Intel, "Remaining issues in Ethernet header compression"

[2] R2-2001290, Qualcomm, "Open issues in Scheduling Enhancements"

[3] R2-1914740, Intel, "Support of multiple SPS and CG configurations"