3GPP TSG RAN WG2 Meeting #109bis-e           	   		    R2-2003323

Electronic meeting, Apr 20 - 30, 2020                                		 Agenda Item:	6.1.8

Source:	Intel Corporation

Title:	Mandatory/optional features for IAB-MT

Document for:	Discussion and Decision

# Introduction

At RAN#87e, RAN plenary discussed the issue of mandatory features for IAB-MTs based on the views expressed that not all features that are mandatory for UEs need to be mandatory for IAB-MTs. The following were agreed:

- RAN WGs to investigate which of the mandatory Rel-15 UE features (as defined in TR 38.822) can be optional for basic operation of (and if found useful, for different classes of IAB-MTs as defined by RAN4).

- RAN WGs should strive to minimize specification impact.

In this contribution we discuss principles to determine mandatory/optional features for IAB MTs.

# Discussion

TS 38.822 exhaustively lists all UE features for NR and marks each one as mandatory or optional. Mandatory features are further marked as ‘mandatory without capability signalling’ or ‘mandatory with capability signalling’. Given than an IAB node is a network node, the deployment of features can largely be controlled by an operator and designated features as mandatory is less important. The exceptions to this are features that are necessary for establishing connection between the IAB MT and the UPF (including RRC and NAS connectivity). Thus, we propose the following guideline for determining which UE features need to be mandatory for an IAB MT: Only UE features that are essential for establishing a connection between the UE and the UPF need to be mandatory for an IAB MT.

Generally, if a feature is optional, a UE can establish a connection to the network even if it does not support the feature. The same holds for IAB MTs. We have not identified any UE features that are optional that would need to be mandatory for IAB MTs. UE features that are optional can remain optional for IAB MTs.

Proposal 1: Any UE features that are optional should be optional for IAB MTs.

UE features that are ‘mandatory with capability signalling’ are used by a network only after checking the specific capability of the UE. In the context of IAB, given that the deployment of features is network controlled and there is capability signalling involved, such features can be considered optional. Even if UE features that are currently ‘mandatory with capability signalling’ are made mandatory in future, this should not apply to IAB MTs.

Proposal 2: Any UE features that are ‘mandatory with capability signalling’ should be optional for IAB MTs.

In general, features that are ‘mandatory without capability signalling’ correspond to basic functionality that is needed to establish connection. However, even among these feature sets, there can be some differences between UEs and IAB MTs. Therefore, it is necessary to consider such features on a case by case basis.

We show a preliminary analysis below based on the features listed in TS 38.822. We focus on the L2 and L3 features only, and specifically those that are marked ‘mandatory without capability signaling’. Below is a list of features that are ‘mandatory without capability signalling’ that could potentially be considered as optional.

| Index | Feature group | Comments |
| --- | --- | --- |
| 0-0 | Basic EN-DC procedures | EN-DC support is anyway up to operator-vendor negotiation |
| 0-3 | DRB | Mandatory support of DRBs may not be needed; and even if mandatory, 8 DRBs may not need to be mandatory. |
| 0-4 | Direct SN addition in first RRC connection reconfiguration after RRC connection establishment | Does not need to be mandatory, similar to 0-0. |
| 4-1 | Intra-NR measurements and reports | Support of mobility can be left to operator-vendor negotiation (in the simplest scenarios, deployments could consist of IAB nodes that do not perform any handover). |
| 4-2 | Inter-NR measurement and reports while in LTE connected | Not needed for Standalone IAB |
| 5 | SDAP | Given the absence of SDAP at the IAB node and the limited traffic with IAB MT as the destination, SDAP related features should be optional. |
| 7-1 | Handover | Support of mobility can be left to operator-vendor negotiation (in the simplest scenarios, deployments could consist of IAB nodes that do not perform any handover). |

Proposal 3: RAN2 should consider the features and sub-features listed in the table above and discuss whether they could be made optional for IAB MTs.

The mandatory and optional nature of the L1 features and the RF/RRM features for IAB MTs will need to be discussed in RAN1 and RAN4 respectively. RAN2 will need to wait for such information from RAN1 and RAN4.

# Conclusion

We have discussed the issue of mandatory/optional features for IAB MTs. A reasonable principle to determine whether a feature needs to be mandatory or optional can be: Only UE features that are essential for establishing a connection between the UE and the UPF need to be mandatory for an IAB MT. Based on this principle and our analysis we present the following proposals:

Proposal 1: Any UE features that are optional should be optional for IAB MTs.

Proposal 2: Any UE features that are ‘mandatory with capability signalling’ should be optional for IAB MTs.

Proposal 3: RAN2 should consider the features and sub-features listed in the table above and discuss whether they could be made optional for IAB MTs.