3GPP TSG-RAN WG2 Meeting #109bis-e	Tdoc R2-2003353

Electronic meeting, 20th – 30th April	Revision of R2-2000979

Agenda Item:	7.1.6

Source:	Ericsson

Title:	S-Criterion interpretation for non-BL UEs

Document for:	Discussion, Decision

# 1	Introduction

Discussion on non-BL UEs camping in enhanced coverage when the S-Criterion for normal coverage is satisfied has been discussed in several previous RAN2 meetings. This paper is a revision of our earlier input in R2-2000979 on this topic and also accounting for the discussion we had during RAN2#109-e. During the previous meeting, no consensus was reached on any of the proposed alternatives.

# 2	Discussion

In [2], [3] and [4] it is proposed that a non-BL UE should be allowed to camp in enhanced coverage in a cell where S-Criterion for normal coverage is satisfied.

Energy/power consumption

One of the arguments supporting this proposal is a reduced UE energy consumption due to monitoring MPDCCH, on narrower band, instead of PDCCH for paging. In [4] it is claimed that according to the observed behaviour, 85% and 50% of times the network allocates no more than 6 PRBs in UL and DL respectively, a 13% gain is expected in terms of UE power consumption if the proposal is accepted. All the underlying assumptions are not fully explained in the paper, for instance how the power consumption is evaluated in those cases where the network allocates more than 6 PRBs, e.g. what is the relation between bitrate or monitored BW and the power consumption of the modem. Moreover, for paging no more than 6 PRBs would be used in any case, but it is not clear from the paper what are the exact traffic models assumed.

Additionally, it is not explained how much more radio resources the network allocated to the non-BL UE(s) in the evaluated case. It is not likely (and not mentioned in[4]) that the evaluation considered situation with massive deployment of LTE-M UEs. In such deployment the control channel resources (MPDCCH) are critical and if UEs would choose to camp in enhanced coverage at will, severe degradation of the system performance could be seen. If there would be no network control to alleviate the issue and guide capable UEs to camp in normal coverage

Without network control, if UEs choose to camp in enhanced coverage at will, the system performance would degrade earlier compared to UEs camping in normal coverage. This would result possible barring and additional power consumption for UEs.

Moreover, there are further aspects to consider in order to decide whether to let UEs freely select whether they should be camping in normal or enhanced coverage. For example, or what kind of paging strategy the test network assumes. Also, in practice the UE would need to possibly acquire and maintain two version of SIBs, resulting in increased power consumption.

Additional complexity is needed on both UE and network side – UE would need to maintain two different SIBs and network paging strategy would be affected.

Paging from network perspective

If the UE has been in enhanced coverage, the eNB may upload the CE Level for paging in a transparent container to the MME. The MME may then provide such coverage level information with the cell ID when paging request is sent to eNB [1]. Note that this is not mandatory; the eNB may not receive any information regarding UEs possible coverage level prior to paging.

If the network assumes that the UE is static and in enhanced coverage, it would make sense to transmit the paging message assuming the same CE level provided in the container. However, if the UE is not static, the network cannot know or reliably guess the UE CE level, the UE could be in normal coverage as well and there would be no need to use MPDCCH resources for paging. In any case, the eNB decision on to use either PDCCH or MPDCCH, or try both, is up to implementation. Thus, there is no guarantee that the network will page UE over MPDCCH (with enough repetitions that correspond to a particular CE level) even if the UE was released in CE when it was last in connected mode.

There is no guarantee that the eNB will page over PDCCH or MPDCCH or both, regardless of if the UE was in enhanced coverage during last connection.

From network resource efficiency standpoint; it would be best to use the most efficient channel to page a UE, when possible and at least as the starting point.

Another impact on paging efficiency, if the UE is allowed to freely choose whether to camp in normal or enhanced coverage, is that the information optionally provided to MME in the transparent container related to the assumed repetitions of MPDCCH becomes less reliable, and there may be need to change the NW paging strategy. For instance, although the information states that the UE was in normal coverage, the eNB would still need to perform a paging attempt in enhanced coverage in the same cell, before e.g. escalating in normal coverage to other cells.

Paging from UE perspective

An additional complexity for the UE is that the UE would need to maintain both non-BR and BR version of the SIBs in order to switch between normal and enhanced coverage.

For UEs, maintaining two versions of up-to-date SIBs would add complexity and reduce the power efficiency.

In general, if a UE has the possibility to choose between normal or enhanced coverage, it should always choose normal coverage. This results in, less UE power consumption and better utilization of radio resources and is how this was originally intended.

Summary

The S-criterion is used to let the UE select the most efficient coverage mode based also on a minimum channel quality guaranteed to the UE (minimum RSRP and RSRQ configured in SI). The network operator should be able to configure the thresholds for normal coverage and enhanced coverage according to network planning – it should not be only up to UEs whether to camp in normal or enhanced coverage.

Therefore, in absence of results showing a clear gain for either UE or the NW, we propose the following:

Except for standalone LTE-M cell, non-BL UE shall camp in enhanced coverage only if S-criterion for normal coverage is not fulfilled and S-criterion for enhanced coverage is fulfilled.

Note that the intention is that non-BL UE may camp in a standalone cell, i.e. in cell where SIB1 is not transmitted but SIB1-BR can be acquired. This should be already clear however in the current TS 36.304, i.e. only in the case SIB1 could not be acquired (but SIB1-BR can), UEs could camp in enhanced coverage when normal coverage S-criterion is satisfied.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Without network control, if UEs choose to camp in enhanced coverage at will, the system performance would degrade earlier compared to UEs camping in normal coverage. This would result possible barring and additional power consumption for UEs.

Observation 2	Additional complexity is needed on both UE and network side – UE would need to maintain two different SIBs and network paging strategy would be affected.

Observation 3	There is no guarantee that the eNB will page over PDCCH or MPDCCH or both, regardless of if the UE was in enhanced coverage during last connection.

Observation 4	For UEs, maintaining two versions of up-to-date SIBs would add complexity and reduce the power efficiency.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Except for standalone LTE-M cell, non-BL UE shall camp in enhanced coverage only if S-criterion for normal coverage is not fulfilled and S-criterion for enhanced coverage is fulfilled.

# References

TS 36.300

R2-1912867, Non-BL UE in normal and enhanced coverage, Intel Corporation, Chongqing, China, October 2019

R2-1913572, Enhancements to idle mode mobility for non-BL UEs, Huawei, HiSilicon, Chongqing, China, October 2019

R2-1915883, Improvements for non-BL UEs camping in CE Mode, Apple Inc, Reno, Nevada, USA, November 2019