3GPP TSG-RAN WG2 Meeting #109bis-e													R2-2003439

Electronic, 20 April – 30 April 2020

Agenda item:	6.1.8

Source:	CATT

Title:	Views on IAB MT Capability

Document for:	Discussion and Decision

1	Introduction

The following were endorsed in the RP #87e meeting, based on which the topic will be discussed in the next WG meetings.

- RAN WGs to investigate which of the mandatory Rel-15 UE features (as defined in TR 38.822) can be optional for basic operation of (and if found useful, for different classes of IAB-MTs as defined by RAN4). - RAN WGs should strive to minimize specification impact.

In this contribution, we discuss on the IAB MT features and provide our views.

2	Discussions on IAB MT Features

2.1 General Principles

For the sake of efficient discussions, we believe it useful to set up a few general principles to guide the detailed discussions on Rel-15 feature handling. The main ideas are

- RAN2 need input from RAN1/4 on the L1/RF/RRM related Rel-15 UE features.

- For L2/3 features, the focus is to identify a subset of mandatory Rel-15 UE features that can be changed to optional considering the IAB MT use cases.

- To minimize the spec impact, it is preferable to start with existing definitions to see what need to be changed.

Therefore, the suggested baseline is as the following.

Proposal 1	For L2/3 features,

a) Rel-15 features that are mandatory with signalling are changed to optional for IAB MT.

b) Rel-15 optional features remain optional for IAB MT.

c) RAN2 discuss on Rel-15 features that are mandatory without signalling. Baseline is to keep these features mandatory, and potentially a subset of these features can be changed to mandatory with signalling or optional if there is consensus.

For L1/RF/RRM features,

- RAN2 wait for input from RAN1/4.

2.2 Mandatory Rel-15 UE features that can be optional for IAB MT

With Proposal 1, it is further discussed which of Rel-15 features that are mandatory without signalling can be changed.

Table 1 lists based on [1] the features for discussions. The highlight parts are what we suggest candidates for potential relaxations.

Table 1 List and discussion of Rel-15 mandatory features

| Index | Feature group | Discussions |
| --- | --- | --- |
| 0-0 | Basic EN-DC procedures | EN-DC support can be optional for IAB MT based on previous RAN2 discussions. It has been agreed that “F1AP over LTE leg signalling for EN-DC IAB-MT” is optional. |
| 0-3 | DRB | FFS, related to on-going RRC discussions. |
| 0-4 | Direct SN addition in the first RRC connection reconfiguration after RRC connection establishment | Seems no need for change. |
| 1-0 | Basic PDCP procedures | Seems no need for change. |
| 2-0 | Basic RLC procedures | Seems no need for change. |
| 3-0 | Basic MAC procedures | Seems no need for change. |
| 4-1 | Intra-NR measurements and reports | Seems no need for change. This is mandatory without signalling for Rel-15 UE in NR SA. |
| 4-2 | Inter-NR measurement and reports while in LTE connected | Can be changed to optional for IAB MT as the main use case is for inter-RAT operations. |
| 5-1 | (SDAP) QoS | FFS |
| 5-2 | (SDAP) HD format | FFS |
| 7-1 | (Mobility) Handover | Intra-frequency HO is mandatory without capability signalling for Rel-15 UE, and this seems no need for change. |
| 8-1 | System information acquisition | Can be changed to optional for IAB MT, considering it is mainly for Idle/inactive UE procedures. |
| 9-1 | RRC buffer size | Seems no need for change. |
| 9-2 | RRC processing time | Seems no need for change. |

With Table 1 we suggest the following WF.

Proposal 2

- Rel-15 UE features 0-0, 4-2, and 8-1 are changed to optional for IAB MT.

- FFS for 0-3 and 5-x.

3	Summary

Based on the discussions we have the following proposals for IAB MT features.

Proposal 1	For L2/3 features,

a) Rel-15 features that are mandatory with signalling are changed to optional for IAB MT.

b) Rel-15 optional features remain optional for IAB MT.

c) RAN2 discuss on Rel-15 features that are mandatory without signalling. Baseline is to keep these features mandatory, and potentially a subset of these features can be changed to mandatory with signalling or optional if there is consensus.

For L1/RF/RRM features,

- RAN2 wait for input from RAN1/4.

Proposal 2

- Rel-15 UE features 0-0, 4-2, and 8-1 are changed to optional for IAB MT.

- FFS for 0-3 and 5-x.

4	Reference

[1] TR 38.822, vf01