3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003513

Online, 20th – 30th Apr, 2020

Title:	[DRAFT] LS response to SA3 on the security related issues for NR SL

Response to:	S3-200478

Release:	Rel-16

Work Item:	5G_V2X_NRSL

Source:	Huawei, HiSilicon [to be RAN2]

To:	SA3

Cc:	-

Contact Person:

Name: 	Xiao Xiao

E-mail Address:	xiaoxiao6@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks SA3 for their reply on the security related issues for NR SL in S3-200478.

RAN2 has a concern on the 16-bit Key ID used for SLRB in NR SL unicast, as indicated in SA3’s incoming LS. From the AS perspective, such a length of Key ID leads to excessive overhead in the radio interface, and makes the processing of PDCP PDU of DRB in Uu unable to be reused to SLRBs in NR SL. These further lead to the consequence of radio resource inefficiency, and increase UE implementation complexity (e.g. resulting in different security hardware implementation for Uu and SL respectively).

To this end, RAN2 is considering the following alternatives to solve the above issue for NR SL unicast:

- include no Key ID in the PDCP PDU format for the SLRB in unicast (leaving handling of security context confusion in rekeying to UE implementation);

- replace the Key ID with a 1-bit indication in the PDCP PDCP format for the SLRB in unicast (distinguishing old or new key being used).

RAN2 kindly asks SA3 to provide feedback on whether the above alternative(s) are feasible, and their guidance on which one is preferred from the perspective of security design.

2. Actions:

To SA3

ACTION: 	RAN2 respectfully requests SA3 to take the RAN2 concern above into account, and kindly asks SA3 to provide feedback on whether above alternative(s) are feasible as well as their guidance on which one is preferred from the perspective of security design.

3. Date of Next TSG-RAN2 Meetings:

RAN2 Meeting #110 	25 – 29 May 2020	Online

RAN2 Meeting #111	24 – 28 Oct 2020	Toulouse, France