3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003570

# Electronic, 20 April – 30 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | Draft | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Need code for CMAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2020-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed e?planations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For warningAreaCoordinatesSegment-r15 in SIB12, the ASN1 has defined it an optional field with Need OR, however, according to the procedural text for the field, the UE should assemble the geographical area coordinates from the received warningAreaCoordinatesSegment (if any) if all segments of a warning message and geographical area coordinates (if any) have been received. So there is a misalignment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the UE behavior upon warningAreaCoordinatesSegment-r15 in the description of ASN1. Impact analysis: Impacted 5G architectures: EN-DC, (NG)EN-DC Impacted functionality: SIB12 Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. - If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When warning area coordinates are provided for the warning message and the total number of warning area coordinates segments is less than the total number of warning message segments, UE may not assemble the geographical area coordinates. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

6.3.1	System information blocks

<Partially>

–	SystemInformationBlockType12

The IE SystemInformationBlockType12 contains a CMAS notification.

SystemInformationBlockType12 information element

-- ASN1START

SystemInformationBlockType12-r9 ::=	SEQUENCE {

messageIdentifier-r9				BIT STRING (SIZE (16)),

serialNumber-r9						BIT STRING (SIZE (16)),

warningMessageSegmentType-r9		ENUMERATED {notLastSegment, lastSegment},

warningMessageSegmentNumber-r9		INTEGER (0..63),

warningMessageSegment-r9			OCTET STRING,

dataCodingScheme-r9					OCTET STRING (SIZE (1))		OPTIONAL,	-- Cond Segment1

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

...,

[[	warningAreaCoordinatesSegment-r15		OCTET STRING	OPTIONAL	-- Need OR

]]

}

-- ASN1STOP

SystemInformationBlockType12 field descriptions

dataCodingScheme Identifies the alphabet/coding and the language applied variations of a CMAS notification. The octet (which is equivalent to the octet of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.52), contains the octet of the equivalent IE defined in TS 23.041 [37], clause 9.4.3.2.3, and encoded according to TS 23.038 [38].

messageIdentifier Identifies the source and type of CMAS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.44) contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.1, while the trailing bit contains bit 0 of second octet of the same equivalent IE.

serialNumber Identifies variations of a CMAS notification. The leading bit (which is equivalent to the leading bit of the equivalent IE defined in TS 36.413 [39], clause 9.2.1.45), contains bit 7 of the first octet of the equivalent IE, defined in and encoded according to TS 23.041 [37], clause 9.4.3.2.2, while the trailing bit contains bit 0 of second octet of the same equivalent IE.

warningAreaCoordinatesSegment If present, carries a segment, with one or more octets, of the geographical area where the CMAS warning message is valid as defined in [98]. The first octet of the first warningAreaCoordinatesSegment is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 [37] and so on. The field uses actions defined by Need ON.

warningMessageSegment Carries a segment, with one or more octets, of the Warning Message Contents IE defined in TS 36.413 [39]. The first octet of the Warning Message Contents IE is equivalent to the first octet of the CB data IE defined in and encoded according to TS 23.041 [37], clause 9.4.2.2.5, and so on.

warningMessageSegmentNumber Segment number of the CMAS warning message segment contained in the SIB. A segment number of zero corresponds to the first segment, one corresponds to the second segment, and so on. If warning area coordinates are provided for the warning message, then this field applies to both warning message segment and warning area coordinates segment.

warningMessageSegmentType Indicates whether the included CMAS warning message segment is the last segment or not. If warning area coordinates are provided for the warning message, then this field applies to both warning message segment and warning area coordinates segment.

| Conditional presence | Explanation |
| --- | --- |
| Segment1 | The field is mandatory present in the first segment of SIB12, otherwise it is not present. |

CHANGE END