# 3GPP TSG-RAN WG2 Meeting #109	bis					 R2-2003605

E-meeting, April 20 – 30, 2020

Agenda Item:	6.18. 3 (NG_RAN_PRN-Core)

Source: 	LG Electronics

Title:         	Intra-frequency reselection upon selecting non-suitable SNPN cell

Document for: 	Discussion and Decision

Introduction

During the email discussion, it was observed that company views are not converging on the following issue:

Do you agree with the following for licensed spectrum:

For a UE in SNPN AM, if the highest ranked cell or best cell according to absolute priority reselection rules is a cell which is not suitable due to not broadcasting the registered or selected SNPN ID, the UE shall not consider this cell as candidate for cell reselection but should continue to consider other cells on the same frequency for cell reselection.

Discussion

For the case when the highest ranked cell is not suitable due to not belonging to R/S-SNPN, companies expressed different views on intra-frequency reselection behaviours. The following is the list of options being provided:

- Option 1: The UE shall onsider other cells on the same frequency for cell reselection

- Option1a: The UE may continue to consider other cells on the same frequency for cell reselection

- Option 2: The UE shall not consider other cells on the same frequency for cell reselection for a maximum of 300s.

- Option 3: The UE shall not consider other cells on the same frequency for cell reselection for 300s.

- Option 4: Network should be able to control which option between option1 and option2 should be applied.

The option1 and its family (1, 1a, and 4) has a clear demerit of possibly causing interference to a best ranked cell that is not suitable to the UE. However, the proponents for option1 family claim the following merits to go for these options:

- This option family allows UE to select other suitable SNPN cell that may exist in the proximity

- This option family allows UE to select a PLMN cell on the same frequency in RAN sharing case

- This option family allows UE to select other CAG cell on the same frequency

While option1 has some merits, we think that the possible interference issue shall not be under-estimated. For this reason, the option1 should be simply dropped out.

Option2 is to take the same/legacy approach as those for the existing case that the selected cell belongs to a PLMN that is not E/R-PLMN on a licensed band. That is, in option, 2, the UE shall not consider this cell and other cells on the same frequency for a maximum of 300s for cell reselection. Note that option2 allows UE to determine how long it should exclude other cells on the same frequency from reselection candidate, given that only upper bound for reselection restriction should be kept.

With the freedom of UE implementation, option2 also offers the same merits as option1, because the UE is not completely prevented from reselecting other cell on the same frequency. Note that 38.304 specification only restricts upper bound, not lower bound for the intra-frequency reselection restriction. Hence, in the concerned case, the UE may choose to operate as option1, i.e., to reselect other cell, e.g. SNPN cell, on the same frequency by exploiting implementation freedom.

On the other hand, with option2, the UE may choose to operate as option 3, i.e., to exclude other cells on the same frequency from reselection candidates to avoid interference as well as to save UE battery power. This way of implementation is desirable in uncoordinated SNPN-SNPN/PLMN deployments in order to prevent UE from causing severe interference.

Option3 is to take the specific IFRI approach where IFRI is set to false. While option3 may look similar to option2, option3 is different from option2 in that option3 does not allow the freedom of UE implementation regarding the duration of intra-frequency reselection restriction. While taking the legacy approach along with option2 is working well, there is no good reason to take different approach.

Option4 is a kind of flexible solution that the network should be in charge of an optimal control. It is however unclear whether it is proper to give an unsuitable SNPN cell a full control of intra-frequency reselection in uncoordinated SNPN deployments. We think that option4 may lead to misbehaviours in RAN sharing case or simply over-specification.

Based on the analysis, we conclude that option2 is still the best for the case of selecting a unsuitable SNPN due to non-R/S-SNPN. Hence we propose:

Proposal 1 : For a UE in SNPN AM, if the highest ranked cell or best cell according to absolute priority reselection rules is a cell which is not suitable due to not broadcasting the registered or selected SNPN ID, the UE shall not consider this cell and other cells on the same frequency for a maximum of 300s for cell reselection.

Conclusion

This contribution discusses intra-frequency reselection behaviours for the case of selecting a unsuitable SNPN due to non-R/S-SNPN, and proposes the following:

Proposal 1 : For a UE in SNPN AM, if the highest ranked cell or best cell according to absolute priority reselection rules is a cell which is not suitable due to not broadcasting the registered or selected SNPN ID, the UE shall not consider this cell and other cells on the same frequency for a maximum of 300s for cell reselection.