3GPP TSG-RAN WG2 Meeting #109bis electronic		R2-2003610

20 April – 30 April 2020

Agenda item:	5.3.1

Source:		Huawei, HiSilicon

Title:	Further discussion on UL skipping for UCI multiplexing

Document for:	Discussion and Decision

1	Introduction

UCI multiplexing in the presence of UL skipping was discussed in RAN2 in the previous RAN2#108 meeting and a LS was sent to RAN1 to ask for the RAN1 solution to address the issue [1]. In the last RAN1#100 e-meeting, the issue and LS was discussed with a response in [2]. In this contribution, we intend to re-discuss the issue by taking RAN1 considerations into account.

2	Discussion

In Rel-15, RAN2 agreed the uplink skipping function which allows the UE not to generate the MAC PDU when no MAC SDU is available. However, in case no MAC PDU is obtained for UL-SCH, UCI multiplexing on UL-SCH is not clear in RAN1 spec. RAN2 understanding is no MAC PDU shall be generated and delivered to the PHY and no change should be made to the current MAC spec, and the UCI should be transmitted on PUCCH.

However, according to the RAN1 LS reply in [2], it is still problematic when UCI overlapping with PUSCH, i.e. UCI multiplexing on UL-SCH for dynamic UL skipping.

Case 2: dynamic PUSCH skipping with overlapping CSI/HARQ-ACK on PUCCH For case 2, a majority of companies in RAN1 agree with the RAN2 common understanding on the desired UE behavior for UCI handling, as described in the LS R1-2000163. However, no consensus could be made in RAN1#100-e, due to implementation concerns raised from both gNB (increased gNB blind detection) and UE side (change of UCI multiplexing behavior).

Observation: It is problematic for UE to transmit UCI overlapping with PUSCH either on PUCCH or PUSCH in relation with UL skipping.

With the RAN2 understanding, we share the implementation concerns from the uncertainty of UCI on PUCCH or on PUSCH, which depends on if there is a MAC PDU obtained from the MAC and therefore causes the burden for gNB detection. In addition, the timing of such a decision is also unpredictable, which also impact the UE implementation. By taking the RAN1 concerns in to account, we may have the following alternatives to go for a final decision:

- Option 1: Leave RAN1 to further address the issue.

- Option 2: MAC generates a MAC PDU when UCI multiplexing on UL-SCH is needed.

- Option 3: Do nothing. (The consequence is that this UCI may be dropped by the UE)

We understand Option 2 would be infeasible and cause NBC to the MAC spec. Currently UCI multiplexing is invisible to the MAC assembly entity and it may introduce more interaction between MAC and PHY for MAC to determine whether to generate a Padding MAC PDU for the sake of UCI multiplexing. Another side effect is it may lose the advantage of UL skipping in most cases. For Option 1, we think a feasible way is to ignore the presence of UL-SCH in PHY. If a MAC PDU is generated, UE transmits UCI along with the UL-SCH. If no data is received in PHY, UCI is still multiplexed on the PUSCH and filling bits are mapped to the REs for UL-SCH in PHY. The filling bits are decided by UE. In this sense, the UE is able to follow the existing processing of UCI multiplexing regardless of whether a MAC PDU is generated or not and no change is made to MAC spec as well.

Proposal: RAN2 confirms that no MAC PDU is generated for the case of UL skipping in MAC and leave RAN1 to address how to transmit the UCI overlapping with PUSCH.

3	Conclusion

In this contribution, we further discussed the UL skipping for UCI multiplexing by taking RAN1 response into account and have the following proposals:

Observation: It is problematic for UE to transmit UCI overlapping with PUSCH either on PUCCH or PUSCH in relation with UL skipping.

Proposal: RAN2 confirms that no MAC PDU is generated for the case of UL skipping in MAC and leave RAN1 to address how to transmit the UCI overlapping with PUSCH.

Reference

[1] R2-1916572 LS on Uplink Skipping, RAN2

[2] R1-2001376 Reply LS on UL skipping, RAN1.