3GPP TSG-WG2 Meeting #109Bis-e	R2-2003621

# Online, 20th Apr 2020 - 30th Apr 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0787 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cell selection on the dedicated frequency after RRC connection reject for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is an issue in the real network that when eNB rejects the RRC connection due to RAN overload (e.g. lack of resources like C-RNTI), UE will keep trying to establish connection on the same cell after an optional wait time, but still be rejected. These actions may repeat serveral times. To reduce UE’s transmission latency and power consumption, UE can perform a cell selection on the dedicated frequency after RRC connection reject. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add cell selection on the dedicated frequency fro NB-IoT Impact analysis Impacted functionality: RRC Connection establishment Inter-operability: If the UE is implemented according to the CR and the NW is not, there is no inter-operability foreseen. If the NW is implemented according to the CR and the UE is not, there is no inter-operability foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN overload situation is not taken care of, UE would repeatly request the connection on the same cell after connection reject, cause UE’s unnecessary transmission latency and power consumption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.7b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | R2-2003622 36.331 CR 4280 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning of change

### 5.2.7b	Cell Selection on the dedicated frequency after RRC connection reject for NB-IoT

UE shall attempt to camp on a suitable cell according to cellSelectionInfo, if included in the RRCConnectionReject-NB message. If the UE cannot find a suitable cell, the UE is allowed to camp on a suitable cell of any NB-IoT carrier.