3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003641

# Electronic, 20 April – 30 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1470 | rev |  | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Uu and PC5 prioritization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eV2X-Core |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | REL-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In LTE V2V, it was agreed that sidelink communication should be prioritized over an overlapping uplink transmisison if the value of the highest priority of sidelink logical channel is lower than a threhsold, except when the overlapping uplink transmission is related to RA procedure (Msg3). - if there is no uplink transmission; or if the MAC entity is able to perform uplink transmissions and transmissions on SL-SCH simultaneously at the time of the transmission; or if there is a MAC PDU to be transmitted in this TTI in uplink, except a MAC PDU obtained from the Msg3 buffer and transmission of V2X sidelink communication is prioritized over uplink transmission; However, the current specification creates an additional unintended exceptional case, causing the UE not being able to generate sidelink transmission even if the sidelink transmission is prioritized over uplink transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a comma to fix the exceptional case, so that a UE generates a sidelink transmission if it’s prioritized over uplink transmissions aside from Msg3 transmission. Impacted functionality: sidelink and uplink transmisions. Inter-operability: - There are no inter-operablity problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will not generate a sidelink transmission even if the sidelink transmission is prioritized over a overlapping uplink transmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.14.1.2.2	Sidelink process

The Sidelink process is associated with a HARQ buffer.

The sequence of redundancy versions is 0, 2, 3, 1. The variable CURRENT_IRV is an index into the sequence of redundancy versions. This variable is updated modulo 4.

New transmissions and retransmissions either for a given SC period in sidelink communication or in V2X sidelink communication are performed on the resource indicated in the sidelink grant as specified in clause 5.14.1.1 and with the MCS selected as specified in clause 5.14.1.1.

If the sidelink process is configured to perform transmissions of multiple MAC PDUs for V2X sidelink communication the process maintains a counter SL_RESOURCE_RESELECTION_COUNTER. For other configurations of the sidelink process, this counter is not available.

If the Sidelink HARQ Entity requests a new transmission, the Sidelink process shall:

-	set CURRENT_IRV to 0;

-	store the MAC PDU in the associated HARQ buffer;

-	store the sidelink grant received from the Sidelink HARQ Entity;

-	generate a transmission as described below.

If the Sidelink HARQ Entity requests a retransmission, the Sidelink process shall:

-	generate a transmission as described below.

To generate a transmission, the Sidelink process shall:

-	if there is no uplink transmission; or if the MAC entity is able to perform uplink transmissions and transmissions on SL-SCH simultaneously at the time of the transmission; or if there is a MAC PDU to be transmitted in this TTI in uplink, except a MAC PDU obtained from the Msg3 buffer, and transmission of V2X sidelink communication is prioritized over uplink transmission; and

-	if there is no Sidelink Discovery Gap for Transmission or no transmission on PSDCH at the time of the transmission; or, in case of transmissions of V2X sidelink communication, if the MAC entity is able to perform transmissions on SL-SCH and transmissions on PSDCH simultaneously at the time of the transmission:

-	instruct the physical layer to generate a transmission according to the stored sidelink grant with the redundancy version corresponding to the CURRENT_IRV value.

-	increment CURRENT_IRV by 1;

-	if this transmission corresponds to the last transmission of the MAC PDU:

-	decrement SL_RESOURCE_RESELECTION_COUNTER by 1, if available.

The transmission of the MAC PDU for V2X sidelink communication is prioritized over uplink transmissions if the following conditions are met:

-	if the MAC entity is not able to perform all uplink transmissions and all transmissions of V2X sidelink communication simultaneously at the time of the transmission; and

-	if uplink transmission is not prioritized by upper layer according to TS 24.386 [15]; and

-	if the value of the highest priority of the sidelink logical channel(s) in the MAC PDU is lower than thresSL-TxPrioritization if thresSL-TxPrioritization is configured.

End of changes