3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003648

Electronic, 20 April – 30 April 2020                         revision of R2-2001598

Agenda Item:	6.7.3.1

Source: 	ASUSTeK

Title:  	Handling UL grant prioritization with non-overlapping PUSCH duration

Document for:	Discussion and Decision

- Introduction

In RAN2#109-e meeting, prioritization between non-overlapping UL grants that shares the same HARQ ID in a UE is not discussed due to time limitation and related proposal is captured in [1]:

Proposal 5. Discuss the following issue by the email discussion after RAN2#109e: - 5-5. Prioritization of uplink grants with same HARQ process ID should be defined.

In this contribution, we discuss cases with prioritization issues regarding UL grants sharing the same HARQ process ID.

- Discussion

According to current endorsed CR [2], a configured grant or a scheduled grant (including PDCCH grant and RAR grant) is prioritized if there are no other UL grants with overlapping PUSCH duration with priority higher than the UL grant’s priority. The priority is determined by the highest priority among priorities of the logical channels with data available that are multiplexed or can be multiplexed in the MAC PDU. For example, a configured grant may prioritized over an scheduled grant if the data carried via the configured grant is (or can be assembled) from a logical channel with priority higher than those of the scheduled grant. However, current CR has not yet considered the prioritization case between two non-overlapping UL grants sharing the same HARQ process.

A scenario is shown in Figure 1. A UE receives a scheduled UL grant indicating a new transmission. The UE assembles a MAC PDU (PDU A) and performs the new transmission using a HARQ process. The UE starts and restarts ConfiguredGrantTimer of the HARQ process when receiving the UL grant and when transmitting the PDU A. When the ConfiguredGrantTimer is running, the UE has a configured grant (e.g. type-1 configured grant) associated with the HARQ process for UL transmission. According to the current specification, a UE shall not use the configured grant when the ConfiguredGrantTimer is running. However, if the configured grant can carry data from a logical channel with higher priority than data in PDU A, the UE should not skip the configured grant, according to prioritization rule, and should assemble a MAC PDU (e.g. PDU B, which overwrites PDU A) and transmits data with higher priority.

Figure 1. HARQ process collision between two UL grants with different priorities

Observation: 	Prioritization between UL grants is also required when PUSCH durations of UL grants associated with the same HARQ process are not overlapped.

In order to prioritize high priority data and reducing latency, we believe the same concept and method can be applied for both PUSCH duration overlapping and non-overlapping cases. That is, for a new UL grant (either configured grant or scheduled grant), a UE should consider priority of both the new UL grant and the previous UL grant (configured grant or scheduled grant that share the same HARQ process) to determine whether to overwrite the previous UL grant (and overwrite MAC PDU in the HARQ buffer) with the new UL grant.

Proposal: 	The prioritization method used in overlapping PUSCH duration between UL grants shall be applied in non-overlapping scenarios where two UL grants share the same HARQ process.

- Conclusion

We have the following observations and proposals for handling intra-UE prioritization:

Observation: 	Prioritization between UL grants is also required when PUSCH durations of UL grants associated with the same HARQ process are not overlapped.

Proposal: 	The prioritization method used in overlapping PUSCH duration between UL grants shall be applied in non-overlapping scenarios where two UL grants share the same HARQ process.

- Reference

[1] R2-2002083, Summary of Data-Data Prioritization and SR-Data Prioritization

[2] R2-2002341, Introduction of NR IIOT