3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003668

Online, 20 April – 30 April 2020

Agenda item:	6.4.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on PDCP

WI/SID:	5G_V2X_NRSL-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In RAN2, the PDCP re-establishment has been proposed not to include PDCP re-establishment.

One of the main reasons for not having the PDCP reestablishment is, that in Uu it is mainly used for Handover from one cell to another. However, Re-keying also occurs when the SN is incremented to a state where it wraps, and hence needs to be reset.

This paper discusses this issue.

# 2	Discussion

In [1] SA3 defines that the security establishment during re-keying may be performed by any UE at any time, but shall be done before the counter for a PDCP bearer repeats with the current key. Furthemore, it is defined that the userplane for each V2X service is only sent or received with the necessary security.

Observation 1: Re-keying is defined in SA3 specification explicitly for sidelink.

In a NR SL scenario, long lived PDCP connections may occur i.e. in platooning, triggering a wrap-around of the counter for the PDCP bearer, and hereby the necessity of a re-keying command should be obvious. Furthermore, allowing the UE to perform re-keying at any point in time, will increase flexibility for the UE. As the PDCP re-establishment procedure handles the security parameters for the connection, we propose that NR SL should include a PDCP re-establishment procedure.

Proposal 1: NR Sidelink to include PDCP re-establishment procedure

# 3	Conclusion

Based on the discussion in section 2 we propose the following:

Observation 1: Re-keying is defined in SA3 specification explicitly for sidelink.

Proposal 1: NR Sidelink to include PDCP re-establishment procedure

# 4	Reference

| [1] | 3GPP, “S3-200347 Proposed text for security establishment clause of NR PC5 unicast security”. |
| --- | --- |