3GPP TSG-RAN WG2 Meeting #109-bis electronic	R2-2003672

20 – 30 April 2020

Title:	Draft LS response to RAN1 on sidelink HARQ

Response to:	R1-2001426

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN2

To:	RAN1

Cc:

Contact Person:

Name:	Li Zhao

E-mail Address:	zhaoli8@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN2 thanks RAN1 for the LS on sidelink HARQ. RAN2 has discussed the question asked by RAN1, copied below, and made the agreements provided below.

Questions to RAN2: - RAN1 assumes that RAN2 will handle selection of appropriate groupcast HARQ feedback option. From RAN1 perspective, a TX UE can use distance HARQ feedback only when the TX UE’s location is available. - RAN1 assumes that RAN2 will handle the case the RX UE’s location is not available. - RAN1 assumes that RAN2 will handle selection of appropriate groupcast HARQ feedback option. From RAN1 perspective, a TX UE can use GC HARQ feedback Option 2 only when the group size is not greater than the number of candidate PSFCH resources associated with the selected PSSCH resource.

RAN2 agreements to the questions:

RAN2 has discussed the above questions and reached the following agreements:

- In case “the member ID and group size information are provided by upper layers and the group size is greater than the number of candidate PSFCH resources associated with the selected PSSCH resource” is not met and “the TX UE’ location is available”, only CG HARQ feedback option 1 can be applied

- In case “the member ID and group size information are provided by upper layers and the group size is NOT greater than the number of candidate PSFCH resources associated with the selected PSSCH resource” and “the TX UE’ location is NOT available”, only CG HARQ feedback option 2 can be applied.

- In case “the member ID and group size information are provided by upper layers and the group size is NOT greater than the number of candidate PSFCH resources associated with the selected PSSCH resource)” and “the TX UE’ location is available”, It is up to UE implementation to decide which HARQ feedback option is used for the groupcast transmission

Besides the above agreements, RAN2 will further discuss how to deal with the case when “the member ID and group size information are provided by upper layers and the group size is greater than the number of candidate PSFCH resources associated with the selected PSSCH resource” is not met and “the TX UE’ location is NOT available”.

2. Actions:

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to take the above information into account in the Rel-16 5G V2X with NR SL work.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #110	25 – 29 May 2020	Electronic

TSG-RAN WG2 Meeting #111	24 – 28 August 2020	Toulouse, France