3GPP TSG-RAN WG2 Meeting #109bis electronic	R2-2003734

# Online, 20 - 30 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4285 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to inter-RAT SFTD measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed e?planations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to 36.133, inter-RAT SFTD measurement configured on neighbours cells shall be terminated upon NR PSCell is added. TS 36.113 cl. 8.1.2.4.25 … In case an NR PSCell is added, the UE shall terminate the inter-RAT SFTD measurement. …. However, measId of inter-RAT SFTD measurement isn’t autonumously removed after NR PSCell addition. As a result, if an inter-RAT SFTD measurement doesn’t trigger measurement reporting before PSCell addition, the UE has to perform inter-RAT SFTD measurement after entering EN-DC since performing SFTD measurements is not relevent to PSCell addition. It conflicts with 38.133. TS 36.331 cl. 5.5.3.1 4> if the reportSFTD-Meas is set to neighborCells in the associated reportConfig: 5> perform SFTD measurements between the PCell and NR cell(s) on the frequency indicated in the associated measObject;. So it is nessesary to autonumously remove measId of SFTD with reportSFTD-Meas set to neighborCells after NR PSCell addition. |
| Summary of change: |  | Measid of SFTD with reportSFTD-Meas set to neighborCells is autonumously removed after NR PSCell addition. Impact analysis: Impacted 5G architectures: EN-DC, (NG)EN-DC Impacted functionality: RRM measurements Inter-operability: - If the network is implemented according to the CR and the UE is not, UE may perform inter-RAT SFTD measurements on NR neighbour cells after entering EN-DC, which is an unexpected behaviour. - If the UE is implemented according to the CR and the network is not, there is no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will wrongly perform inter-RAT SFTD measurements after entering EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is based on the draft specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

#### 5.5.2.2a	Measurement identity autonomous removal

The UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the associated reportConfig concerns an event involving a serving cell while the concerned serving cell is not configured; or

2>	if the associated reportConfig concerns an event involving a WLAN mobility set while the concerned WLAN mobility set is not configured; or

2>	if the associated reportConfig concerns an event involving a transmission resource pool for V2X sidelink communication while the concerned resource pool is not configured; or

2>	if the associated reportConfig concerns an event involving reportSFTD-Meas set to pSCell while the nr-Config is not configured; or

2>	if the associated reportConfig concerns an event involving reportSFTD-Meas set to neighborCells while the nr-Config is configured:

3>	remove the measId from the measIdList within the VarMeasConfig;

3>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3>	stop the periodical reporting timer if running, and reset the associated information (e.g. timeToTrigger) for this measId;

NOTE 1:	The above UE autonomous removal of measId's applies only for measurement events A1, A2, A6, and also applies for events A3 and A5 if configured for PSCell and W2 and W3 and V1 and V2 and event involving reportSFTD-Meas set to pSCell, if configured.

NOTE 2:	When performed during re-establishment, the UE is only configured with a primary frequency (i.e. the SCell(s) and WLAN mobility set are released, if configured).

<End of modified section 1>