3GPP TSG-RAN WG2#109bis eMeeting	Tdoc  R2-2003820

# Electronic, 20th- 30th April 2020

Agenda Item:		7.0.1

Souce:	Samsung

Title:	General ASN.1 issues for 36.331 Rel-16 (S007)

Document for:		Discussion and decision

# Introduction

The following table lists general protocol issues raised during ASN.1 review for 36.331 Rel-16  as discussed in this contribution.

| RIL# | Description |
| --- | --- |
| S007 | Shouldn’t we use the default approach of non-critical extension for adding transfer of F1AP information to ULInformationTransfer? This merely requires a minor tweak i.e. state that whenever F1AP is included contents of the mandatory dedicatedInfoType is invalid and to be ignred by network |

# Discussion

## S007: Support of delta signalling

The following ASN.1 shows a case for which support of delta signalling was questioned. Rather than repeating similar discussions on a case by case basis, S007 indicates that it seems preferable to agree some general principles and use these by default.

TA-ValidationConfig-r16 ::=		SEQUENCE {

pur-TimeAlignmentTimer-r16		CHOICE {

release							NULL,

setup							ENUMERATED {sXX, sYY, ffs}

}		OPTIONAL,	--Need ON

pur-RSRP-ChangeThreshold-r16	CHOICE {

release							NULL ,

setup							SEQUENCE {

rsrp-IncreaseThresh-r16			RSRP-ChangeThresh-r16,

rsrp-DecreaseThresh-r16			RSRP-ChangeThresh-r16	OPTIONAL		--Need OP

}

}		OPTIONAL		--Need ON

}

RSRP-ChangeThresh-r16 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

mo-Data, rna-Update, mo-VoiceCall, spare1

Some general criteria used in the past included:

We think that in the past RAN2 aimed at somewhat limiting delta signalling as it increases UE complexity (so initially, it was agreed to not have delta signalling at the lowest levels), and to avoid it for small fields given the relatively higher cost of the additional presence bit. Altogehter we propose:

Proposal	Regarding use of delta signalling, RAN2 is requested to confirm the following baseline for R16:

- For fields placed after an extension marker, use delta signalling even if size is just a single bit (as extension marker introduces 3 octet overhead)

- Otherwise, Use delta signalling only for fields exceeding a size of 1 octet

Note that a typical case for a), concerns the use of a BOOLEAN field with need ON

# Conclusion & recommendation

This document discusses some general protocol issues raised during ASN.1 review for 36.331 Rel-16. The document includes the following proposals that RAN2 is requested to discuss and conclude:

Proposal	Regarding use of delta signalling, RAN2 is requested to confirm the following baseline for R16:

- For fields placed after an extension marker, use delta signalling even if size is just a single bit (as extension marker introduces 3 octet overhead)

- Otherwise, Use delta signalling only for fields exceeding a size of 1 octet

# References

[1] TS 36.331 Radio Resource Control