3GPP TSG-RAN WG2 #109-bis-e	Tdoc R2-2003832

Online, April 20th – 30th 2020

Agenda Item:	5.4.3 - UE capabilities and Capability Coordination

Source:		Ericsson, AT&T, T-Mobile, Vodafone, Deutsche Telekom, Telecom Italia S.p.A, NTT DOCOMO INC., Turkcell, BT, China Telecom, Reliance Jio, Telstra, LG Uplus, Orange, Dish Network

Title:	Fallback band combinations

Document for:	Discussion, Decision

# 1	Introduction

RAN2 has been discussing the RAN4 LS on FR2 fallbacks. In this paper we propose a way forward.

# 2	Discussion

## 2.1	What are fallback band combinations?

First, the UE indicates to the network the "capabilities" of the UE. These capabilities indicate which features have been implemented in the UE, which bands the UE supports and which combinations of serving cells the UE supports for carrier aggregation and dual connectivity. The latter is expressed in terms of “band combinations”.

A band combination in the UE capabilities comprises of one or more band parameter entries. For each of those the UE indicates the number of contiguous carriers and their respective bandwidths.

One example: if a UE supports up to 8 contiguous carriers for a certain band, this can be expressed as 8 different band combinations: one with 1 carrier, one with 2 carriers, one with 3 carriers, …, and one with 8 carriers. According to 38.306, those with 1 to 7 contiguous carriers are considered as “fallback band combinations” of the combination with 8 carriers:

Fallback band combination: A band combination that would result from another band combination by releasing at least one SCell or uplink configuration of SCell, or SCG. An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination.

But according to the procedure described in 38.331, section 5.6.1.4, the UE shall omit the entries for 1, 2, 3 ... 7 contiguous carriers from its ASN.1 data structure:

2> if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of "candidate band combinations": 3> remove the band combination from the list of "candidate band combinations";

Based on this, the UE only needs to tell the NW that it supports e.g. 8 contiguous carriers, and the other band combinations that would result from those 8 contiguous carriers (band combinations with 7, 6 , ..., 1 carriers) are all implicit and never signalled to the network. These implicit entries are what RAN2 call "fallback band combinations". Note that the UE has actually no means to tell the NW that it does not support those fallback band combinations.

The UE shall omit fallback band combinations from the supportedBandCombinationList in ASN.1 but it shall support those combinations anyway.

Admittedly, the word "fallback" is confusing as it may seem to imply that a UE was first configured to use 8 carriers, but later the network de-configured a carrier for the UE. However, this is not the case. In retrospective, RAN2 should have called the "fallback band combinations" something like "implicit band combinations" since they are not explicitly signalled but instead implicitly known based on a band combination which the UE actually signals. In the above, the 7-, 6-, 5-, …, 2-carrier band combinations are never sent but instead implicitly supported by the UE since the UE explicitly indicated an 8-carrier band combination.

“Fallback band combination” indicates how the network may configure the UE - irrespective of any preceding configuration.

## 2.2	What does the RAN4 suggestion imply?

It has been argued that it is cumbersome to do testing of all fallback band combinations. To reduce the testing effort, it has been proposed that the UE should not be required to support all fallbacks. There is a proposal in RAN2 for how to implement the change the RAN4 LS suggests. With this proposal, a UE which indicates in its ASN.1 capability a band combination with 8 carriers, would be known to support only 8 carriers or 1 carrier. The network would not be allowed to assume that the UE supports a configuration with 7, 6, 5, ..., 2 carriers.

Of course, UE vendors could, if they want, explicitly signal a band combination entry which indicates that the UE supports also e.g. 4 carriers. The CRs that were discussed in RAN2 would also allow such a UE to indicate in two different band combination entries that it supports up to 4 carriers (i.e., also 3, 2 or 1) as well as 8 carriers (but not 7, 6, 5). Which band combinations the UE supports with and without implicit fallbacks would, according to the CRs, be entirely up to the UE vendor. This is what can be called "cherry-picking" of band combinations.

## 2.3	Issues with the RAN4 suggest

Specification-wise the RAN4 suggestion is not complicated to implement. But it has three main drawbacks:

- Computational complexity

- Capability signalling overheard

- Risk of market fragmentation

Computational complexity: When a UE connects, the gNB tries to find the optimum configuration for the UE. What is the optimum may depend on the carriers the gNB has deployed, the band combinations the UE supports, the current load in the cells, measurements, etc. etc. etc. The search for the configuration to use for the UE must be completed within a couple of milliseconds because any delay in the search would result in a delay for starting to serve data traffic to the UE.

Currently, with the implicit fallbacks, as in the specifications, the gNB knows that a UE which doesn’t support N carriers, also does not support N+1 carriers meaning that the search for the optimum configuration can be cut short by not having to check a configuration for N+1 carriers if the UE does not support N carriers.

Without the implicit fallbacks, the network would have to perform more search steps since the UE may support 8 carriers even if it does not support 5, 6 or 7 carriers. As a result, the search for the best configuration may either take much longer and thereby increase the control plane latency. Or the network may terminate the search resulting in a non-best configuration.

The RAN4 suggestion increases computational complexity and may result in that the best configuration cannot be used.

Capability signalling overhead: With the proposal the UE would explicitly signal band combinations which, based on today's signalling, are implicit/not signalled. The signalling overhead increases due to this. And due to the increased number of explicitly signalled band combinations, the time needed to validate possible configurations (see above) will also take longer. The RACS WI introduced UE capability segmentation, which would allow a larger maximum size of the UE capability signalling, but with the RAN4 proposal we would quicker approach the maximum, again.

The RAN4 suggestion increases capability signalling overhead.

Market fragmentation: if a UE starts cherry-picking of the band combinations in their implementation/testing, without supporting all fallbacks (i.e. combinations with fewer carriers) it means that a UE may support only 8, 4 or 1 carriers. Which means that the network can only configure exactly 8, exactly 4 or exactly 1 carrier for this UE. It will be impossible to ever configure such a UE with 7, 6, 5, 3, or 2 carriers. If the UE would enter CONNECTED mode in a gNB, or gets handed over to a gNB, which has deployed 7 carriers, the gNB could only configure 4 carriers for this UE. The UE would reject any configuration message which attempts to configure anything else than 8, 4 or 1 carrier.

So, while the UE may have been well suited to operate in one operator's network (or one part within one operator's network) another operator may not be able to use the full potential of the UE in its network. Or if later, or in another part of the network, fewer carriers are used, or if for some reason the operator cannot configure the 8th carrier for a UE, the operator would end up only be able to serve the UE with 4 carriers.

The RAN4 suggestion has a risk of creating market fragmentation.

## 2.4	Other aspects

Restructuring of n260: The band which triggered the discussion is n260. This band is currently very fragmented. However, work is ongoing to restructure this band after which operators with spectrum in this band will all have contiguous spectrum. This will simplify both implementations, and testing hence the motivation for the RAN4 suggestion is no longer very clear. More info can be found in  https://docs.fcc.gov/public/attachments/DA-20-253A1.pdf.

Work is ongoing to restructure the spectrum in band n260 and with it, the motivation for the RAN4 suggestion is no longer very clear.

Testing: The argument for the suggested RAN4 change is that current specifications requires a lot of testing. It seems though that the amount of testing is not as high as has been suggested. This issue regarding testing has also been brought up in RAN2 and companies have different understandings of the amount of testing which really is needed.

There is no common understanding among companies how much testing is actually needed. Companies need to get aligned (in RAN4 mainly) to determine if testing really is an issue.

RAN4 workload: It has been argued that a motivation for removing the implicit fallbacks is to reduce RAN4 workload since it is claimed that RAN4 cannot complete their specifications w.r.t. the fallbacks. However, this is a RAN4 issue and there are means for RAN4 to reduce their load. For example, the large tables in section 5.5A.2 in 38.101-2 can be removed since all information is provided explicitly in ASN.1-capability signalling. This is also a RAN4 issue and needs to be discussed there.

It would be possible to reduce RAN4 workload w.r.t. defining band combinations by removing the large tables in section 5.5A.2 in 38.101-2. RAN4 can explore these possibilities.

## 2.5	Suggested Way forward

3GPP should make well-informed decisions. There were some RAN2 aspects which RAN4 didn’t consider (e.g. computational complexity, capability signalling overhead, and market fragmentation). Therefore, we think that RAN4 need to discuss if/how their specifications can be simplified to address the "workload" issue. RAN4 should discuss and agree on what really needs to be tested. And only if the benefits of removing implicit fallbacks are justified even in light of the RAN2 aspects (computational complexity, capability signalling overhead, and market fragmentation) RAN2 should revisit this topic.

RAN2 should allow RAN4 to discuss the topic of FR2 fallback band combinations one more round in order for 3GPP to make a well-informed decision.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 should allow RAN4 to discuss the topic of FR2 fallback band combinations one more round in order for 3GPP to make a well-informed decision.

# Annex

It has been mentioned several times how RAN4 has discussed all possible aspects of the FR2-fallback skipping topic. And how the RAN4 CRs has been "agreed". However, looking at the history of how the change in the CR ended up in the specification, we get a different picture which suggests that the discussion in RAN4 was not considering all the aspects of this topic.

Below are three excepts:

- MoM from the RAN4 meeting where the CRs where "agreed"

- Cover page of the RAN4 CR.

- MoM from the plenary which approved the CRs

From 1 it can be seen that the CRs were never "agreed", only "endorsed". Usually ways of working are that endorsed CRs are not implemented in specifications unless explicitly agreed. And if explicitly agreed to be approved, the endorsed CRs should have be converted in to real CRs.

From 2 we see the endorsed CR was a draft CR. "Draft" CRs should not be implemented in the specifications. If the endorsed draft CR would be properly agreed, then of course the "draft" should be removed from the title when converted to a real CR.

From 3 we can see that MCC believed that the endorsed draft CR was "agreed". Hence wrongfully implemented it in the RAN4 specification.

## RAN4 #92 RF MoM:

Below is the MoM from the RAN4 meeting which "agreed" the CRs. As can be seen. RAN4 actually only "endorsed" the CRs.

R4-1910238 CR for Handling of fallbacks for combined contiguous and non-contiguous CA in FR2 38.101-2 CR- rev Cat: F (Rel-15) v15.6.0 Source: Apple Abstract: Mandating all possible fallbacks for mixed contiguous and non-contiguous intra-band CA in FR2 results in an exessive number of fallback combinations to be specified in 3GPP and supported in the UE Discussion: Decision: The document was endorsed.

## CR cover page

Below is from the CR-cover page of R4-1910238. As can be seen it was a "draft" CR.

| Title: | Draft CR for handling of fallbacks for combined contiguous and non-contiguous CA in FR2 |
| --- | --- |

## RAN#85 MoM

Below is an excerpt from the MoM of the plenary where the endorsed draft CRs where implemented.

RP-192049 RAN4 CRs to Core Part of NR Access Technology, part 4 RAN4 Replaces MCC: includes so called "big CRs" which are merging several draft CRs for NR_newRAT-Core agreed in RAN4 The document was approved.