3GPP TSG-RAN WG2 Meeting #109bis	R2-R2-2003902

Elbonia, 20 April – 30 April 2020

Agenda Item: 	6.16.3

Source:	Apple

Title:	Summary of offline#103: BFR on SpCell (2nd round)

Document for:		Discussion and Decision

# Introduction

Following agreements were reached during online discussion in RAN2#109bis-e meeting.

Agreements:

- SpCell BFR enhancement is supported in R16.

- BFR MAC CE for SCell BFR is used for SpCell BFR reporting (i.e. no new BFR MAC CE is introduced).

- A single octet bitmap should be used if SpCell beam failure is detected and truncated BFR MAC CE cannot be accommodated in available UL grant.

Working assumptions:

- BFR MAC CE for SpCell is only transmitted in Msg3 and MsgA via CBRA. Further discuss offline

- AC and candidate beam ID is not contained in the BFR MAC CE for SpCell. Further discuss offline

This is the summary report of offline#103 BFR on SpCell - second round, which is focusing on the two working assumptions (marked in yellow).

# Discussion

Companies are encouraged to provide your view on the working assumption.

- Working assumption#1

- BFR MAC CE for SpCell is only transmitted in Msg3 and MsgA via CBRA. Further discuss offline

Question 1: Do you have any technical concerns on sending SpCell BFR in any dedicated UL grant on any activated serving cell(s)?

| Company | Reply? (y/n) | Additional comments |
| --- | --- | --- |
| vivo | Maybe y | Issue 1: The UE may still need to continue RACH after the transmission of the SpCell BFR MAC CE. For example, if the UE is configured with very short periodicity CG on a SCell, the SpCell BFR MAC CE is not able to be transmitted in Msg3 in most cases. However the current specification still requires the UE to trigger RACH for SpCell BFR. We are not sure whether we should spend extra time on discussing the RACH cancellation. Issue 2: The transmission of the SpCell BFR MAC CE should be more reliable than the transmission of the SCell BFR MAC CE. For example, if the transmission of a SCell grant including the SpCell BFR MAC CE fails due to no power ramping, the gNB is still not able to know whether the RACH is triggered by BFR or BSR. Then the gNB may not change the TCI state of the SpCell to recover the failed beam. Unlike the SCell BFR, the bad radio condition of the SpCell may finally cause RLF. |
| Samsung | Yes | The main motivation for SpCell BFR enhancement (as concluded in earlier email discussion) is that NW is not aware of SpCell BFR info if the CBRA is used for SpCell BFR; No explicit BFR indication is carried in Msg3 transmission, therefore NW cannot distinguish whether RACH is triggered by BFR or by other events (e.g. uplink data arrival). This can be resolved only if BFR MAC CE for SpCell BFR is transmitted in Msg3. |
| OPPO | Yes | Since the purpose is to enable the network to be aware of the BFR for contention based RACH, we agree this principle that BFR MAC CE is only transmitted for BFR CBRA. There is no issue if the UL BWP is only configured with contention based RACH resources, in this case, UE MAC would instruct LCP to multiplex the BFR MAC CE into the MAC PDU for msg3 transmission. However, we see some issues for the case when both CFRA resources and CBRA resources are configured. The issue is that, since as the working assumption saying, we limit the BFR MAC CE to be tansmiited on CBRA instead of CFRA, then if UE switches from CBRA to CFRA, it needs to rebuild the MAC PDU so that the BFR MAC CE is taken out from the already buffered MAC PDU, so don’t need such behavior in the UE side. Thus, one simple way is to just limit the feature to the case when UL BWP is only configured with contention based RACH resouces. [Apple] For the case of CBRA switching to CFRA, the UE behavior is same as today and not required to rebuild the MAC PDU. The handling of the BFR MAC CE is same as C-RNTI MAC CE handling. |
| CATT | Yes | Seems the issue raised OPPO can be discussed and clarified. |
| Qualcomm | No | What we prefer is to give UE more flexibility in how to signal and recover from beam failure on SpCell, e.g. UE can choose either perform RACH or send BFR MAC CE over a UL grant on a serving cell, whichever option that can help UE recover faster based on UE’s situation when BFR is triggered. We understand that companies may have concern on the reliability of sending SpCell BFR MAC CE over a UL grant not in msg3 or msgA. But we believe UE has enough information and method to evaluate reliability of such a transmission and handle unexpected failure if it does happen, e.g. - If a UL grant is scheduled on a serving cell which is not QCL with SpCell and whose BFD-RS’ have strong signal strength, there is a good chance this PUSCH can go through and hence it is faster to send BFR MAC CE over this UL grant instead of triggering a RACH. Otherwise, UE performs RACH based BFR as in legacy. If the PUSCH Tx fails to go through, BFR will be triggered again. UE then performs RACH based BFR as in legacy. - Or, to really guarantee reliability, UE can perform both, i.e. it performs RACH based BFR as well as sends SpCell BFR MAC CE in PUSCH. Note that since it is up to UE implementation, it is perfectly fine too if UE performs only RACH based BFR as in legacy. On the network side, since gNB has to handle SCell BFR MAC CE in any UL grant from any serving cell, we do not expect what we propose would add extra complexity to gNB implementation. |
| Sharp | Yes | Based on the intention of introduction of Spcell BFR MAC CE in RA, we prefer to apply the Spcell BFR MAC CE only in CBRA procedure at least in R16. |
| ZTE | Yes | Regarding the main intention for introducing the SpCell BFR is due to the CBRA can not indicate the case of triggering RACH , and then it is natural to make a decision that l BFR MAC CE only can be triggered by CBRA. Assuming that if UE can send this MAC CE on any serving cells, there are so many issues would be raised from this assumption, for example: how to trigger, maintain, terminate this MAC CE? In our understanding, it is so time-consuming discussion if we go for this assumption. And if we just reuse mechanism of the current BFR MAC CE for SCell, which means there are two independent BFR procedures for SpCell BFR, and this issue can not be resolved by RAN2 itself, which is not our intention. |
| Nokia, Nokia Shanghai Bell | Yes | Since this was already concluded in the post-meeting email discussion, we prefer to stick to the principle. |
| LG | Yes | Considering that SpCell BFR MAC CE is introduced to notify the network that CBRA is triggered due to SpCell BFR, if SpCell BFR MAC CE can be transmitted on any dedicated UL grant, in some cases, the network may not differentiate whether CBRA is triggered by SpCell BFR or by BSR. |
| MediaTek | Yes | For two reasons, we prefer to restrict the transmission of BFR MAC CE for SpCell only on Msg3. First, the main intention is to indicate to NW whether the CBRA is triggered by SpCell BFR. Without the restriction, the MAC CE may not be included in Msg3. And second, although “no restriction” may perform quick BFR MAC CE transmission, we have similar concern on reliability as raised by vivo. |
| Futurewei | Yes | At this late stage of R16, it’d be better to focus on the main intention of having BFR MAC CE for SpCell, which is to indicate that the CBRA is triggered by BFR. |
| Huawei | Yes | As mentioned by the above companies, if MAC CE can be transmitted on Scells, the issue will be further complicated. - If SpCell has already initiated RACH and - If transmitted on the other cells, the exact motivation for designing this does not hold |

Question 2: Do you agree the 1st working assumption?

| Company | Agree? (y/n) | Additional comments |
| --- | --- | --- |
| vivo | Y |  |
| Samsung | Y |  |
| OPPO | Y but | We can limit the feature to the case when UL BWP is only configured with CBRA resources. [Apple] For the case of CBRA switching to CFRA, the UE behavior is same as today and not required to rebuild the MAC PDU. The handling of the BFR MAC CE is same as C-RNTI MAC CE handling. Therefore, we donot need to have the limitation. |
| CATT | Yes |  |
| Qualcomm | No | We think it is unnecessarily restrictive, for the reasons given in our comment to Q1. |
| Sharp | Yes |  |
| ZTE | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes |  |
| LG | Y |  |
| MediaTek | Y |  |
| Ericsson | Y |  |
| Futurewei | Y |  |
| Apple | Y |  |
| Huawei | Y |  |

- Working assumption#2

- AC and candidate beam ID is not contained in the BFR MAC CE for SpCell. Further discuss offline

Question 3: Do you agree the 2st working assumption?

| Company | Agree? (y/n) | Additional comments |
| --- | --- | --- |
| vivo | Y | If companies want to introduce the candidate beam ID in the SpCell BFR MAC CE, we probably need to ask RAN1 for further information. |
| Samsung | Y |  |
| OPPO | Y but some clarifications are needed | We are wondering whether the BFR MAC CE can include the beam failure information for other SCells, i.e., can the fields corresponding to SCells in the octet be set to 1 if those SCells have beam failure? [Apple] If the UL grant of Msg3/MsgA is large enough, and SCell BFR is triggered, I donot see the problem to include both SCell and SpCell info in the same BFR MAC CE. In such MAC CE, the AC and candidate beam ID is not contained for SpCell, but can be contained for SCell. |
| CATT | Y |  |
| Qualcomm | No | Even in the case where SpCell BFR MAC CE is sent in msg3 or msgA, we see benefits for including candidate beam ID in the MAC CE, because that would enable RACH occasion to be decoupled from candidate beam. The motivation behind that is periodicity of NR’s PRACH configuration can be quite long (over 100msec in some cases). If UE can signal candidate beam in msg3/msgA, then it does not have to wait for the Ros associated with its candidate beam, i.e. it can just use a suitable beam which is available sooner to perform RACH. In our view, latency is an important performance metric for BFR. |
| Sharp | Yes |  |
| ZTE | Yes | It is for accommodating the MAC CE into the small container along with the CRNTI-MAC CE. @OPPO, I think the filed corresponding to SCells in the octet can be set to 1 if failure is occurred since it is beneficial for NW to derive the additional beam information from UE. |
| Nokia, Nokia Shanghai Bell | Yes | As discussed in the post-meeting email discussion, the candidate beam information in the BFR MAC CE may not correspond to the one indicated with the RA preamble. From NW point of view, the beam indicated by the preamble would anyway be used. |
| LG | Y |  |
| MediaTek | Y |  |
| Ericsson | Y |  |
| Futurewei | Y |  |
| Apple | Y |  |
| HW | Y |  |

# Summary

During offline discussion, 13/14 companies support the two working assumptions.

Given the big majority in supporting the working assumptions, Rapporteur propose to agree the two working assumptions.

Proposal 1: BFR MAC CE for SpCell is only transmitted in Msg3 and MsgA via CBRA.

Proposal 2: AC and candidate beam ID is not contained in the BFR MAC CE for SpCell.

The corresponding TP is provided in section 4, which was worked out during the Post109e#17 email discussion.

Proposal 3: Capture the TP in section 4 in the EMIMO MAC CR.

# Text proposal

### 5.1.3a	MSGA transmission

The MAC entity shall, for each MSGA:

1>	if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and

1>	if the notification of suspending power ramping counter has not been received from lower layers; and

1>	if LBT failure indication was not received from lower layers for the last MSGA Random Access Preamble transmission; and

1>	if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	select the value of DELTA_PREAMBLE according to clause 7.3;

1>	set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP;

1>	if this is the first MSGA transmission within this Random Access procedure:

2>	if the transmission is not being made for the CCCH logical channel:

3>	indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission.

2> if the Random Access procedure was initiated for beam failure recovery:

3> indicate to the Multiplexing and assembly entity to include a BFR MAC CE or a Truncated BFR MAC CE in the subsequent uplink transmission.

2>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the MSGA buffer.

1>	compute the MSGB-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted;

1>	instruct the physical layer to transmit the MSGA using the selected PRACH occasion and the associated PUSCH resource, using the corresponding RA-RNTI, MSGB-RNTI, PREAMBLE_INDEX, PREAMBLE_RECEIVED_TARGET_POWER, preambleReceivedTargetPower, and the amount of power ramping applied to the latest MSGA preamble transmission (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP);

1>	if LBT failure indication is received from lower layers for the transmission of this MSGA Random Access Preamble:

2>	instruct the physical layer to cancel the transmission of the MSGA payload on the associated PUSCH resource;

2>	perform the Random Access Resource selection procedure for 2-step RA type (see clause 5.1.2a).

NOTE:	The MSGA transmission includes the transmission of the PRACH Preamble as well as the contents of the MSGA buffer in the PUSCH resource corresponding to the selected PRACH occasion and PREAMBLE_INDEX (see TS 38.213 [6])

The MSGB-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted, is computed as:

MSGB-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 2

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8], f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier). The RA-RNTI is calculated as specified in clause 5.1.3.

### 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission;

2>	monitor for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while ra-ResponseWindow is running.

1>	else:

2>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission;

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission on the search space indicated by recoverySearchSpaceId is received from lower layers on the Serving Cell where the preamble was transmitted; and

1>	if PDCCH transmission is addressed to the C-RNTI; and

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	consider the Random Access procedure successfully completed.

1>	else if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded:

2>	if the Random Access Response contains a MAC subPDU with Backoff Indicator:

3>	set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI.

2>	else:

3>	set the PREAMBLE_BACKOFF to 0 ms.

2>	if the Random Access Response contains a MAC subPDU with Random Access Preamble identifier corresponding to the transmitted PREAMBLE_INDEX (see clause 5.1.3):

3>	consider this Random Access Response reception successful.

2>	if the Random Access Response reception is considered successful:

3>	if the Random Access Response includes a MAC subPDU with RAPID only:

4>	consider this Random Access procedure successfully completed;

4>	indicate the reception of an acknowledgement for SI request to upper layers.

3>	else:

4>	apply the following actions for the Serving Cell where the Random Access Preamble was transmitted:

5>	process the received Timing Advance Command (see clause 5.2);

5>	indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP);

5>	if the Random Access procedure for an SCell is performed on uplink carrier where pusch-Config is not configured:

6>	ignore the received UL grant.

5>	else:

6>	process the received UL grant value and indicate it to the lower layers.

4>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

5>	consider the Random Access procedure successfully completed.

4>	else:

5>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

5>	if this is the first successfully received Random Access Response within this Random Access procedure:

6>	if the transmission is not being made for the CCCH logical channel:

7>	indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission.

6> if the Random Access procedure was initiated for beam failure recovery:

7> indicate to the Multiplexing and assembly entity to include a BFR MAC CE or a Truncated BFR MAC CE in the subsequent uplink transmission.

6>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3 buffer.

NOTE:	If within a Random Access procedure, an uplink grant provided in the Random Access Response for the same group of contention-based Random Access Preambles has a different size than the first uplink grant allocated during that Random Access procedure, the UE behavior is not defined.

1>	if ra-ResponseWindow configured in BeamFailureRecoveryConfig expires and if a PDCCH transmission on the search space indicated by recoverySearchSpaceId addressed to the C-RNTI has not been received on the Serving Cell where the preamble was transmitted; or

1>	if ra-ResponseWindow configured in RACH-ConfigCommon expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received:

2>	consider the Random Access Response reception not successful;

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

2>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

3>	if the Random Access Preamble is transmitted on the SpCell:

4>	indicate a Random Access problem to upper layers;

4>	if this Random Access procedure was triggered for SI request:

5>	consider the Random Access procedure unsuccessfully completed.

3>	else if the Random Access Preamble is transmitted on an SCell:

4>	consider the Random Access procedure unsuccessfully completed.

2>	if the Random Access procedure is not completed:

3>	select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF;

3>	if the criteria (as defined in clause 5.1.2) to select contention-free Random Access Resources is met during the backoff time:

4>	perform the Random Access Resource selection procedure (see clause 5.1.2);

3>	else if the Random Access procedure for an SCell is performed on uplink carrier where pusch-Config is not configured:

4>	delay the subsequent Random Access transmission until the Random Access Procedure is triggered by a PDCCH order with the same ra-PreambleIndex, ra-ssb-OccasionMaskIndex and UL/SUL indicator TS 38.212 [9].

3>	else:

4>	perform the Random Access Resource selection procedure (see clause 5.1.2) after the backoff time.

The MAC entity may stop ra-ResponseWindow (and hence monitoring for Random Access Response(s)) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX.

HARQ operation is not applicable to the Random Access Response reception.

### 6.1.3.23	BFR MAC CEs

The BFR MAC CEs consists of either:

-	BFR MAC CE; or

-	Truncated BFR MAC CE.

The BFR MAC CEs are identified by a MAC subheader with LCID as specified in Table 6.2.1-2.

BFR MAC CE has a variable size. It includes a bitmap and in ascending order based on the ServCellIndex, beam failure recovery information i.e. octets containing candidate beam availability indication (AC) for SCells indicated in the bitmap. For BFR MAC CE, a single octet bitmap is used when the highest ServCellIndex of this MAC entity's SCell configured with beam failure detection is less than 8, otherwise four octets are used.

For Truncated BFR MAC CE, a single octet bitmap is used for the following cases, otherwise four octets are used:

-    the highest ServCellIndex of this MAC entity's SCell configured with beam failure detection is less than 8; or

-    beam failure is detected for SpCell (as specified in Clause 5.17) and the SpCell is to be indicated in a Truncated BFR MAC CE and the UL-SCH resources available for transmission cannot accommodate the Truncated BFR MAC CE with the four octets bitmap plus its subheader as a result of LCP.

The fields in the BFR MAC CEs are defined as follows:

-     SP: This field indicates beam failure detection (as specified in clause 5.17) for the SpCell of this MAC entity. The SP field is set to 1 to indicate that beam failure is detected for SpCell only when BFR MAC CE or Truncated BFR MAC CE is to be included into a MAC PDU as part of Random Access Procedure (as specified in 5.1.3a and 5.1.4), otherwise, it is set to 0.

-	Ci (BFR MAC CE): This field indicates beam failure detection (as specified in clause 5.17) and the presence of an octet containing the AC field for the SCell with ServCellIndex i as specified in TS 38.331 [5]. If the Ci field set to 1, beam failure is detected and the octet containing the AC field is present for the SCell with ServCellIndex i. If the Ci field set to 0, the beam failure is not detected and octet containing the AC field is not present for the SCell with ServCellIndex i. The octets containing the AC field are present in ascending order based on the ServCellIndex;

-	Ci (Truncated BFR MAC CE): This field indicates beam failure detection (as specified in clause 5.17) for the SCell with ServCellIndex i as specified in TS 38.331 [5]. If the Ci field set to 1, beam failure is detected and the octet containing the AC field for the SCell with ServCellIndex i may be present. If the Ci field set to 0, the beam failure is not detected and the octet containing the AC field is not present for the SCell with ServCellIndex i. The octets containing the AC field, if present, are incuded in ascending order based on the ServCellIndex. The number of octets containing the AC field included is maximised, while not exceeding the available grant size;

NOTE:	The number of the octets containing the AC field in the Truncated BFR format can be zero.

-	AC: This field indicates the presence of the Candidate RS ID field in this octet. If at least one of the SSBs with SS-RSRP above rsrp-ThresholdBFR amongst the SSBs in candidateBeamRSSCellList or the CSI-RSs with CSI-RSRP above rsrp-ThresholdBFR amongst the CSI-RSs in candidateBeamRSSCellList is available, the AC field is set to 1; otherwise, it is set to 0. If the AC field set to 1, the Candidate RS ID field is present. If the AC field set to 0, R bits are present instead;

-	Candidate RS ID: This field is set to the index of an SSB with SS-RSRP above rsrp-ThresholdBFR amongst the SSBs in candidateBeamRSSCellList or to the index of a CSI-RS with CSI-RSRP above rsrp-ThresholdBFR amongst the CSI-RSs in candidateBeamRSSCellList. The length of this field is 6 bits.

-	R: Reserved bit, set to 0.

Figure 6.1.3.23-1: BFR and Truncated BFR MAC CE with single octet bitmap

Figure 6.1.3.23-2: BFR and Truncated BFR MAC CE with four octets bitmap