3GPP TSG-RAN WG2 Meeting #109-bis electronic	R2-2004074

20 – 30 April 2020

Title:	Draft LS to RAN1 to check the view on sidelink

Response to:	-

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN2

To:	RAN1

Cc:

Contact Person:

Name:	Li Zhao

E-mail Address:	zhaoli8@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

- RAN2 has reached the agreement that

- “Keep the parameters sl-NrOfHARQ-Processes-r16 and sl-HARQ-ProcID-offset-r16 in TS 38.331. Remove directly the related Editor’s Note in SL-ConfiguredGrantConfig. How the two parameters are used is further discussed in MAC”.

RAN2 would like to enquire RAN1 whether the IIoT equation for HARQ process ID calculation, in the case of multiple configured grants, can be used for NR SL:

HARQ Process ID = [floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

- Regarding the parameters sl-MinMCS-PSSCH and sl-MaxMCS-PSSCH included in sl-ScheduledConfig in TS 38.331, V16.0.0 (corresponding to minMcs-Mode1 and maxMcs-Mode1 in R1-2001478), RAN2 made the working assumption that

- “only one MCS range is configured applying to both dynamic grant and configured grant type 1/2; no configured grant type 1/2 specific MCS range is further needed”.

- Regarding the parameter sl-PSFCH-RB-Set in TS 38.331, V16.0,0 (corresponding to rbSetPSFCH in R1-2001478), RAN2 agreed that

- “The leftmost bit indicated by the bitmap refers to the lowest RB index in the resource pool”.

2. Actions:

To RAN1

- RAN2 respectfully asks RAN1 to indicate whether the IIoT equation in point 1 above can be used for NR sidelink.

- RAN2 respectfully asks RAN1 to confirm whether there is any concern on the working assumption made in point 2.

3)	RAN2 respectfully asks RAN1 to take point 3 into account and provide feedback, if any.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #110	25 – 29 May 2020	Electronic

TSG-RAN WG2 Meeting #111	24 – 28 August 2020	Toulouse, France