3GPP TSG RAN WG2 Meeting #109bis-e		R2-2004175

Electronic, 20th – 30th April 2020

Title:	LS on RAN2 DCP Open Issues

Release:	Release 16

Work Item:	NR_UE_pow_sav-Core

Source:	RAN WG2

To:	RAN WG1

Contact person:	Dylan Watts

Dylan.watts@interdigital.com

# 1	Overall description

RAN2 has been discussing remaining issues related to DCP, and would like to inform RAN1 of the following discussions/agreements:

1. Collision between DCP and RAR addressed to C-RNTI

RAN2 has discussed UE behavior when a DCP monitoring occasion overlaps with the ra-ResponseWindow or msgB-ResponseWindow. RAN2 understanding is that according to current TS 38.213 prioritization rules, if DCP collides with RAR addressed to C-RNTI (e.g. during BFR) and the search spaces are not quasi-collocated, DCP will be prioritized as it is type-3 CSS and thus impacting legacy RAR behavior.

RAN2 would like to ask RAN1 the following:

- To confirm RAN2 understanding that if DCP and RAR search spaces are not quasi-collocated, a collision between DCP and RAR addressed to C-RNTI will impact legacy RAR handling.

  - From RAN2 point of view, the understanding is that RAR addressed all RNTIs should be prioritized over DCP by the UE.

- RAN2 would like to ask if RAN1 has any concerns with the understanding above? If RAN1 doesn’t have any concerns, what is RAN1 preference on where to capture this behavior e.g. TS 38.213 or in TS 38.321 via a DCP monitoring exception rule similar to overlap with DRX Active time?

2. RAN2 agreements

RAN2 would also like to inform RAN1 of the following agreements:

Agreements

1	RAN2 confirms that the flags ps-TransmitPeriodicL1-RSRP and ps-TransmitPeriodicCSI are defined per cell group

2	The flags ps-TransmitPeriodicCSI and ps-TransmitPeriodicL1-RSRP are independent, and it is possible to control UE to report all types of periodic CSI apart from L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

3	The flag ps-TransmitPeriodicCSI  is renamed to ps-TransmitOtherPeriodicCSI

# 2	Actions

To RAN WG1:

ACTION: 	RAN2 respectfully requests RAN1 to take into account RAN2 agreements and questions above, and provide feedback, if any.

# 3	Dates of next TSG RAN WG 2 meetings

RAN2#110-e	1st – 12th June 2020	Online

RAN2#111	24th – 28th August 2020	Toulouse, France