3GPP TSG-RAN WG2 #110	R2-2004638

Electronic meeting, 1-12 June 2020

Agenda Item:	6.8.2.1

Source:	Ericsson

Title:	Text Proposal for on demand system information procedure

Document for:	Discussion, Decision

# 1	Introduction

The stage 2 38.305 is missing the on demand SI procedure description. In this a text proposal/draft CR has been provided.

# 2	Discussion

The stage 2 description of ond emand procedure needs to be reviewed and added.

- RAN2 to agree on the below Text Proposal/draft CR.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to agree on the below Text Proposal/draft CR.

3GPP TSG-RAN Meeting #110	R2-2004638

# Electronic meeting, 1st – 12th June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | draftCR | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of on demand PosSIB request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Pos-Core |  |  |  |  |  | Date: |  |  | 2020-05-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Besides the normal broadcast, it is also possible for the UE to retreive assistance data by means of on demand sytem information procedure. The on demand mechanism is currently not captured in the stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | On demand system information procedure has been briefly captured in the current broadcast chapter 7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | On demand system information procedure will be missing from stage 2 description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.1, (7.5.X) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning Of Change

## 7.5	Procedures for Broadcast of Assistance Data

### 7.5.1	General

Positioning assistance data can be included in positioning System Information Blocks (posSIBs) as described in TS 36.331 [13], TS 38.331 [14] and TS 36.355 [19]. The posSIBs are carried in RRC System Information (SI) messages. UE may also request posSIBs by means of on demand SI request in RRC Idle/Inactivate states and using on demand connected mode procedure while in connected mode as described in TS 38.331 [14]. The mapping of posSIBs (assistance data) to SI messages is flexibly configurable and provided to the UE in SIB1 for NG-RAN node TS 36.331 [13], TS 38.331 [14].

For each assistance data element, a separate posSIB-type is defined in TS 36.355 [19]. Each posSIB may be ciphered by the LMF using the 128-bit Advanced Encryption Standard (AES) algorithm (with counter mode) as described in TS 36.355 [19], either with the same or different ciphering key. The posSIBs which exceed the maximum size limit defined in TS 36.331 [13], TS 38.331 [14] shall be segmented by the LMF.

### 7.5.2	Broadcast Procedures

The general procedures for broadcast of positioning assistance data and delivery of ciphering keys to UEs is described in TS 23.273 [35]. This clause defines the overall sequences of operations that occur in the LMF, NG-RAN node and UE.

Figure 7.5.2-1: Procedures to support broadcast of assistance data.

1.	The LMF sends an NRPPa Assistance Information Control message to the NG-RAN node with an indication to start broadcasting assistance information. The message includes one or more System Information groups, where each group contains the broadcast periodicity and one or more posSIB types together with meta data to be scheduled in the same SI message. Each posSIB type may be ciphered and/or segmented at the LMF. The meta data may include an indication whether the posSIB type in the System Information group is ciphered or not, as well as an indication of an applicable GNSS type.

2.	The NG-RAN node includes the received System Information groups in RRC System Information Messages and corresponding scheduling information in SIB1 as described in TS 36.331 [13], TS 38.331 [14]. The UE applies the system information acquisition procedure according to TS 36.331 [13], TS 38.331 [14] for acquiring the assistance data information that is broadcasted.

3.	If the posSIB types were ciphered by the LMF, the LMF invokes the Nlmf_Broadcast_CipheringKeyData Notify service operation towards the AMF carrying one or more ciphering keys. For each ciphering key, the LMF includes a ciphering key value, a ciphering key identifier, a validity period, a set of applicable tracking areas, and a set of applicable posSIB types. The AMF may then distribute successfully stored ciphering keys and their validity times and validity areas to suitably subscribed UEs using a mobility management message as described in TS 23.273 [35]. The LMF repeats this procedure whenever a ciphering key changes.

4.	At any time after Step 1, the NG-RAN node may send a NRPPa Assistance Information Feedback message to the LMF providing feedback on assistance information broadcasting. The message may include an assistance information failure list indicating that certain posSIB types could not be configured for broadcasting by the NG-RAN node.

5.	If the assistance information in a System Information group changes, the LMF provides updated information in a NRPPa Assistance Information Control message.

6.	The NG-RAN node replaces the previously stored System Information groups with the new information received at Step 5 and includes the new System Information groups in RRC System Information Messages.

7.	If the LMF wants to abort the broadcast of a System Information Group, it sends a NRPPa Assistance Information Control message to the NG-RAN node including an indication to stop broadcasting the assistance information.

### 7.5.X	On demand System Information Procedures

UE in RRC Idle or RRC Inactive state may request positioning SI message(s) by sending msg1 (specific preambles) configured by NW for SI request or using msg3 RRCSystemInfoRequest as described in TS 38.331 [14].

UE in RRC connected mode may request posSIB(s) by sending an UL message (DedicatedSIBRequest) as described in TS 38.331 [14]. The NG-RAN Node provides the posSIB(s) using RRCReconfiguration message as described in TS 38.331 [14].

Figure 7.5.2-2: Procedures to support On demand delivery of assistance data.

The steps are same as described in section 7.5.2. The only change is for step 2 or subsequent step 6 as below.

2a.	The NG-RAN node includes the received System Information groups in RRC System Information Messages and corresponding scheduling information in SIB1 as described in TS 38.331 [14]. The UE checks the posSI-SchedulingInformation TS 38.331 [14] for deciding whether on demand request is to be made.

2b.	The UE requests either positioning SI(s) in RRC Idle/Inactive state or posSIB(s) while in RRC Connected state.

2c. The NG-RAN Node provides the positioning SI via broadcast for the request made in RRC Idle/Inactive state. The NG-RAN provides the requested posSIB(s) either via broadcast or unicast for the requet made in RRC Connected state.

6.	The NG-RAN node replaces the previously stored System Information groups with the new information received at Step 5 and includes the new System Information groups in RRC System Information Messages to be provided by means of broadcast or on demand system information procedure.

End of Change