# 3GPP TSG-RAN WG2 Meeting #110 electronic											R2-2005007

# 1st – 12th June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4301 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on minimum UE requirements for the number of RLC bearers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-06-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The minimum UE requirement for the number of RLC entities is not clear in TS 36.331 and it is better to clarify. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the minimum UE requirement for the number of RLC bearers to be: - 8 RLC bearers per UE, if the UE not supporting any of extendedNumberOfDRBs-r15, pdcp-Duplication-r15 and split bearer; - 8 RLC bearers per cell group, if the UE supporting pdcp-Duplication-r15 but not supporting extendedLCID-Duplication-r15, or the UE supporting split bearer but not supporting extendedNumberOfDRBs-r15; - 15 RLC bearers per UE, if the UE supporting split bearer but not supporting extendedNumberOfDRBs-r15. - 15 RLC bearers per cell group, if the UE supporting pdcp-Duplication-r15 and extendedLCID-Duplication-r15, or the UE supporting split bearer and extendedNumberOfDRBs-r15. Impact Analysis Impacted 5G architecture options: LTE Standalone, (NG)EN-DC, NE-DC Impacted functionality: Minumum UE requirement Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is a potential inter-operability problem, since the network and the UE may have different understanding on the minimum UE requirement regarding the number of RLC bearers. 2. If the UE is implemented according to the CR and the network is not, there is a potential inter-operability problem, since the network and the UE may have different understanding on the minimum UE requirement regarding the number of RLC bearers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network and the UE may have different understanding on the minimum UE requirement regarding the number of RLC bearers, and may therefore cause RRC reconfiguration failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------------------------------START OF CHANGE-------------------------------------------

11.1	UE capability related constraints

The following table lists constraints regarding the UE capabilities that E-UTRAN is assumed to take into account.

| Parameter | Description | Value | NB-IoT |
| --- | --- | --- | --- |
| #DRBs | The number of DRBs that a UE shall support | 8, 15 (NOTE2, NOTE3) | (0, 1, 2) NOTE1 |
| #RLC entities for DRB | The number of RLC entities for DRBs that a UE shall support | 8 per UE (NOTE 4, NOTE5), 8 per cell group (NOTE4, NOTE6) 15 per UE (NOTE4, NOTE7), 15 per cell group (NOTE4, NOTE8) | (0, 1, 2) NOTE1 |
| #RLC-AM | The number of RLC AM entities that a UE shall support | 10, 17 | (2, 3) NOTE1 |
| #minCellperMeasObjectEUTRA | The minimum number of neighbour cells (excluding black list cells) that a UE shall be able to store within a MeasObjectEUTRA. NOTE. | 32 | N/A |
| #minBlackCellRangesperMeasObjectEUTRA | The minimum number of blacklist cell PCI ranges that a UE shall be able to store within a MeasObjectEUTRA | 32 | N/A |
| #minCellperMeasObjectUTRA | The minimum number of neighbour cells that a UE shall be able to store within a MeasObjectUTRA. NOTE. | 32 | N/A |
| #minCellperMeasObjectGERAN | The minimum number of neighbour cells that a UE shall be able to store within a measObjectGERAN. NOTE. | 32 | N/A |
| #minCellperMeasObjectCDMA2000 | The minimum number of neighbour cells that a UE shall be able to store within a measObjectCDMA2000. NOTE. | 32 | N/A |
| #minCellTotal | The minimum number of neighbour cells (excluding black list cells) that UE shall be able to store in total in all measurement objects configured | 256 | N/A |
| NOTE: In case of CGI reporting, the limit regarding the cells E-UTRAN can configure includes the cell for which the UE is requested to report CGI i.e. the amount of neighbour cells that can be included is at most (# minCellperMeasObjectRAT - 1), where RAT represents EUTRA/UTRA/GERAN/CDMA2000 respectively. |  |  |  |
| NOTE1: #DRBs based on UE capability, #DRB = #RLC entities for DRB, #RLC-AM =#DRBs + 2. |  |  |  |
| NOTE2: Value 15 applies if the UE supports extendedNumberOfDRBs-r15. |  |  |  |
| NOTE3: The requirement is applicable in EN-DC, NGEN-DC and LTE standalone. |  |  |  |
| NOTE4: The requirement is applicable in EN-DC, NGEN-DC, NE-DC and LTE standalone. |  |  |  |
| NOTE5: The requirement is applicable if the UE not supporting any of extendedNumberOfDRBs-r15, pdcp-Duplication-r15 and split bearer. |  |  |  |
| NOTE6: The requirement is applicable if the UE supporting pdcp-Duplication-r15 but not supporting extendedLCID-Duplication-r15, or the UE supporting split bearer but not supporting extendedNumberOfDRBs-r15. |  |  |  |
| NOTE7: The requirement is applicable if the UE supporting split bearer but not supporting extendedNumberOfDRBs-r15. |  |  |  |
| NOTE8: The requirement is applicable if the UE supporting pdcp-Duplication-r15 and extendedLCID-Duplication-r15, or the UE supporting split bearer and extendedNumberOfDRBs-r15. |  |  |  |

---------------------------------------------END OF CHANGE---------------------------------------------