3GPP TSG-RAN WG2 #110-e	R2-2005119

# Electronic meeting, 1st - 12th June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1650 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on maximum number of supported PDSCH Resource Element mapping patterns |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-05-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the RAN1 LS R1-2002828, it should be clarified that the absence of pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot or pdsch-RE-MappingFR2-PerSymbol/pdsch-RE-MappingFR2-PerSlot imply in the support of the following values (for each corresponding field that is absent): - pdsch-RE-MappingFR1-PerSymbol: 10 - pdsch-RE-MappingFR1-PerSlot: 16 - pdsch-RE-MappingFR2-PerSymbol: 6 - pdsch-RE-MappingFR2-PerSlot: 16 Therefore, the aformentioned capabilities should be updated to take the above into account. Impact analysis Impacted 5G architecture options: Standalone, EN-DC, NGEN-DC, NE-DC, NR-DC Impacted functionality: PDSCH RE mapping patterns Inter-operability: If the network implements the CR and the UE does not, there is no inter-operability issue, since it is expected that the UE would be able to support the minimum value from the capabilities above, even if they are not reported. If the UE implements the CR and the network does not, there is no inter-operability issue as long as the network assumes that the UE support the minimum value from the capabilities above, even if they are not reported. If the network would assume that the UE supports more than the minimum the network may attempt to configure more patterns than the UE supports and the configuration would fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 4.2.7.10 Phy-Parameters - Clarified what the UE supports upon absence of pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot and pdsch-RE-MappingFR2-PerSymbol/pdsch-RE-MappingFR2-PerSlot; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be specified how many PDSCH RE mapping patterns the UE can support when any of the capabilities related to this feature are not reported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 4.2.7.10	Phy-Parameters

| pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot Indicates the maximum number of supported PDSCH Resource Element (RE) mapping patterns for FR1, each described as a resource (including NZP/ZP CSI-RS, CRS, CORESET and SSB) or bitmap. The number of patterns coinciding in a symbol in a CC and in a slot in a CC are limited by the respective capability parameters. Value n10 means 10 RE mapping patterns and n16 means 16 RE mapping patterns, and so on. If pdsch-RE-MappingFR1-PerSymbol is not included, the UE is mandated to support 10 RE mapping patterns coinciding in a symbol in a CC. If pdsch-RE-MappingFR1-PerSlot is not included, the UE is mandated to support 16 RE mapping patterns coinciding in a slot in a CC. | UE | Yes | No | FR1 only |
| --- | --- | --- | --- | --- |
| pdsch-RE-MappingFR2-PerSymbol/pdsch-RE-MappingFR2-PerSlot Indicates the maximum number of supported PDSCH Resource Element (RE) mapping patterns for FR2, each described as a resource (including NZP/ZP CSI-RS, CORESET and SSB) or bitmap. The number of patterns coinciding in a symbol in a CC and in a slot in a CC are limited by the respective capability parameters. Value n6 means 6 RE mapping patterns and n16 means 16 RE mapping patterns, and so on. If pdsch-RE-MappingFR2-PerSymbol is not included, the UE is mandated to support 6 RE mapping patterns coinciding in a symbol in a CC. If pdsch-RE-MappingFR2-PerSlot is not included, the UE is mandated to support 16 RE mapping patterns coinciding in a slot in a CC. | UE | Yes | No | FR2 only |
| precoderGranularityCORESET Indicates whether the UE supports receiving PDCCH in CORESETs configured with CORESET-precoder-granularity equal to the size of the CORESET in the frequency domain as specified in TS 38.211 [6]. | UE | No | No | No |
| pre-EmptIndication-DL Indicates whether the UE supports interrupted transmission indication for PDSCH reception based on reception of DCI format 2_1 as defined in TS 38.213 [11]. | UE | No | No | No |
| pucch-F2-WithFH Indicates whether the UE supports transmission of a PUCCH format 2 (2 OFDM symbols in total) with frequency hopping in a slot. This field shall be set to supported. | UE | Yes | No | Yes |
| pucch-F3-WithFH Indicates whether the UE supports transmission of a PUCCH format 3 (4~14 OFDM symbols in total) with frequency hopping in a slot. This field shall be set to supported. | UE | Yes | No | Yes |
| pucch-F3-4-HalfPi-BPSK Indicates whether the UE supports pi/2-BPSK for PUCCH format 3/4 as defined in 6.3.2.6 of TS 38.211 [6]. It is optional for FR1 and mandatory with capability signalling for FR2. | UE | CY | No | Yes |
| pucch-F4-WithFH Indicates whether the UE supports transmission of a PUCCH format 4 (4~14 OFDM symbols in total) with frequency hopping in a slot. | UE | Yes | No | Yes |