3GPP TSG-RAN WG2 Meeting #110-e	R2-2005682

Online, 1 – 12 June 2020

Agenda Item		:	6.9.4

Source		:	LG Electronics Inc.

Title		:	CHO Evaluation handling during Legacy HO

Document for		:	Discussion and Decision

1. Introduction

RAN2 has following conclusions related to CHO evaluation handling during mobility;

In RAN2#109 meeting [1],

RAN2 assumes that the UE does not apply CHO configuration when a new execution condition is met during HO/CHO. FFS whether and how this can be captured in RRC CR (not done for the CRs submitted to RANP).

In RAN2#109Bis meeting [2],

Agreements

4 Rely on existing Stage-2 text that UE stops evaluating execution condition and capture nothing additional in NR/LTE RRC specification about CHO execution conditions after the CHO condition is met and CHO execution is started.

In this contribution, we discuss an issue of whether the UE stops evaluating CHO execution during the legacy HO.

2. Discussion

RAN2 had discussed how the UE handles the CHO evaluation or execution during HO/CHO and concluded that RAN2 relies on existing Stage 2 text for this issue. That is, the UE stops evaluating the CHO execution condition after the CHO execution condition is met.

However, we think there is still an unclear point whether the UE evaluates the CHO execution condition during the legacy HO. This is because RAN2 only had an agreement for stopping the CHO evaluation in case of that the CHO execution condition is met. Moreover, it isn’t captured in the current Stage 2 and 3 specifications to stop evaluating CHO execution condition upon reception of the legacy HO command despite RAN2 had discussed and concluded.

According to the previous conclusion in RAN2#109, RAN2 assumes that the UE does not apply CHO configuration not only when the CHO execution condition is met but also when the legacy HO command is received. Considering that stopping evaluating the CHO execution condition is one of UE behaviours reflecting not to apply the CHO configuration, we think the final agreement in RAN2#109B seems not completely enough for the legacy HO scenario.

From our point of view, to prevent the unclear point of evaluating the CHO execution condition during the legacy HO, RAN2 needs to consider stopping evaluating the CHO execution condition in case of the legacy HO command is received. Then, since RAN2 has agreed to rely on existing Stage 2 text for the case of CHO execution condition is met and we think the text is pretty normative, we don’t need to specify this issue in Stage 3 and we can simply add to specify stopping evaluating the CHO execution condition when the legacy HO command is received in Stage 2.

Therefore, we ask RAN2 to consider our proposal based on the associated CR in R2-200xxxx.

Proposal: RAN2 is respectively asked to consider stopping CHO evaluation when the legacy HO command is received based on the associated CR in R2-200xxxx.

3. Conclusion

In this contribution, we have the following conclusion:

Proposal: RAN2 is respectively asked to consider stopping CHO evaluation when the legacy HO command is received based on the associated CR in R2-200xxxx.

4. Reference

[1] R2-2001661 Minutes LTE legacy, LTE TEI16 and NR/LTE Rel-16 Mobility topics, Vice Chairman (Nokia)

[2] R2-2003801 Report on LTE legacy, LTE TEI16 and NR/LTE Rel-16 Mobility topics, Vice Chairman (Nokia)