3GPP TSG-RAN WG2 Meeting #110	R2-200xxxx

- Electronic, 1 June – 12 June 2020

Agenda item:	5.4.1.1

Source:	Apple

Title:	Offline-007: DC Configuration

Document for:	Discussion & Decision

# Introduction

This is a summary of the following offline discussion on DC configuration:

- [AT110e][007][NR15] DC Configuration (Apple)

Scope: Treat R2-2005531, R2-2005532, R2-2005533, R2-2005534, R2-2005634, R2-2005635, R2-2004488, R2-2004489 (proponents are responsible to explain and drive)

Part 1: Decision whether to make corrections or not, identify agreeable corrections. Deadline: June 4, 0700 UTC.

Part 2: For agreeable parts, continuation to agree CRs. Deadline: June 10, 0700 UTC

This document covers the following contributions submitted to RAN2#110-e meeting:

PSCell Addition NR-DC - SMTC

R2-2005531	SMTC Configuration for PSCell Addition for NR-DC	Apple, ZTE Corporation, Sanechips, Qualcomm Incorporated	CR	Rel-15	38.331	15.9.0	1675	-	F	NR_newRAT-Core

R2-2005532	SMTC Configuration for PSCell Addition for NR-DC	Apple, ZTE Corporation, Sanechips, Qualcomm Incorporated	CR	Rel-16	38.331	16.0.0	1676	-	A	NR_newRAT-Core

R2-2005533	SMTC Configuration for PSCell Addition for NR-DC	Apple, ZTE Corporation, Sanechips, Qualcomm Incorporated	CR	Rel-15	38.306	15.9.0	0340	-	F	NR_newRAT-Core

R2-2005534	SMTC Configuration for PSCell Addition for NR-DC	Apple, ZTE Corporation, Sanechips, Qualcomm Incorporated	CR	Rel-16	38.306	16.0.0	0341	-	A	NR_newRAT-Core

4 Treated by email [007]

SCG establishment – MAC default

R2-2005634	MAC Default Configuration for SCG	Qualcomm Incorporated	CR	Rel-16	38.331	16.0.0	1685	-	A	NR_newRAT-Core

R2-2005635	MAC Default Configuration for SCG	Qualcomm Incorporated	CR	Rel-15	38.331	15.9.0	1686	-	F	NR_newRAT-Core

2 Treated by email [007]

Radio bearer config NR-DC NE-DC

R2-2004488	Clarification for radioBearerConfig and radioBearerConfig2	vivo	discussion

R2-2004489	Clarification for radioBearerConfig and radioBearerConfig2	vivo	CR	Rel-15	38.331	15.9.0	1608	-	F	NR_newRAT-Core

2 Treated by email [007]

Companies are invited to provide their views for each issue.

# Discussion: Part 1

## 2.1 Issue #1. SMTC Configuration for PSCell Addition for NR-DC (R2-2005531 to R2-2005534)

Regarding which issues that need resolution, it is suggested to use the reason for change from R2-2005531 to R2-2005534 as the input:

For EN-DC, the SMTC configuration for NR PSCell addition is provided in LTE RRCConnectionReconfiguration message. For NR PSCell change in MR-DC, the SMTC configuration is provided in secondaryCellGroup -> SpCellConfig -> reconfigurationWithSync. But for NR-DC, the SMTC configuration for NR PSCell addition and SN change is missing.

Q1.1) Do companies agree with the reason for change in R2-2005531 to R2-2005534?

| Company | Agree/Disagree | Comments (if any) |
| --- | --- | --- |
| Ericsson | Disagree | Our understanding is that the smtc is received by the UE via the reconfigurationWithSync field. Having a look at the ASN.1 we have: CellGroupConfig ::= SEQUENCE { cellGroupId CellGroupId, rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig OPTIONAL, -- Need N rlc-BearerToReleaseList SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity OPTIONAL, -- Need N mac-CellGroupConfig MAC-CellGroupConfig OPTIONAL, -- Need M physicalCellGroupConfig PhysicalCellGroupConfig OPTIONAL, -- Need M spCellConfig SpCellConfig OPTIONAL, -- Need M sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig OPTIONAL, -- Need N sCellToReleaseList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex OPTIONAL, -- Need N ..., [[ reportUplinkTxDirectCurrent ENUMERATED {true} OPTIONAL -- Cond BWP-Reconfig ]], (fields omitted) SpCellConfig ::= SEQUENCE { servCellIndex ServCellIndex OPTIONAL, -- Cond SCG reconfigurationWithSync ReconfigurationWithSync OPTIONAL, -- Cond ReconfWithSync rlf-TimersAndConstants SetupRelease { RLF-TimersAndConstants } OPTIONAL, -- Need M rlmInSyncOutOfSyncThreshold ENUMERATED {n1} OPTIONAL, -- Need S spCellConfigDedicated ServingCellConfig OPTIONAL, -- Need M ... } ReconfigurationWithSync ::= SEQUENCE { spCellConfigCommon ServingCellConfigCommon OPTIONAL, -- Need M newUE-Identity RNTI-Value, t304 ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000}, rach-ConfigDedicated CHOICE { uplink RACH-ConfigDedicated, supplementaryUplink RACH-ConfigDedicated } OPTIONAL, -- Need N ..., [[ smtc SSB-MTC OPTIONAL -- Need S ]] The field condition linked to the reconfigurationWithSync it states that the field is mandatory present upon SpCell change and PSCell addition. ReconfWithSync; The field is mandatory present in the RRCReconfiguration message: - in each configured CellGroupConfig for which the SpCell changes, - in the masterCellGroup at change of AS security key derived from KgNB, - in the secondaryCellGroup at: - PSCell addition, - SCG resume with NR-DC or (NG)EN-DC, - update of required SI for PSCell,; - change of AS security key derived from S-KgNB while the UE is configured with at least one radio bearer with keyToUse set to secondary and that is not released by this RRCReconfiguration message, Otherwiseit is optionally present, need M. The field is absent in the masterCellGroup in RRCResume and RRCSetupmessages.. Therefore, we believe that the problem pointed out in the CRs does not exist. |
| Qualcomm | Agree | We have noticed the field condition cited by Ericsson, and agree it is possible to indicate SMTC for NR-DC PSCell addition in target cell’s ReconfigurationWithSync. However, we think the field description of smtc within ReconfigurationWithSync missed the cases of NR-DC PSCell addition and SN change: ReconfigurationWithSync field descriptions. smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and NR PCell change. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon. For case of NR PCell change, the smtc is based on the timing reference of source PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.. As highlighted above, it only described that cases of NR PSCell change and NR PCell change, i.e. the cases of NR PSCell addition and SN change are missing. Note that its corresponding timing reference is also missed, which may cause ambiguity. Thus, we think the issue of the CR is valid because there is something broken in the current spec. If Ericsson don’t prefer to change ASN.1, we think we can try another way we list in Q2 (keeping ASN.1 and adding missed descriptions on NR PSCell addition and SN change in field description of smtc) |
| Apple | Agree | (proponent) The condition for reconfigurationWithSync includes the PSCell addition case, but the condition for smtc configuration in the reconfigurationWithSync doesnot include the PSCell addition case. That’s why we bring the CR set to correct it. For PSCell addition case, the smtc should be based on the timing reference of MCG PCell. > If the smtc is provided in reconfigurationWithSync of NR SCG configuration for PSCell addition by SN, SN is required to provide the smtc configuration based on MN PCell timing. I am not sure whether SN has the knowledge of UE’s PCell timing. > If the smtc is provided in RRCReconfiguration by MN, MN knows the timing and there is no problem to provide the smtc. In addition, for EN-DC PSCell addition, the smtc is provided by MN and in RRCReconfiguration. We assume SN may have problem to have the MN timing and could not provide the smtc based on MN timing. Therefore, we propose to follow the same way as EN-DC PSCell addition to provide the smtc configuration. |
| Huawei | Agree | Although we hold the opinion that providing the smtc is just an optimization (since UE can use the SMTC in measObjectNR to facilitate the synchronisation with target SSB or even blindly detecting the SSB on the given frequency), it is ok for us to capture the missing scenario. |
| Nokia | Disagree | For Rel-15, we prefer to have a simple solution without any change in current specification. - For PSCell addtion and SN change in NR-DC, it is up to NW to configure measObjectNR (incl. SMTC configuration) having the same SSB frequency and subcarrier spacing before. Otherwise, new capability and new IE should be introduced which will bring more complex in NW implementation (i.e. to support two branches, with or without capability). For Rel-16, we prefer to extend the scope of the SMTC configuration provided in secondaryCellGroup -> SpCellConfig -> reconfigurationWithSync, to include PSCell addtion and SN change in NR-DC. |
| Samsung | Disagree | Shared the view with the Ericsson. Cannot find any motivation to introduce new IE while the relevant IE is already there. We can consider updating field description if needed. |
| MediaTek | Agree | We think the intention is correct. Regarding to Ericsson’s comment, we have same understanding as Apple. It is the reconfigurationWithSync would be mandatory in case of PSCell addition case but smtc is not required to present. |
| vivo | Partly | We agree that adding SpCell change and PSCell addition make the specification clearer. However based on Ericsson comments, we also agree that it is kind of clarification. |

In R2-2005531 to R2-2005534, it is proposed to add the SMTC configuration in NR RRCReconfiguration message for NR PSCell addition and SN change.

If it is not approved, for PSCell addtion and SN change in NR-DC, the UE cannot know the timing information of the target PSCell for the initial cell search if UE is not configured with the measObjectNR (incl. SMTC configuration) having the same SSB frequency and subcarrier spacing before.

Q1.2) If the answer to Q1.1 is “Yes”, do you agree with the changes made in R2-2005531 to R2-2005534?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | See comment to Q1.1. We believe the problem pointed out by the CRs does not exist. |
| Qualcomm | Agree; And we are also fine with another solution | We co-sourced this CR. So, we agree the change. In Ericsson have concern on its ASN.1 change. We provide another way: - Alt-2: add missed descriptions on NR PSCell addition and SN change in field description of smtc: ReconfigurationWithSync field descriptions. smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell addition, SN change, NR PSCell change and NR PCell change. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon. For case of NR PCell change, the smtc is based on the timing reference of source PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. For the cases of NR PSCell addition to NRDC and SN change, the smtc is based on the timing reference of source NR PCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.. Note that people may argue that reconfiguratioWithSync is generated by target PSCell, which may not have timing difference with source Cell. However, SFTD measurements have been introduced in CG-ConfigInfo message, so we understand source can forward SFTD to target upon NR PSCell addition. Thus, no further spec change is required. We are fine with either way |
| Apple | Agree | For the alt-2 provided by QC, we are fine with it if NW can provide the smtc based on MN PCell timing. |
| Huawei | Agree | Considering the timing should be based on PCell, we prefer not to put extra burden for the PSCell to fetch the timing difference. |
| Nokia | Disagree | See response to Q1.1 |
| Samsung | Disagree | QC’s alt 2 seems the way to go. |
| MediaTek | Agree with comment | First we think that UE capability is not necessary. If the UE does not support the new field, the UE just ignores it as it is in non-critical extension. No functional problem will occur, as SMTC is supposed only to speed up (but not to enable) cell detection (i.e. it is used as additional assistance information). On the Alt-2 provide by QC, we prefer to add new signaling and follow the same principle as in EN-DC (i.e. original proposal). Alt-2 requires extra effort on exchange of SFTD information and seems not necessary. The current wording of Alt-2 is also confusing. The UE does really know the difference between “SN Change” and “PSCell Change”, so it determines the timing reference cell based on whether it is MCG configuration or SCG configuration. If SN change and PSCell change share the same smtc configuration, it would be unclear to the UE on how to use the field. |
| vivo |  | We have no strong view about the CR. |
| Ericsson2 | Disagree but | If companies really want to fix this now, we think that clarifying this in the field condition is the only way to go. We are not ok on adding any new field in the ASN.1 and we are not ok, in fact, to have any NBC change. For these reason, our compromise would be to modify the field description as follow (as similarly proposed by other companies): smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change, NR PCell change, and NR PSCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon. For case of NR PCell change and NR PSCell addition, the smtc is based on the timing reference of source PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.. |

## 2.2 Issue #2. MAC Default Configuration for SCG (R2-2005634, R2-20056345)

The MAC-CellGroupConfig IE is an optional Need M, therefore network may or may not include it when configuring or adding a cell group.

Per the spec, when configuring or adding the MCG (with FullConfig or during establishment or during IRAT HO to NR) the UE shall apply the default configuration.

Q2.1) Do you agree that the spec has ensured when MCG is being established/configured, if MAC configuration is not provided UE applies the default MAC configuration as per the 9.2.2 table?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Ericsson | Agree but | However, the reason of having it during the RRC establishment is because the UE did not get yet the RRCReconfiguration and thus it needs to use the MAC default configuration. In general, it will look a bit strange if the network does not include the mac-CellGroupConfig when sending the first RRCReconfiguration. |
| Apple | Agree |  |
| Qualcomm | Agree |  |
| Huawei | Agree but | Agree with Ericsson. MCG addition needs default configuration before UE acquires RRCReconfiguration. The case for SCG is different, because the network is able to include all the MAC configuration to the UE when adding SCG. |
| Nokia | Agree |  |
| Samsung | Agree | Minor comment to cover sheet of the CRs: the architecture option should also include '(NG)EN-DC', and NR-SA should be removed. |
| MediaTek | Disagree | Based on Ericsson and Huawei’s comment, we think better way is to say that the NW would provide mac-CellGroupConfig for SCG while establishing the SCG for the first time. |
| vivo | agree |  |

Q2.2) Do you agree that per the spec, when network adds an SCG, if MAC-CellGroupConfig IE was not included (or partially included) in the reconfiguration message, the expected UE behavior is not defined?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Ericsson | Maybe | As we stated in our previous comment, the situation on when adding the SCG is a bit different from when the UE initiates the RRC establishment. In general, we believe that there is no reason for the network to not add the mac-CellGroupConfig when sending the first RRCReconfiguration. However, if the other companies believe that this is good to have, we are fine to go with majority view. |
| Apple | Agree | From spec point of view, NW is allowed to not provide mac-CellGroupConfig wen adding the SCG. So it’s better to make the UE behaviour clear. |
| Qualcomm | Agree | The expectation is always the network should include mac-CellGroupConfig, however since this IE is not mandatory (optional Need M), as a UE vendor, we’re not sure if UE should declare RLF due to misconfiguration or should UE fall back to default configuration. |
| Huawei | Disagree | Agree with Ericsson that the network will always include the mac-CellGroupConfig upon SCG addition. Therefore nothing needs to be modified. We usually don’t add something simply due to wrong configuration of the NW. Employing a default MAC configuration for SCG addition will add extra UE requirement and is non-backward compatible. |
| Nokia | Agree | This is okay to be clarified otherwise may cause confusion |
| Samsung | Agree | From our recollection, to not include mac-CellGroupConfig is allowed to support SN-terminated MCG bearers only (i.e. PDCP in SN and RLC/MAC/PHY in MN), even though this scenario seems not common at all… So the specification should be clear when mac-CellGroupConfig is not configured. |
| MediaTek | See Comment | We think that a better way is just to specify “the network will always include this field for SCG cell group upon SCG addition” in the field description of mac-CellGroupConfig. Then we don’t have to discuss the UE behaviour due to wrong NW configuration. In case there is SN-terminated MCG bearers (as point out by Samsung), the whole CellGroupConfig is not configured. From UE point of view, the SCG does not exist and this is not the case we are discussing. |
| vivo | agree |  |

Q2.3) as solution to address the concern in Q2.2, do you agree to let the UE applies the default MAC configuration upon SCG establishment if MAC-CellGroupConfig IE was not included (or partially included) in the reconfiguration message?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Ericsson | Maybe | In general, we believe that there is no reason for the network to not add the mac-CellGroupConfig when sending the first RRCReconfiguration. However, if the other companies believe that this is good to have, we are fine to go with majority view. |
| Apple | Agree |  |
| Qualcofmm | Agree | If mac-CellGroupConfig IE was missed, and in order to handle this misconfiguration gracefully, allowing the UE to apply a default configuration is a preferred approach. |
| Huawei | Disagree | The network will always include the mac-CellGroupConfig upon SCG addition. Therefore nothing needs to be modified. We usually don’t add something simply due to wrong configuration of the NW. Employing a default MAC configuration for SCG addition will add extra UE requirement and is non-backward compatible. |
| Nokia | Agree | Okay to go with majority view |
| Samsung | Agree | - |
| MediaTek | Disagree | See our comment in Q2.2. There is no need to specify the UE behaviour for bad NW configuration. |
| vivo | agree |  |

Q2.4) if you don’t agree with the solution provided in Q2.3, please provide an alternate solution.

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

## 2.3 Issue #3. Clarification for radioBearerConfig and radioBearerConfig2 (R2-2004488, R2-2004489)

In TS 38.331, the current field descriptions of radioBearerConfig and radioBearerConfig2 in RRCReconfiguration are insufficient to clearly express the corresponding usage.

radioBearerConfig issue:

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. In EN-DC this field may only be present if the RRCReconfiguration is transmitted over SRB3.

nr-SecondaryCellGroupConfig Includes the NR RRCReconfiguration message as specified in TS 38.331 [82]. In this version of the specification, the NR RRC message only includes fields iab-F1AP-TransferOverSRB-r16, secondaryCellGroup, conditionalReconfiguration and/ or measConfig. If nr-SecondaryCellGroupConfig is configured, the network always includes this field upon MN handover to initiate an NR SCG reconfiguration with sync and key change.

Q3.1) Do companies agree that radioBearerConfig can also be used for NR-DC and NE-DC case?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Ericsson | Agree | We did not fully get the issue pointed out in these CRs. First, the clarification that the field radioBearerConfig is only absent is a very strange formulation and we would like to avoid it. The reason why we have the restriction for EN-DC is because the NR radioBearConfig, when SRB3 is not configured, is delivered within the E-UTRA RRCConnectionReconfiguration message (i.e., the SN sends the radioBearConfig via INM to the MN). Further, to our understanding, radioBearerConfig and radioBearerConfig2 can be used inter-changeably in NR-DC and therefore, we do not see the need to have such restriction. For these reasons, unless there is a real need/issue for having this change, we prefer to not pursue these CRs. |
| Apple | Agree |  |
| Quacomm | Agree |  |
| Huawei | Agree |  |
| Nokia | Agree, nothing required to be clarified | It is same as Option 2 as indicated in email discussion R2-1814575 Summary on email discussion [103#48] on RRC details for NR-DC. Hence, nothing needed to be clarified. |
| Samsung | Agree | - |
| MediaTek |  | A little bit unclear on which radioBearerConfig in this question is refer to. Case 1 - (For NR-DC and NE-DC) The radioBearerConfig in MN RRC message  yes it could be used. Case 2 - (for NR-DC) The radioBearerConfig in SN RRC message sent via SRB3  yes it could be used Case 3 – (for NR-DC) The radioBearerConfig in SN RRC message embedded in MN RRC message and sent via SRB1  no it cannot be used. In this case, the radioBearerConfig2 in MN RRC message is used. We understand the intention of the CR is to clarify the case 3 and we think the intention is correct. |
| vivo | agree | The issue is that it is not totally clear how the radioBearerConfig is used in NR-DC and NE-DC case. From our understanding that, RadioBearerConfig is only used as the below two cases for NR-DC. Case 1: configure MN terminated radio bearers via SRB1. Case 2: configure SN terminated radio bearers via SRB3. The network only uses RadioBearerConfig2 to configure SN terminated radio bearers via SRB1. It is similar configuration operation with EN DC because we also have the similar description in mrdc-SecondaryCellGroup. mrdc-SecondaryCellGroup Includes an RRC message for SCG configuration in NR-DC or NE-DC. For NR-DC (nr-SCG), mrdc-SecondaryCellGroup contains the RRCReconfiguration message as generated (entirely) by SN gNB. In this version of the specification, the RRC message can only include fields secondaryCellGroup and measConfig. For NE-DC (eutra-SCG), mrdc-SecondaryCellGroup includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration. It is the reason why we give the CR to claim that only case1 and case2 should be supported by the sentence “In NR-DC, this field is only absent if the RRCReconfiguration is received within mrdc-SecondaryCellGroup.” |

Q3.2) Do companies agree the CR in R2-2004489, to clarify the field description of radioBearerConfig, that in NR-DC it is only absent if the RRCReconfiguration is received within mrdc-SecondaryCellGroup, like EN-DC case.

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | We did not fully get the issue pointed out in these CRs. First, the clarification that the field radioBearerConfig is only absent is a very strange formulation and we would like to avoid it. The reason why we have the restriction for EN-DC is because the NR radioBearConfig, when SRB3 is not configured, is delivered within the E-UTRA RRCConnectionReconfiguration message (i.e., the SN sends the radioBearConfig via INM to the MN). Further, to our understanding, radioBearerConfig and radioBearerConfig2 can be used inter-changeably in NR-DC and therefore, we do not see the need to have such restriction. For these reasons, unless there is a real need/issue for having this change, we prefer to not pursue these CRs. |
| Apple | Disagree | We do not think the change has UE impact. |
| Qualcomm | Partially Agree | the "only" word is causing confusion, and maybe it should be removed from statement, as the radioBearerConfig IE may be absent when RRCReconfig doesn't involve any RB change. |
| Huawei | Disagree | Agree with QC that the “only” is wrong. Even if “only” is removed, the sentence seems quite obvious: In NR-DC, this field is absent if the RRCReconfiguration is received within mrdc-SecondaryCellGroup. There is no reason for the network to include radioBearerConfig in mrdc-SecondaryCellGroup. So we think nothing needs to be modified. |
| Nokia | Disagree, nothing is required to be clarified |  |
| Samsung | Disagree | We don’t see any problem with the current specification. Both radioBearerConfig and radioBearerConfig2 will not be included in RRC reconfiguration message of SCG-container, which is clear from the functionality of the IEs and the field descriptions. |
| MediaTek | Need rewording | Suggest to reword like this: "In NR-DC, this field can't be present if the RRCReconfiguration is received within mrdc-SecondaryCellGroup" Suggest also to add corresponding restriction to radioBearerConfig2: "In NR-DC or NE-DC, this field may only be present if the RRCReconfiguration is received via SRB1." |
| vivo | Agree | Like the answer in Q3.1, we would like to cover the case1 and case2 by the sentence “In NR-DC, this field is only absent if the RRCReconfiguration is received within mrdc-SecondaryCellGroup.” Using “only” can also cover case2 i.e., SRB3 case. |

radioBearerConfig and radioBearerConfig2 issue:

Since that both radioBearerConfig and radioBearerConfig2 can be used in NR-DC and NE-DC, it is not clear how to use them.

Clarify which of the following usages applies to radioBearerConfig and radioBearerConfig2 in NE-DC and NR-DC:

- Option1: The usage of RadioBearerConfig and radioBearerConfig2 in NE-DC and NR-DC are the same;

- Option2: RadioBearerConfig is used to configure MN terminated radio bearers via SRB1 and to configure SN terminated radio bearers via SRB3, while RadioBearerConfig2 is only used to configure SN terminated radio bearer via SRB1.

radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. This field can only be used if the UE supports NR-DC or NE-DC.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. In EN-DC this field may only be present if the RRCReconfiguration is transmitted over SRB3.

Q3.3) Do companies think which of the above usages applies to radioBearerConfig and radioBearerConfig2 in NE-DC and NR-DC.

| Company | Option1/Option2 or other | Comments |
| --- | --- | --- |
| Ericsson | Option 1 (no specification change) | To our understanding, radioBearerConfig and radioBearerConfig2 can be used inter-changeably in NR-DC and therefore, we do not see the need to have such restriction. For these reasons, unless there is a real need/issue for having this change, we prefer to not pursue these CRs and not have any specification change. |
| Huawei | Option 1 | Agree with Ericsson |
| Nokia | Option 1 | Agree with above two network vendors |
| Samsung | Not sure anything is needed | The usage of IEs is already clear. It has in principle no dependency to architecture option |
| MediaTek | Option 1 | We agree that radioBearerConfig and radioBearerConfig2 can be used inter-changeably. But we think the intention of the CR is not to prevent this use case. |
| vivo | Option2 | If majority is option 1, we would like to capture something to give a clarification. |

# Conclusion: Part 1

Based on the above, RAN2 is request to agree the following proposals:

TBD

# Reference