3GPP TSG RAN WG2 #110-e	draft R2-20xxxxx

Electronic meeting, 1 – 12 Jun 2020

Source: 	Huawei, HiSilicon

Title: 	[draft] Summary for Offline [028][Other] Inter-Freq measurements without Gaps

Agenda Item:	6.19.1

Document for:	Discussion and decision

# Introduction

This document is to collect companies’ views on inter-frequency measurements without gaps:

- [AT110e][028][Other] Inter-Freq measurements without Gaps (Huawei)

Scope: Treat R2-2004367, R2-2005445, R2-2005446, R2-2005447, R2-2004477 (R2-2006017), R2-2004824, R2-2004825, R2-2004757, R2-2004726, R2-2005424 (proponents are responsible to explain and drive)

Part 1: Identify agreeable changes. Deadline: June 4, 0700 UTC. (Remaining parts if needed can be revisited on-line).

Part 2: For agreeable parts, continuation to agree CRs. Deadline: June 10, 0700 UTC

# Discussion

## Issues related to CR drafting

In the RAN4 LS (R2-2004367/R4-2005350), an open issue was left to RAN2 decision:

- Option A: update the UE capability simultaneousRxDataSSB-DiffNumerology to indicate whether the UE supports concurrent intra-frequency measurement on serving cell or neighbouring cell or inter-frequency measurement without measurement gap and PDCCH or PDSCH reception from the serving cell with a different numerology;

- Option B: introduce a new UE capability to indicate whether the UE supports concurrent inter-frequency measurement without measurement gap and PDCCH or PDSCH reception from the serving cell with a different numerology as defined in clause 8 and 9 of TS 38.133

Q1: Which option do you prefer?

| Company | Option A/B | Comments |
| --- | --- | --- |
| Huawei | Option A | When the target SSB is contained in the active BWP, the reason of gapless measurement for intra-f and inter-f should be the same: RF retuning is not needed. Therefore, if UE can simultaneously perform data transmission (SCS1) with serving cell and intra-f measurement (SCS2, different from SCS1), it can perform concurrent data transmission (SCS1) with serving cell and inter-f measurement (SCS2) as well. |
| MediaTek | Option B | First, we think it is good practice to extend R15 capability to support more feature. And we think that there are actually three SCS here - The SCS of data (SCS1) - The SCS of intra-frequency SSB (SCS2) - The SCS of inter-frequency SSB (SCS3) We are not sure whether reusing of original capability meaning that the UE shall be able to receive the 3 different of SCS. We think it would be safer to separate the capability. |
| Qualcomm Incorporated | Option B | It is not a good idea to extend the existing UE capability parameter to cover something new. |
| Ericsson | Option A | In principle, both solution work. We have a slight preference to Option A since it does not need to create a completely new capability just for the inter-frequency measurement without measurement gap. However, we can go with majority view. |
| CMCC | Option A | In our view, from UE implementation perspective, if the measured SSB is within UE active BWP, measurement and data reception with the different SCS is the same from UE implementation perspective, no matter the measurement is inter-frequency measurement without MG or intra-frequency measurement. Meanwhile, in RAN4 LS, it is mentioned that both options are feasible from RAN4 perspective, which means that reuse the existing capability is feasible. |

RAN4 also agreed to explicitly enable “inter-frequency measurement without MG” feature with a release 16 configuration flag.

- If network configures the flag, when SMTC is partially overlapped with network configured MG, UE perform inter-frequency without MG measurement outside gap

- If network does not configure the flag, when SMTC is partially overlapped with network configured MG, UE perform inter-frequency measurement within gap.

Therefore, it is suggested to add the flag into MeasConfig.

Moreover, RAN4 has just agreed in R4-2007745 that the capability of supporting inter-frequency measurements without gap is optional with UE capability signalling. Corresponding changes can be made to the 38.306 CR.

Q2: Do you have any comment on the above (introducing a flag, and changing the capability to optional in 38.306 CR)?

| Company | Comments |
| --- | --- |
| Huawei | No comment. |
| MediaTek | We agree to add a flag in MeasConfig and also fine to change the capability to optional. |
| Qualcomm Incorporated | All agreeable. |
| Ericsson | All are ok |
| CMCC | Agree |

## 38.331 CR (R2-2005445, R2-2004824) & 38.306 CR (R2-2005446, R2-2004825)

The capability related to simultaneousRxDataSSB-DiffNumerology is discussed in Q1, thus will not be repeated in this section.

Q3: Which container do you think should include the inter-f gapless measurement capability? UE-NR-Capability or UE-MRDC-Capability or both?

| Company | NR/MR-DC/both | Comments |
| --- | --- | --- |
| Huawei | NR | The capability is also applicable to UE in EN-DC and NE-DC, however, for EN-DC and NE-DC, only the NR node is able to configure the inter-f gapless measurement. |
| MediaTek | Both | It is not just about which node should be aware of this capability. It is also about whether we want to separate the capability for EN-DC and NR (Separate the case for NR as MCG or SCG). Although we agree that the measurement is similar no matter NR is in MCG or SCG, we however see some benefit to have separate capability for IOT purpose. We prefer to have separate capability bit for this. |
| Qualcomm Incorporated | At least NR, but | We should react based on RAN4 feature list. |
| Ericsson | NR but | We agree with Qualcomm that we may need to wait for RAN4 feature list to have a more clear view about this. |
| CMCC | NR | We think this capability is applicable to all scenarios, but only NR can configure this. But OK to wait for RAN4 feature list. |

Q4: Do you think the inter-f gapless measurement capability should differentiate between FR1 and FR2?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei | No | In R15, SSB based intra-frequency measurement can be performed without gaps when the target SSB is contained in the active BWP, which is captured in 38.300 without differentiation between FR1 and FR2. The idea of inter-f gapless measurement is the same: target SSB is in active BWP so RF retuning is not needed. It does not make a difference whether it is in FR1 or FR2. |
| MediaTek | Yes | We think that this could follow the same design as simultaneousRxDataInterSSB-DiffNumerology, where there is FR1/FR2 separation. Actually, this should be designed by RAN4. But anyway, we would be fine to follow majorities. |
| Qualcomm Incorporated | FFS | We should react based on RAN4 feature list. |
| Ericsson | FFS | We agree with Qualcomm that we may need to wait for RAN4 feature list to have a more clear view about this. |
| CMCC | No | But can wait for RAN4 feature list |

The rapporteur suggests to take R2-2005445 (38.331 CR) and R2-2005446 (38.306 CR) as a baseline.

Q5: Apart from the issues discussed in Q1~Q4, do you have any other comment to R2-2005445 (38.331 CR)?

| Company | Comments |
| --- | --- |
| Huawei | No comment. |
| MediaTek | <1> For the new added flag in MeasConfig, we think there is no need to have procedure text for this flag. The procedure does not provide too much information. <2> In the field description of new added flag (interFrequency-NoGap-R16), it should clarified that it is used for SSB based measurement and also it is for the case that “while the target SSB is completely contained in the active BWP”. BTW, according to the discussion in capability section, we have to separate the part for capability and function. The capability related CR need to be endorsed and then merged to be the mega CR. |
| Ericsson | Should be okay but agree with MediaTek for the part related to separate changes for capability and function. |
| CMCC | OK with the changes proposed by MediaTek, and can wait for other companies’ comments, then we can revise the CR accordingly. |

Q6: Apart from the issues discussed in Q1~Q4, do you have any other comment to R2-2005446 (38.306 CR)?

| Company | Comments |
| --- | --- |
| Huawei | No comment. |
| MediaTek | Seems ok except for the part discussed in Q1 ~ Q4. |

## 38.300 CR (R2-2006017)

Q7: Do you agree with the changes in R2-2006017?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei | Yes |  |
| MediaTek | No | We think that 38.300 CR for this feature is not necessary. It conflict with the agreed in principle CR R2-2004808 (NeedForGap) in last meeting. In the CR R2-2004808, it already specifies that whether gap is required for inter-frequency measurement depending on UE capabilities. This also cover the case that is discussed here. Therefore, we prefer not to have further change on 38.300. |
| Qualcomm Incorporated | No | Technical content of the CR looks correct, but collision with in-principle agreed CR in R2-2004808 should be looked at. Readability of the section becomes quite poor if the changes from the two CRs are applied. |
| Ericsson | Maybe No | As also pointed out by MediaTek and Qualcomm, maybe for the time being there is no need to have a stage 2 CR. Once that the CR R2-2004808 is implemented we can double check whether some delta is needed. |
| CMCC | Yes | 38.300 CR is necessary since this capability is different from NeedForGap. So far we don’t think there is collision between these 2 CRs. We are also OK with Ericsson’s suggestion to have stage 2 CR later when the already principle agreed CR is implemented. |

## Reply LS (R2-2005424)

Q8: Do you think a reply LS is needed? If Yes, any comment to R2-2005424 is welcome.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei | Yes |  |
| MediaTek | No urgent | It seems RAN4 does not really need our reply to make some decision. The only open question is about this Option A and Option B on simultaneousRxDataInterSSB-DiffNumerology. But it is up to RAN2 to make the decision. So, we don’t really think a reply LS is needed. But would be fine if majorities want to send it. |
| Qualcomm Incorporated | Yes | The content needs to be updated based on agreements reached in this meeting. |
| Ericsson | Maybe Yes | In principle, RAN4 is not asking to RAN2 any reply. However, maybe is good to send the agreements we will take on this topic so to make them aware. |
| CMCC | Yes |  |

## R2- 2004757

Proposal 1 is covered in Q1.

Proposal 2: Suggest to send an LS to RAN4 on the indication of simultaneousRxDataSSB-DiffNumerology in cases of intra and inter-frequency gapless measurement when the SSB is outside of active BWP, which are introduced in dynamic NeedForGap feature.

Q9: Do you think a reply LS is needed?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei | No | In our understanding, if SSB is outside of the active BWP and the UE wants to perform measurement without gaps, a free RF is needed (expanding bandwidth is not feasible because it introduces interruption and should not be an autonomous behaviour of UE), therefore simultaneousRxDataSSB-DiffNumerology is not relevant. We are open on this issue and would like to hear from more companies. |
| MediaTek | Tend to think No | While the measurement is outside the current active BWP, we have similar view as Huawei that a different RF chain is needed and in this case, imultaneousRxDataSSB-DiffNumerology is not relevant. So, nothing need to be further specified here. However, if RAN2 could not have common understanding, we are also fine to ask RAN4. |
| Qualcomm Incorporated | Could wait RAN4 | We understand RAN4 is discussing the same issue (at least there are submitted papers in RAN4). It is our understanding that the UE indication of “gap” and “no-gap” needs to take into account all configured BWPs. Whereas RAN4’s enhancement is applied more dynamically depending on the BWP that is currently active. |
| Ericsson | No |  |
| CMCC | No | We have similar views with Huawei and MediaTek, simultaneousRxDataSSB-DiffNumerology is not relevant if UE has a free RF chain. |

## R2- 2004726

Intel summarizes the cases for inter-frequency measurement (the rapporteur took the liberty to change the first “gap” to “no-gap”):

- If the UE indicates “no-gap” in NeedForGap:

  - Gap is not configured regardless of SSB is within the active BWP or not

- If the UE indicates “gap” in NeedForGap:

  - If any SSB for inter-frequency measurement is not within the active BWP:

    - Gap is configured

  - If all SSBs for inter-frequency are within the active BWP:

    - If the UE capability indicates NOT support of gapless measurement in inter-frequency:

      - Gap is configured

    - If the UE capability indicates support of gapless measurement in inter-frequency:

      - Gap is not configured

      - If network still configure gap:

        - The UE performs measurement using gap (but this should be up to UE implementation)

Proposal: there is no RAN2 impact beside introduction of the UE capability signal to support the inter-frequency gapless scenario.

Q10: Do you agree with the above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei | No | 1) The procedure disregards RAN4’s request to add an R16 flag to enable/disable the inter-f gapless measurement 2) When configuring gaps, all configured BWPs are considered, not only active BWP. A reference is the R15 intra-frequency gapless measurement defined in 38.300: - For SSB based intra-frequency measurement, a measurement gap configuration is always provided in the following case: - Other than the initial BWP, if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP. Activation/de-activation of BWPs is controlled by DCI, whereas gap configuration cannot be adjusted according to DCI. Therefore, when configuring gaps, all configured BWP are considered, and the activation/de-activation of BWPs only affects whether the UE will actually use the gap. |
| MediaTek | See comment | Actually, we are not sure the intention of the proposal in R2-2004726. It is clear in RAN4 LS that a control flag is needed. Is the intention to revert RAN4 agreement? |
| Qualcomm Incorporated | No | We do not fully understand the logic of this document, and how they conclude no network configuration is needed. |
| Ericsson | No | This seems to contradict/revert what has been decided already in RAN4. |
| CMCC | No | - First, RAN4 already agreed to introduce a network flag to enable/disable the inter-frequency gapless feature, The motivation is to avoid the negative impact on legacy network. For example, Rel-15 network will not consider this new feature when configure the gap, so it could happen that network configure gap but UE will not use it since it is capable of gapless measurement. - Secondly, regarding the following bullet. - If network still configure gap: - The UE performs measurement using gap (but this should be up to UE implementation) RAN4 already reach agreements on this case. If network still configure gap, the UE behaviour or UE requirements should follow 38.133, which is also mentioned in RAN4 LS, This is not in RAN2 scope. |

# Conclusion

To be added.