3GPP TSG RAN WG2 #110-e	draft R2-20xxxxx

Electronic meeting, 1 – 12 Jun 2020

Source: 	Huawei, HiSilicon

Title: 	Summary for [AT110e][033][Other] Overheating (Huawei)

Agenda Item:	6.20.1.1

Document for:	Discussion and decision

# Introduction

This document is to kick off the below offline discussion:

- [AT110e][033][Other] Overheating (Huawei)

Scope: Treat R2-2005401, R2-2005404, R2-2005402, R2-2005403 (proponents are responsible to explain and drive)

Part 1: Identify agreeable changes. Deadline: June 4, 0700 UTC.

Part 2: For agreeable parts, continuation to agree CRs. Deadline: June 10, 0700 UTC

# Discussion

Based on the email discussion [Post109bis-e][050][TEI16] Overheating [1], the proposed conclusions are provided below.

Proposal 1 (7/7): In (NG)EN-DC, the new field for overheating assistance information in LTE RRC (TS36.331) refers to the NR OverheatingAssistance IE as specified in TS 38.331.

Proposal 2 (7/7): In (NG)EN-DC, if the new field for NR overheating assistance information encapsulated in LTE message is reported by the UE, the MN forwards this encapsulated information to the SN.

Proposal 3 (6/7): For Rel-15 legacy overheating IEs, the reducedCCsDL/UL (for (NG)EN-DC and NR-DC), reducedBW-FR1/FR2-DL/UL (for NR-DC) or reducedMIMO-LayersFR1/FR2-DL/UL (for NR-DC) are interpreted as for both MCG and SCG, the MN can indicate the allowed CC/BW/MIMO to the SN for coordination on reduced configuration.

Proposal 4 (6/7): In (NG)EN-DC, the interpretation for the Rel-15 legacy overheating IE (i.e. reducedCCsDL/UL) is not changed, i.e. it is always interpreted as the preference for both MCG and SCG.

Proposal 5 (7/7): In (NG)EN-DC, introduce a new UE capability in LTE capability container for the new field (i.e. overheating assistance information for SCG) in LTE assistance information message.

Proposal 6 (6/7): In (NG)EN-DC, MN determines the configuration for overheating assistance information for SCG. The configuration for the new overheating IE comes together with the configuration for the legacy overheating IE.

Proposal 7 (6/7): UE can report reducedMaxCCs in both legacy overheating IE (overheatingAssistance-r14) and new overheating IE (overheatingAssistanceForSCG-r16), reducedMaxCCs in legacy IE is intended for MCG+SCG, reducedMaxCCs in new IE is intended for only SCG.

Proposal 8 (5/7): SN indicating/suggesting whether to enable the new overheating information reporting for SCG is not supported in TEI16.

Proposal 9 (6/7): SN proposed value of reducedMaxCCs in CG-Config is not supported in TEI16.

Proposal 10 (7/7): The UE supporting new overheating assistance information for SCG shall also indicate support of legacy overheatingInd.

Proposal 11 (6/7): The reporting of new overheating field and legacy overheating field are controlled by one shared prohibit timer.

## Q1: Please provide the comments for the above proposals if any.

| Company | Comments |
| --- | --- |
| MediaTek | We are not fully aligned with majorities in some of proposals but fine to compromise. |

If the above proposals can be agreed, the associated CRs are provided in [2][3][4].

## Q2: Please provide the comments for the associated CRs in [2][3][4] if any.

| Company | Comments |
| --- | --- |
| MediaTek | R2-2005402 (36.331) <1> In 5.6.10.3, we should not refer to 5.7.4.3. The 38.331 5.7.4.3 is used for transmission of NR UEAssistanceInformation message. It is not suitable to have LTE RRC refer to this section. <2> For the overheating configuration – There is no need to use BOOLEAN. I think no present is the same as “FALSE”, thus ENUMERATED {true} is more suitable. R2-2005403 (38.331) <1> The way capture in current draft CR is inappropriate. We should have a new section in 38.331 (e.g. 5.7.4.3a Setting the contents of OverheatingAssistance) and the 36.331 should refer to this section on setting of the container. <2> In IE ConfigRestrictInfoSCG, you can combine the new fields with the previous R16 extension block and add suffix –r16 for that. |

# Conclusion

To be added.

# Reference

[1] R2-2005401	Report for [Post109bis-e][050][TEI16] Overheating	Huawei, Huawei Device

[2] R2-2005402	36.331 CR for overheating in (NG)EN-DC and NR-DC

[3] R2-2005403	38.331 CR for overheating in (NG)EN-DC and NR-DC

[4] R2-2005404	36.306 CR for overheating in (NG)EN-DC and NR-DC