3GPP TSG RAN WG2 #110-e	draft R2-20xxxxx

Electronic meeting, 1 – 12 Jun 2020

Source: 	Huawei, HiSilicon

Title: 	Summary for [AT110e][033][Other] Overheating (Huawei)

Agenda Item:	6.20.1.1

Document for:	Discussion and decision

# Introduction

This document is to kick off the below offline discussion:

- [AT110e][033][Other] Overheating (Huawei)

Scope: Treat R2-2005401, R2-2005404, R2-2005402, R2-2005403 (proponents are responsible to explain and drive)

Part 1: Identify agreeable changes. Deadline: June 4, 0700 UTC.

Part 2: For agreeable parts, continuation to agree CRs. Deadline: June 10, 0700 UTC

# Discussion

Based on the email discussion [Post109bis-e][050][TEI16] Overheating [1], the proposed conclusions are provided below.

Proposal 1 (7/7): In (NG)EN-DC, the new field for overheating assistance information in LTE RRC (TS36.331) refers to the NR OverheatingAssistance IE as specified in TS 38.331.

Proposal 2 (7/7): In (NG)EN-DC, if the new field for NR overheating assistance information encapsulated in LTE message is reported by the UE, the MN forwards this encapsulated information to the SN.

Proposal 3 (6/7): For Rel-15 legacy overheating IEs, the reducedCCsDL/UL (for (NG)EN-DC and NR-DC), reducedBW-FR1/FR2-DL/UL (for NR-DC) or reducedMIMO-LayersFR1/FR2-DL/UL (for NR-DC) are interpreted as for both MCG and SCG, the MN can indicate the allowed CC/BW/MIMO to the SN for coordination on reduced configuration.

Proposal 4 (6/7): In (NG)EN-DC, the interpretation for the Rel-15 legacy overheating IE (i.e. reducedCCsDL/UL) is not changed, i.e. it is always interpreted as the preference for both MCG and SCG.

Proposal 5 (7/7): In (NG)EN-DC, introduce a new UE capability in LTE capability container for the new field (i.e. overheating assistance information for SCG) in LTE assistance information message.

Proposal 6 (6/7): In (NG)EN-DC, MN determines the configuration for overheating assistance information for SCG. The configuration for the new overheating IE comes together with the configuration for the legacy overheating IE.

Proposal 7 (6/7): UE can report reducedMaxCCs in both legacy overheating IE (overheatingAssistance-r14) and new overheating IE (overheatingAssistanceForSCG-r16), reducedMaxCCs in legacy IE is intended for MCG+SCG, reducedMaxCCs in new IE is intended for only SCG.

Proposal 8 (5/7): SN indicating/suggesting whether to enable the new overheating information reporting for SCG is not supported in TEI16.

Proposal 9 (6/7): SN proposed value of reducedMaxCCs in CG-Config is not supported in TEI16.

Proposal 10 (7/7): The UE supporting new overheating assistance information for SCG shall also indicate support of legacy overheatingInd.

Proposal 11 (6/7): The reporting of new overheating field and legacy overheating field are controlled by one shared prohibit timer.

## Q1: Please provide the comments for the above proposals if any.

| Company | Comments |
| --- | --- |
| MediaTek | We are not fully aligned with majorities in some of proposals but fine to compromise. |
| Ericsson | We are fine with the proposals above. |

If the above proposals can be agreed, the associated CRs are provided in [2][3][4].

## Q2: Please provide the comments for the associated CRs in [2][3][4] if any.

| Company | Comments |
| --- | --- |
| MediaTek | R2-2005402 (36.331) <1> In 5.6.10.3, we should not refer to 5.7.4.3. The 38.331 5.7.4.3 is used for transmission of NR UEAssistanceInformation message. It is not suitable to have LTE RRC refer to this section. <2> For the overheating configuration – There is no need to use BOOLEAN. I think no present is the same as “FALSE”, thus ENUMERATED {true} is more suitable. R2-2005403 (38.331) <1> The way capture in current draft CR is inappropriate. We should have a new section in 38.331 (e.g. 5.7.4.3a Setting the contents of OverheatingAssistance) and the 36.331 should refer to this section on setting of the container. <2> In IE ConfigRestrictInfoSCG, you can combine the new fields with the previous R16 extension block and add suffix –r16 for that. |
| Ericsson | R2-2005402 (36.331) <1> We think some change is needed if we want to refer to 5.7.4.3 38.331. On transmission part therein we could have a condition: The UE shall submit the UEAssistanceInformation message to lower layers for transmission, except if it is configured in EN-DC. But we would also be fine to have a new clause dedicated for this case. <2> We agree with the point <2> raised by Mediatek above for R2-2005402 (36.331). R2-2005403 (38.331) <1> We think overheatingAssistanceSCG should not be under ConfigRestrictInfoSCG. The MN does not modify this message, thus it is simply what the UE reported. <2> Maybe not so nice that now we refer to OverheatingAssistance in allowedReducedConfigForOverheating, and now the IE is used in two different places but maybe not defined generically for the two cases (e.g. the field descriptions for OverheatingAssistance IE). In any case, it may also work that way so we are fine to keep the structure if companies would prefer to keep it. |

# Conclusion

To be added.

# Reference

[1] R2-2005401	Report for [Post109bis-e][050][TEI16] Overheating	Huawei, Huawei Device

[2] R2-2005402	36.331 CR for overheating in (NG)EN-DC and NR-DC

[3] R2-2005403	38.331 CR for overheating in (NG)EN-DC and NR-DC

[4] R2-2005404	36.306 CR for overheating in (NG)EN-DC and NR-DC