3GPP TSG-RAN WG2 Meeting #110	R2-200xxxx

Electronic, 1 June– 12 June2020

Agenda Item:	6.10.3

Source: 	OPPO

Title:  	Email report of [AT110e][050][DCCA] MAC updates (OPPO)

Document for:	Discussion and decision

# Introduction

This paper aims to trigger the email discussion for MAC update in DCCA WI.

- [AT110e][050][DCCA] MAC updates (OPPO)

Scope: Treat documents under 6.10.3, determine agreeable parts and and make agreements. Implement meeting agreements in updated CRs.

Part 1: Agreements (rapporteur sets the deadline)

Part 2: Updated Agreed CR 38321

Deadline: June 11 0700 UTC

# Open issues for MAC update

## CR 1: MAC running CR agreed in last RAN2 meeting

R2-2004582	Corrections on dormant BWP operation	OPPO, Nokia, Ericsson, Huawei	CR	Rel-16	38.321	16.0.0	0743	-	F	LTE_NR_DC_CA_enh-Core	R2-2004183

| Company | Comments |
| --- | --- |
| OPPO | This CR is in principle agreed in last RAN2 meeting and no changes compared with the agreed version in R2-2004183. |

## CR 2: Excluding AP-CSI in MAC

R2-2004813	Correction on CSI-RS measurement in dormant BWP	MediaTek Inc.	draftCR	Rel-16	38.321	16.0.0	F	LTE_NR_DC_CA_enh-Core

| Company | Comments |
| --- | --- |
| OPPO | I think the change in this CR is not necessary. I think it is up to RRC configuration to ensure that AP-CSI is excluded. And the MAC indicates the CSI measurement based on “if configured”. |

## CR 3: Dormant BWP configuration

R2-2004837	MAC and dormant SCell configuration	Nokia, Nokia Shanghai Bell	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

| Company | Comments |
| --- | --- |
| OPPO | For MAC spec changes: - For the first change, I think it is not necessary, it will be described in RRC spec. In MAC, we do not need to describe the configuration details. - For the second and third changes, we think it is not necessary. The current text is clear. |

## CR 4: Corrections on Activation/Deactivation of SCells procedure

R2-2005623	Correction on activating SCells into dormant or non-dormant BWP	vivo	CR	Rel-16	38.321	16.0.0	0757	-	F	LTE_NR_DC_CA_enh-Core

| Company | Comments |
| --- | --- |
| OPPO | All the changes is discussed in last RAN2 meeting in email discussion: [AT109bis-e][037][DCCA] MAC (OPPO). We think the changes are not necessary. For the following change: We dicussed this change in last RAN2 meeting. bwp-InactivityTimer is for each SCell, i.e. cell specific behavior. The text in 5.15 focus on the BWP level behaviour. We think it is bettwer to capature it in 5.9. For the following changes: In last RAN2 meeting, Huawei CR mentioned a similar change. We did not agree the change because we think the order of each action should not be changed. E.g. we should first active the scell, then we active the BWP. |

# Conclusions

# Reference

[1] R2-2004582	Corrections on dormant BWP operation	OPPO, Nokia, Ericsson, Huawei	CR	Rel-16	38.321	16.0.0	0743	-	F	LTE_NR_DC_CA_enh-Core	R2-2004183

[2] R2-2005241	Remaining issues on SCell dormancy behaviour	Huawei, HiSilicon	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

[3] R2-2005280	Resolve the issue with SRS for dormant SCell	Futurewei	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

the only change in MAC spec in above two CR is for the SRS transmission. So the two CR will not be included in the email discussion because we agree that SRS is not supported for dormant BWP.

[4] R2-2004813	Correction on CSI-RS measurement in dormant BWP	MediaTek Inc.	draftCR	Rel-16	38.321	16.0.0	F	LTE_NR_DC_CA_enh-Core

[5] R2-2004837	MAC and dormant SCell configuration	Nokia, Nokia Shanghai Bell	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

[6] R2-2005623	Correction on activating SCells into dormant or non-dormant BWP	vivo	CR	Rel-16	38.321	16.0.0	0757	-	F	LTE_NR_DC_CA_enh-Core