3GPP TSG-RAN WG2 Meeting#110-e	Draft_R2-200xxxx

Online, 1st -12th Jun. 2020

Agenda Item:	6.11.2

Source:	Huawei

Title:	Summary of UP open issues for Rel-16 UE power saving

Document for:	Discussion and Decision

# Introduction

This document is to summarise the following contributions submitted to AI 6.11.2 on UP remaining issues for Rel-16 UE power saving:

R2-2004428	Clarification on DCP configuration					Samsung		discussion

R2-2004642	Remaining issues for DCP						vivo			discussion

(Apart from Proposal 4 which has been discussed in TEI16)

R2-2004967	Correction on RAR and DCP monitoring				Nokia			draftCR

R2-2005125	Remaining issues on CSI report when DCP is configured		ZTE			discussion

R2-2005362	Remaining issues on DCP						LG			discussion

R2-2005418	Prioritization between DCP and RAR addressed to C-RNTI	Samsung		discussion

# Summary of proposals

## DCP for short DRX cycle

Regarding the support of DCP for short DRX cycle, the related proposals are:

| Company | Tdoc | Proposals |
| --- | --- | --- |
| vivo | R2-2004642 | Proposal 1: RAN2 to confirm RAN1 decision that PDCCH-WUS is not applicable for Short DRX cycle in Rel-16. Proposal 2: PDCCH-WUS only locates before onDuration timer for long DRX cycle and no PDCCH-WUS needs to be detected when UE is in short DRX cycle. |
| LG | R2-2005362 | Proposal 1. As RAN1’s final decision, i.e., the working assumption still stand, RAN2 should not support Short DRX cycle on DCP. |

Two companies propose to confirm that DCP is not supported for short DRX cycle. Please indicate whether you support the proposal and the specification impact, if any.

Discussion point 1. Companies are invited to provide comments on above proposal and indicate the specification impact, if any.

| Company name | Agree/Disagree | Comments |
| --- | --- | --- |
| OPPO | Disagree | This issue has been discussed in both RAN1 and RAN2, and so far: - According to RAN1 running CR DCP is only supported for long DRX cycle, but RAN1 has not concluded there are any technical feasibility concerns to support DCP for short DRX. - RAN2 previously agreed to support DCP for both short and long DRX and it can be configurable by the network. In previous discussion, some companies have concern that problematic combinations of DCP configuration (e.g. ps_offset), and short DRX configurations may occur. But NW could be aware of it, and configure whether to support DCP to short DRX cycle. In addition, there is limit impact to the current RAN1 specification if DCP can be supported for short DRX cycle. So we prefer to support DCP for short DRX cycle and it can be configurable by the network |
| Nokia | Disagree | RAN2 has already agreed to support DCP for short DRX. RAN1 has not indicated any related concerns. |
| ERI | Disagree | We agree with the Nokia comment, that RAN1 did not agree on any technical concerns, and that from a RAN2 perspective short DRX cycle is supported: - LS to RAN1 (R2-1914060): - From RAN2 perspective, support WUS for both short and long DRX and it should be configurable by the network. - RAN2 would like to ask RAN1 if there are technical feasibility concerns to support WUS for short DRX. |
| CATT | Agree | These proposals re-state/confirm the current status after last meeting's discussions. This issue was extensively discussed at last meeting as part of and following offline [Post109e#41] [PowSav] DCP open issues and no agreement could be reached in the end that DCP should be supported for short DRX, and no associated FFS is captured in Chairlady’s report. And the logical outcome is that the above proposals are already captured in the specifications via the CR in R2-2003975 which was reviewed and endorsed without any editor’s note capturing any FFS about this. RAN1 specifications have also been further updated to reflect this outcome. Therefore this discussion is closed from our perspective and we do not see the point of re-discussing this issue. |
| ZTE | Agree | Share the same view with CATT |
| Xiaomi | Disagree | This has been discussed many many times… We can just follow the majority views from the previous discussion that if NW think it is problematic, it just does not configuring DCP for short DRX for simplicity. |
| vivo | Agree | We share the same view as CATT. This issue has a so long discussion in both RAN1 and RAN2. It is very clear that there is no consensus in RAN1/RAN2 on the technique feasibility. We don’t see any necessity to repeat the discussion. |
| LG | Agree | We think that RAN2 should not support Short DRX cycle on DCP as following RAN1’s working assumption. |
| Lenovo | Agree | Same view as CATT |
| Apple | Agree | Share the same view as CATT |
| Qualcomm | See comment | We support DCP for short DRX cycle, which is also RAN2’s standing agreement. But clearly RAN1 is the bottleneck, i.e. if RAN1 does not change their working assumption, the feature can’t be implemented even if we RAN2 agree to support it. Therefore, we think RAN2 should not discuss the issue again until RAN1 makes a change to their working assumption. |
| Huawei, HiSilicon | See comment | Same view as Qualcomm. RAN2 has already agreed that short DRX cycle should be supported from RAN2 perspective. But RAN1 cannot provide any technique concern. At the same time they did not confirm their WA explicitly. We also think we cannot make any censuses unless RAN1 makes progress on their WA. Thus we propose not to discuss this in RAN2 again before RAN1 further progress on this issue. |
| MediaTek | Yes | As it is clear that RAN1 cannot reach consensus on this aspect, we have to take the safe approach and assume that DCP is not supported for short DRX |
| Samsung | Agree | We have assumed that the reason RAN1 made no consensus on it is that the feasibility is unclear and the expected gain is not significant. |
| Intel | Agree | We share the same view as CATT. |

Summary:

15 replies: - 9 companies agree to confirm that DCP is not supported for short DRX cycle. - 4 companies disagree and 2 companies think that we should not further discuss this issue before RAN1 makes further conclusion on their working assumption. Although it is slightly majority view to confirm that DCP is not supported for short DRX cycle (9 v.s. 6), given that we have discussed this for very long time and there is no new argument or technique points in the comments (all about RAN1/2 situation, i.e. RAN2 has agreed on the benefit but RAN1 cannot reach consensus, etc.), we do not think easy agreement can be made without further input from RAN1. Thus, we suggest not to discuss this again in RAN2 before further RAN1 input. Proposal 1: Do not further discuss DCP for short DRX cycle before further RAN1 input.

## UE behaviour when DCP overlaps with RAR

Regarding the UE behaviour when DCP overlaps with RAR, the related proposals are:

| Company | Tdoc | Proposals |
| --- | --- | --- |
| Nokia | R2-2004967 | Draft CR with the following TP: 1> if the Long DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset: 2> if DCP is configured for the active DL BWP: 3> if DCP indication associated with the current DRX Cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX Cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or within BWP switching interruption length, or during a measurement gap, or within ra-ResponseWindow or msgB-ResponseWindow; or 3> if ps-Wakeup is configured with value true and DCP indication associated with the current DRX Cycle has not been received from lower layers: 4> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe. |
| Samsung | R2-2005418 | Proposal: The prioritization between DCP and RAR addressed to C-RNTI is under NW control. There is no impact on RAN2 specifications. |

Regarding the UE behaviour when DCP overlaps with RAR, RAN2 has sent LS to RAN1 to confirm RAN2 understanding that RAR addressed all RNTIs is prioritized over DCP by the UE and ask where to capture the behaviour and the issue is under discussion in RAN1. Thus, it seems we should wait for RAN1 feedback before further discussion.

Discussion point 2. Companies are invited to provide comments on above proposals and indicate whether we should wait for RAN1 reply first.

| Company name | Wait for RAN1? | Comments |
| --- | --- | --- |
| OPPO | Yes | We should wait for RAN1 response. |
| Nokia | Yes | We can wait RAN1, but if RAN1 cannot conclude RAN2 can agree R2-2004967. Samsung proposal affects legacy random access handling in the network side which should be avoided. |
| ERI | Yes, but not indefinitely | RAN1 should discuss and conclude on this topic this week. |
| CATT | Yes | We sent an LS for this purpose and so should wait for the LS reply. |
| ZTE | Yes |  |
| Xiaomi | Yes | Yes, we wait for RAN1 response. |
| vivo | Yes | As far as I know, RAN1 already made some progress. We can just wait for RAN1 conclusion. |
| LG | Yes | After RAN1 checking, RAN2 can discuss this subsequently. |
| Lenovo | Yes |  |
| Apple | Yes | Yes, we can wait for RAN1 response |
| Qualcomm | Yes | We should wait for RAN1 response on this issue. |
| Huawei, HiSilicon | Yes | We should wait for RAN1 response. |
| MediaTek | Yes |  |
| Samsung | No | We have identified that RAN1 discussed it yesterday, but reached no agreement. Furthermore, no plan for further discussion. Therefore, there is no reason for RAN2 to wait for RAN1 input. According to current TS 38.213 prioritization rules, if DCP collides with RAR addressed to C-RNTI (e.g. during BFR) and the search spaces are not quasi-collocated, DCP will be prioritized as it is type-3 CSS and thus impacting legacy RAR behaviour is correct. The BFR search space can be either type-3 CSS or USS. If BFR search space is type-3 CSS, priority between DCP and RAR addressed to C-RNTI is determined based on the SearchSpace-Id. It means the prioritization can be handled by gNB implementation, and RAN2 need not specify something. |
| Intel | Yes | We share the view about waiting for RAN1’s response |

Summary:

15 replies: - 14 companies think we should wait for RAN1 response. - 1 company indicates that RAN1 cannot get agreement and there will not be response. During the discussion, RAN1 has replied the previous RAN2 LS as following []: RAN1 would like to thank RAN2 on DCP open issues and informing RAN1 of the RAN2 agreements. RAN1 discussed the issue on the collision of DCP and RAR when a DCP monitoring occasion overlaps with the ra-ResponseWindow or msgB-ResponseWindow. Regarding RAN2’s understanding on RAR impact with the collision of DCP and RAR when the corresponding search spaces are not quasi-collocated, RAN1 understanding is: - RAR addressed to C-RNTI would not be impacted if the DCI scheduling RAR addressed to C-RNTI is configured on type-3 CSS with lower index than that of DCP, according to TS38.213; - RAR addressed to C-RNTI would not be prioritized over DCP if it is configured on USS or on type-3 CSS with higher index than that of DCP. RAN1 could not reach a consensus on whether to always prioritize RAR addressed to C-RNTI over DCP by RAN1 specification change or to leave it to be handled by network implementation. Considering that RAN1 has replied the LS and the reply was not taken into consideration in almost all above comments, we suggest to discuss the issue online according to RAN1 reply. It seems there are three options on the table when DCP overlaps with RAR window: - Option 1: DCP is not monitored in ra-ResponseWindow and msgB-ResponseWindow (proposed in R2-2004967) - Option 2: Confirm that the prioritization between DCP and RAR addressed to C-RNTI is under NW control. There is no impact on RAN2 specifications (proposed in R2-2005418) - Option 3: Wait for further RAN1 progress (not in this meeting) Proposal 2: Discuss the following options regarding UE behaviour when DCP overlaps with RAR online: - Option 1: DCP is not monitored in ra-ResponseWindow and msgB-ResponseWindow (proposed in R2-2004967) - Option 2: Confirm that the prioritization between DCP and RAR addressed to C-RNTI is under NW control. There is no impact on RAN2 specifications (proposed in R2-2005418) - Option 3: Wait for further RAN1 reply (not in this meeting)

## Dormancy behaviour when DCP is missed

The related proposal is:

| Company | Tdoc | Proposals |
| --- | --- | --- |
| LG | R2-2005362 | Proposal 2. RAN2 confirms that DCP has no impact on dormancy behaviour and no change in RAN2 specfication is needed. |

The agreements made in RAN1#100bis-e [1] are list as following for reference:

Conclusion - No consensus to add additional clarifications on UE behavior For SCell(s) configured with dormant BWP, when DCI 2-6 is not detected - UE continues on same dormant/non-dormant BWP until an indication (L1 or RRC) to change BWP is detected or (if applicable) BWP inactivity timer expires - Note: No TP required - Discuss further if TP is required and finalize in next phase

Discussion point 3. Companies are invited to provide comments on above proposal and indicate the specification impact, if any.

| Company name | Agree/Disagree | Comments |
| --- | --- | --- |
| OPPO | Agree | We agree with the above proposal. No need to change RAN2 spec. |
| Nokia |  | We need to wait RAN1 to capture above agreements in their specifications. Possible MAC changes can be considered later after RAN1 specs are ready for this. |
| ERI | Agree | We agree that there is no impact on RAN2 specification, and we also think that SCell dormancy should not be visible in MAC. |
| CATT | Agree | This issue has been discussed at each e-meeting since RAN2#109 with always the same outcome that it is rather a RAN1 issue with no impact on RAN2 power saving session. |
| ZTE | Agree |  |
| Xiaomi | Agree | No need to capture in RAN2 spec. |
| vivo | Agree | We also agree that there is no RAN2 impact. |
| LG | Agree | According to RAN1 conclusion, UE keeps staying same dormant/non-dormant BWP. So, DCP has no impact on dormancy behaviour and no change in RAN1 specification is needed. |
| Lenovo | Agree | No RAN2 impact foreseen |
| Apple | Agree |  |
| Qualcomm | Agree |  |
| Huawei, HiSilicon | Agree |  |
| MediaTek | Agree |  |
| Samsung | Agree |  |
| Intel | Agree |  |

Summary:

15 replies: - 14 companies agree to confirm that DCP has no impact on dormancy behaviour and no change in RAN2 specification is needed. - 1 company thinks MAC changes can be considered after RAN1 specification is ready. There is a majority view to confirm that DCP has no impact on dormancy behaviour and no change in RAN2 specification is needed. Proposal 3: Confirm that DCP has no impact on dormancy behaviour and no change in RAN2 specification is needed.

## CSI report within DRX onDuration

The related proposals are:

| Company | Tdoc | Proposals |
| --- | --- | --- |
| ZTE | R2-2005125 | Proposal 1: Confirm whether the following operation is the expected behaviour: - For one onduration period, if the drx-ondurationTimer will not be started based on the DCP received, but the onduration period will be considered as active time due to the pending SR or any other cases, the CSI should be reported no matter CSI masking is setup or not. Proposal 2: if proposal 1 is not agreed, RAN2 is kindly asked what is the correct UE behavior on CSI reporting under above issue - Since the drx-ondurationTimer is not started for the onduration period, the ps-TransmitPeriodicL1-RSRP and ps-TransmitOtherPeriodicCSI is applicable to this case. - Since the onduration period is considered as active time but the drx-ondurationTimer is not running, if CSI masking is not setup, the CSI shall be reported; otherwise, the CSI shall not be reported. |

We think similar issue has been discussed in RAN2#109-e [2]. Corresponding summary and proposal are listed as following for reference:

- Issue #1: Capturing CSI reporting when the drx-onDurationTimer is not started due to DCP indication, but the MAC entity is in Active Time during on-duration due to other reasons Company/Tdoc: CATT Error: Reference source not found Proposed solution: When evaluating Active Time when drx-onDurationTimer is not started due to DCP, the same triggers, with same ambiguity period (4ms) as in legacy should be taken into account. … Proposal 1 (10/12): The issue of capturing CSI reporting when the drx-onDurationTimer is not started due to DCP indication, but the MAC entity is in Active Time during on-duration due to other reasons will be addressed in MAC specification.

Discussion point 4. Do you support to confirm the understanding in proposal 1? If not, please indicate your preference between the two options in proposal 2.

| Company name | Yes/No, which option | Comments |
| --- | --- | --- |
| OPPO | Yes, option3 | According to 38.321, if CSI masking is configured, whether UE reports CSI on PUCCH depends on whether drx-onDurationTimer would be running or not. Since DCP will have an impact on the start of drx-onDurationTimer, DCP should also be considered as a factor when evaluating whether drx-onDurationTimer is running if CSI masking is configured. So for the case the onduration period is considered as active time but the drx-onDurationTimer is not running, if CSI masking is setup, the CSI shall not be reported; otherwise, the CSI shall be reported. |
| Nokia | Yes | We think that it would be beneficial to have CSI reporting also in case where on-duration timer is not running. In case the drx-ondurationTimer is not started for the onduration period, the ps-TransmitPeriodicL1-RSRP and ps-TransmitOtherPeriodicCSI should be applicable to this case. This seems to require MAC spec changes as follows: 1> if DCP is configured for the active DL BWP; and 1> if the current symbol n occurs within drx-onDurationTimer duration; and 1> if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause; and 2> if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7]; 3> not report semi-persistent CSI configured on PUSCH; 2> if ps-Periodic_CSI_Transmit is not configured with value true: 3> if ps-TransmitPeriodicL1-RSRP is not configured with value true: 4> not report periodic CSI on PUCCH. 3> else: 4> not report periodic CSI on PUCCH, except L1-RSRP report(s). |
| ERI |  | Our reading of current 38.321 is that when DCP is configured, and drx-OnDurationTimer is not started and the UE is in Active Time due to other activity, then CSI is not reported when CSI-mask is set, i.e. you end up in the green part below (one of the “and” conditions in the blue part is not met): 1> if DCP is configured for the active DL BWP; and 1> if the current symbol n occurs within drx-onDurationTimer duration; and 1> if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause; and 1> if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause: 2> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7]; 2> not report semi-persistent CSI configured on PUSCH; 2> if ps-TransmitPeriodicL1-RSRP is not configured with value true: 3> not report periodic CSI that is L1-RSRP on PUCCH. 2> if ps-TransmitOtherPeriodicCSI is not configured with value true: 3> not report periodic CSI that is not L1-RSRP on PUCCH. 1> else: 2> in current symbol n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7]; 3> not report CSI on PUCCH and semi-persistent CSI configured on PUSCH. 2> if CSI masking (csi-Mask) is setup by upper layers: 3> in current symbol n, if drx-onDurationTimer would not be running considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause: 4> not report CSI on PUCCH. When DCP is configured, and drx-OnDurationTimer is not started then periodic CSI is reported is determined by the reporting flags (ps-TransmitPeriodicL1-RSRP and ps-TransmitOtherPeriodicCSI) and the CSI-mask is ignored. PS: about the stage 3 proposal by Nokia above, i.e. does it really change the behaviour when the indentation is changed? (PS: there is still an “and” in the 3rd 1> bullet)? |
| CATT | Yes | We confirm that proposal 1 in ZTE’s R2-2005125 is the correct understanding, and is also independent on the ps-TransmitPeriodicL1-RSRP and ps-TransmitOtherPeriodicCSI configurations. This is because: - CSI masking only controls CSI reporting outside the on-duration - CSI shall be reported within an on-duration interval if MAC is in Active Time independently of ps-TransmitPeriodicL1-RSRP and ps-TransmitOtherPeriodicCSI configurations per the above proposal 1 in R2-2001913 mentioned by the rapporteur, and further agreed as agreement #4 in RAN2#109-e. And it is our understanding that it is already captured in the specification because, when symbol n is within any on-duration interval (using coloured text from above Ericsson’s input): - the blue text is skipped because MAC is in Active Time - the yellow text is skipped because MAC is in Active Time - the green text is skipped irrespective of DCP, because DCP is not mentioned in the list of causes considered when assessing whether the timer is running or not. And if all three text blocks are skipped, then CSI is indeed reported, as expected. Regarding Nokia’s TP, we share Ericsson’s view that there seems to be a typo related to the “and” before the indentation. But anyways, based on the above understanding, we don't think anything is needed. |
| ZTE | Yes | For this issue, since each company may have different understanding on the expected UE behavior ,we would like to confirm it in such last stage discussion. For simplicity. we can separate this issue into the following two subissues: - Subissue 1: Whether the csi-Mask is applicable in the case that the drx-ondurationTimer is not running by the indication of DCP - Subissue 2: If the answer to the subissue 1 is positive, whether the csi reporting flag (i.e ps-TransmitPeriodicL1-RSRP and ps-TransmitOtherPeriodicCSI ) is applicable in order to make UE report the corresponding CSI. If sub-issue 1 is not agreed, the current specification shall be maintained. If sub-issue 1 is agreed but sub-issue 2 is not, we will go for the second choice in proposal 2. If sub-issue 2 is agreed, we will go for the first choice in proposal 2. As a network vendor, since UE is already in active status, we think it is beneficial for NW to schedule grant/assignment if UE have a valid CSI measurement results to report. Thus we slightly prefer to keep the current specification, which is negative to the subissue 1. |
| Xiaomi |  | We think the CSI masking only controls CSI reporting when the drx-onDurationTimer is running. In this case, the drx-OnDurationTimer is not started and the UE is in Active Time due to other activity, then CSI is not reported when CSI-mask is set; otherwise, the CSI shall be reported. So the second bullet of ZTE’s proposal 2 is in line with our understanding. “ Since the onduration period is considered as active time but the drx-ondurationTimer is not running, if CSI masking is not setup, the CSI shall be reported; otherwise, the CSI shall not be reported.” |
| vivo | Yes | We would like to first confirm the understanding in P1 in ZTE contribution R2-2005125. In our understanding, CSI masking only controls CSI reporting outside the on-duration according to current specification. For the following condition: (Quote from the proposal) For one onduration period, if the drx-ondurationTimer will not be started based on the DCP received, but the onduration period will be considered as active time due to the pending SR or any other cases, According to current specification, (using coloured text from above Ericsson’s input): The blue part will be skipped due to active time; The yellow part will also be skipped due to active time; The green part will be skipped because drx-onDurationTimer is not running. Then CSI will be reported. In our understanding, CSI report is needed in this case. |
| LG | No | According to the current specification, when CSI masking is configured, CSI is not reported if drx-onDurationTimer is not started but the onDuration period is in Active time. Note that CSI is not reported only one onDuration period and can be reported on next onDuration period. So, we think there is no critical issue even if the network cannot receive CSI reporting for one onDuration period. |
| Lenovo | Yes | In our understanding proposal 1 is the correct behaviour. We see benefits to have CSI reporting also for cases where on-duration timer is not running, but the UE is in Active Time due to other reasons. |
| Qualcomm | See comment | We agree the intention of Proposal 1 but do not think it is accurately enough. - CSI masking does not matter in this scenario, because it controls CSI reporting only outside on duration period; - ps-TransmitPeriodicL1-RSRP and ps-TransmitOtherPeriodicCSI matter only when UE is not in active time during on duration period. If UE is in active time during on duration period, that “4ms paragraph” controls CSI reporting. In summary, we think the existing spec is fine and no change is needed. |
| Huawei, HiSilicon | Yes | Same view as CATT and Qualcomm. We believe P1 is the intended behaviour and has been captured in the current specification. |
| MediaTek | Yes | Agree with QC that this is the behaviour captured in the specs |
| Samsung | Yes | Share with OPPO’s view. |
| Intel | Yes | We share the view described by CATT and Qualcomm. |

Summary:

14 replies on this discussion point, but some replies are not clear to us. The proposal 1 from ZTE’s contribution is to confirm: for one onduration period, if the drx-ondurationTimer will not be started based on the DCP received, but the onduration period will be considered as active time due to the pending SR or any other cases, the CSI should be reported no matter CSI masking is setup or not. Some companies say “yes” but according to the comments they seem have different understanding with the proposal 1. We tried to summarise based on the comments as below: For one onduration period, if the drx-ondurationTimer will not be started based on the DCP received, but the onduration period is Active Time due to the pending SR or any other cases: - 8 companies think that CSI should be reported independently of the two CSI flags. Those companies also think that this is the UE behaviour captured in the current specification. - 4 companies think that it depends on csi-Mask, i.e. if csi-Mask is setup, the CSI shall not be reported; otherwise: - 3 companies think that the CSI shall be reported. - 1 company thinks the report is controlled by the two flags. - 1 company thinks that in this case the report should be controlled by the two CSI flags. - 1 company thinks CSI should not be reported. There is a majority view to confirm that for one onduration period, if the drx-ondurationTimer will not be started based on the DCP received, but the onduration period will be considered as active time, the CSI should be reported no matter CSI masking is setup or not (the UE behaviour captured in the current specification). Proposal 4: Confirm that for one onduration period, if the drx-ondurationTimer will not be started based on the DCP received, but the onduration period will be considered as active time, the CSI reporting should not be impacted by whether CSI masking is configured or not. No change to the current specification is needed.

## Proposal on CR optimisation

The proposals related to CR optimisation are summarised in this section.

Proposal on the condition for DCP in MAC CR:

| Company | Tdoc | Proposals |
| --- | --- | --- |
| Samsung | R2-2004428 | Proposal 1: Update the condition 'if DCP is configured for the active DL BWP' in MAC to 'if the MAC entity is configured with dcp-Config, and the active DL BWP of SpCell is configured with the search space for DCI format 2_6' merely for clarity. TP: When DRX is configured, the MAC entity shall: … 1> if the Long DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset: 2> if the MAC entity is configured with dcp-Config, and the active DL BWP of SpCell is configured with the search space for DCI format 2_6: - 3> if DCP indication associated with the current DRX Cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or - 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX Cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or within BWP switching interruption length, or during a measurement gap; or - 3> if ps-Wakeup is configured with value true and DCP indication associated with the current DRX Cycle has not been received from lower layers: 4> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe. 2> else: - 3> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe. … 1> if the MAC entity is configured with dcp-Config, and the active DL BWP of SpCell is configured with the search space for DCI format 2_6; and 1> if the current symbol n occurs within drx-onDurationTimer duration; and |

Discussion point 5a. Companies are invited to provide comments on above TP in R2-2004428 on MAC CR.

| Company name | Agree/Disagree | Comments |
| --- | --- | --- |
| OPPO | Agree | We agree with the above TP. |
| Nokia | Disagree | We think that current specification is already clear. |
| ERI | Disagree | We do not see a strong need for this clarification, but we can go with majority view, if needed. |
| CATT | Disagree | DCP means DCI with CRC scrambled by PS-RNTI. This is sufficient information for a reader to find in 38.213 what means "DCP is configured for the active DL BWP". Maybe adding reference to the appropriate 38.213 section (10.3) could make sense. |
| ZTE | Have no strong point of view |  |
| Xiaomi | Disagree | The current specification is already clear. |
| vivo | Disagree | We also think the current specification is clear enough. |
| LG | Agree with the first part, Disagree with the second part | We agree with the first part, i.e., the MAC entity is configured with dcp-Config. However, regarding the second part, the MAC specification generally does not indicate DCI format and just refers physical specification. Therefore, it would be good to refer physical specification, e.g., the active DL BWP of SpCell is configured for DCP as specified in TS 38.213 [6]. |
| Lenovo |  | We don’t have a strong opinion but think that the current specification is sufficiently clear. |
| Apple | Disagree | The current specification is already clear |
| Qualcomm | No strong view | We have the same comment as LG |
| Huawei, HiSilicon | Disagree | We think the current specification is clear thus we prefer to keep the current wording unless there is critical issue. |
| MediaTek | Disagree | We prefer the current specification as it is clear enough |
| Samsung | Agree (fine with LG’s TP) | It should be noted that the current wording is incorrect, as dcp-Config is configured per MAC entity, while the current wording is saying that ‘DCP is configured for the active DL BWP’. So, we think the text should be corrected/improved anyway. Regarding the actual TP, we now tend to agree with LG, and are fine with suggestion from them. |
| Intel | No strong view | We are ok with current TP but would also be open to update it as suggested by LG. |

Summary:

15 replies where 3 companies agree with the TP and 8 companies disagree. 4 companies have no strong view.Those companies also think that this is the UE behaviour captured in the current specification. There is a majority view that the proposed TP is not needed.

Proposal on PHY/MAC alignment about invalid DCP MO:

| Company | Tdoc | Proposals |
| --- | --- | --- |
| vivo | R2-2004642 | Proposal 3: RAN2 to discuss whether alignment between PHY and MAC about invalid DCP MO is needed: - TS 38.213 to remove the invalid DCP MO cases captured in MAC (Condition 2 in MAC) |

Discussion point 5b. Companies are invited to provide comments on above proposal in R2-2004642.

| Company name | Agree/Disagree | Comments |
| --- | --- | --- |
| OPPO | Disagree | It is RAN1 scope and it is up to RAN1 to decide whether the update is nedded. |
| Nokia | Disagree | MAC spec includes conditions when on-duration timer is started hence we can add reference to 38.213 to cover invalid DCP MO cases. |
| ERI | Agree | We think that RAN1 and RAN2 ended up with a strange cross-referencing scenario, i.e. PHY refers to MAC when to send a wake-up indication to MAC: 38.213: If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next DRX cycle, or does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next DRX cycle the UE shall send the Wake-up indication value “1” to higher layers start the drx-onDurationTimer for the next DRX cycle. 38.321: 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX Cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or within BWP switching interruption length, or during a measurement gap; or We are open to discuss how this can be done in the best way, or most agreeable way: - We tend to agree with vivo, i.e. we do not see the point why PHY needs to refer to MAC, i.e. we think the best way is to remove the yellow part in 38.213. - In case we consider a clarification in MAC, then we can try to clarify that bullet 3 is one case of invalid DCP monitoring occasion. But neither in PHY nor MAC the wording of valid/invalid PDCCH monitoring occasion for DCP is currently used, i.e. not sure how we achieve a good connection. The suggestion that MAC refers to PHY for bullet 3 would only increase the unnecessary referencing. The reference would also have to be specific for the sentence/paragraph, not just a section. |
| CATT | Agree although no major issue and nothing is broken. | We acknowledge the below yellow text in 38.213: If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle the physical layer of the UE reports a value of 1 for the Wake-up indication bit to higher layers for the next long DRX cycle. is redundant with the below yellow text in 38.321: 2> if DCP is configured for the active DL BWP: 3> if DCP indication associated with the current DRX Cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX Cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or within BWP switching interruption length, or during a measurement gap; or 3> if ps-Wakeup is configured with value true and DCP indication associated with the current DRX Cycle has not been received from lower layers: 4> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe. However, we don't think this is a major issue and the system does not break. We are open though to suggest RAN1 to remove the text “, and in Clause 5.7 of [14, TS 38.321]”. |
| ZTE | Have no strong point of view | Since we can not find any technical issue from this redundant description between RAN2 and RAN1 spec. Maybe we can leave it as it is. There is no need for us to ask RAN1 to modify their specification if there is no any technical issue can be found. |
| Xiaomi |  | Updating the 38.213 is not in the scope of RAN2. But: We agree that it seems that with the latest TS38.213 and TS38.321, there are some overlap conditions between PHY and MAC about the DCP invalid cases. A little bit different view from CATT is that we think the yellow highlight part in 38.213(clauses 12) has already captured the invalid DCP MO case of BWP switching and measurement gap which has already been captured in MAC spec : If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle the physical layer of the UE reports a value of 1 for the Wake-up indication bit to higher layers for the next long DRX cycle. The below yellow text in 38.321: 2> if DCP is configured for the active DL BWP: 3> if DCP indication associated with the current DRX Cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX Cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or within BWP switching interruption length, or during a measurement gap; or 3> if ps-Wakeup is configured with value true and DCP indication associated with the current DRX Cycle has not been received from lower layers: 4> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe. So we are wondering this redundant part in MAC can be removed since an indication will be received from PHY. While for the invalid DCP MO due to overlap with active time, since RAN1 specification does not capture the relation between Active Time and the ambiguous period, i.e. “4 ms prior to…” is not considered in TS38.213, it can be kept as it is. |
| vivo | Agree | Agree with the above views from Ericsson and CATT. Our suggestion for RAN1 is to remove the text “, and in Clause 5.7 of [14, TS 38.321]” to resolve this strange cross-referencing scenario and avoid misleading/redundant description. |
| LG | Disagree | We have no problem with the redundant descriptions specified in both PHY and MAC specification. |
| Lenovo | Agree | Agree with Ericsson |
| Qualcomm | No strong view | We have the same comment as CATT |
| Huawei, HiSilicon | Disagree | Similar view as CATT and ZTE. We think the current specification (both RAN2 and RAN1) can work. Considering that we had very long discussion on the modelling in both RAN1 and RAN2, we prefer to keep the current specification unless critical technique issue has been identified. |
| MediaTek | Agree | Agree with Ericsson |
| Samsung | No strong view | It seems slightly strange for RAN2 to decide the update on RAN1 specification. On the other hand, if needed for clear understanding, RAN2 can ask it to RAN1. |
| Intel | No strong view | We understand that RAN1 text should be discussed/updated, if needed, by RAN1; therefore. at most RAN2 could mainly provide a recommendation. |

Summary:

14 replies where 5 companies agree that TS 38.213 should remove the invalid DCP MO cases captured in MAC and 4 companies disagree. 4 companies have no strong view. 1 company proposes to update RAN2 spec. This proposal is about RAN1 specification and there is no majority view on whether to send LS to RAN1. Proposal 5: FFS: send LS to ask RAN1 to remove “, and in Clause 5.7 of [14, TS 38.321]” in TS 38.213.

# Proposed wayforward

Proposal 1: Do not further discuss DCP for short DRX cycle before further RAN1 input.

Proposal 2: Discuss the following options regarding UE behaviour when DCP overlaps with RAR online:

- Option 1: DCP is not monitored in ra-ResponseWindow and msgB-ResponseWindow (proposed in R2-2004967)

- Option 2: Confirm that the prioritization between DCP and RAR addressed to C-RNTI is under NW control. There is no impact on RAN2 specifications (proposed in R2-2005418)

- Option 3: Wait for further RAN1 progress (not in this meeting)

Proposal 3: Confirm that DCP has no impact on dormancy behaviour and no change in RAN2 specification is needed.

Proposal 4: Confirm that for one onduration period, if the drx-ondurationTimer will not be started based on the DCP received, but the onduration period will be considered as active time, the CSI reporting should not be impacted by whether CSI masking is configured or not. No change to the current specification is needed.

Proposal 5: FFS: send LS to ask RAN1 to remove “, and in Clause 5.7 of [14, TS 38.321]” in TS 38.213.

# References

- Final_Minutes_report_RAN1#100b-e_v100

- R2-2001913, “Summary of open issues for PDCCH”, CATT, RAN2#109-e

- R2-2006098, “LS Reply on DCP Open Issues”, RAN1, RAN2#110-e