3GPP TSG-RAN WG2 #110-e	Draft R2-200xxxx

Online, 1 – 12 June, 2020

Agenda item:	6.4.3.1

Source: 	LG Electronics Inc. (Rapporteur)

Title: 	[Offline Disc#704] Identified proposals to V2X MAC open issues

Document for:	Discussion and decision

# Introduction

This document is the trigger of offline discussion #704

- [AT110-e][704][V2X] MAC issues (LG)

Discuss and conclude MAC issues in R2-2005725 (in R2-2005956).

Deadline is 6/8 10:00am (UTC).

In this document, the number of issues in R2-2005725 are reused. The issues which can be discussed during CR implementation are excluded and will be discussed in a separate email discussion on the MAC CR.

# Offline discussion

## Issue 1: Remaining issues on UL/SL Prioritization

The related proposals are also available below:

| Company | Tdoc | Proposals |
| --- | --- | --- |
| Apple | R2-2004759 | Proposal 5 When UE is under SL incapable RAN node, UE uses LTE V2X method for UL/SL prioritization, i.e., only emergency call and MSG1/MSG3 in RACH gets prioritized. |

As been agreed in RAN2#1090e, the case which the RAN node is not upgraded yet has to be supported in NR V2X. In this case, RAN node does not support the SL configuration due to reasons like the RAN node is not upgraded yet.

Agreements on MAC - RAN may not always provide SL configuration/function to UE e.g. when the RAN node is not upgraded yet.

With such understanding, R2-2004759 proposes that RAN2 should have discussion on how to make a proper configuration to solve the prioritization problem. It was proposed that explicit condition is needed to be specified in the MAC specification, when SL UE in under SL incapable.

However, according to 38.321, if no threshold for prioritization is configured, UE does not perform threshold-based prioritization. Also, in 5.4.2.2, regardless of whether UE is under SL incapable RAN node, the MAC entity shall prioritize RACH over SL at all times. In addition, in 5.22.1.3.1, SL cannot be prioritized when UL transmission is prioritized by upper layers. Such UL prioritization over SL is not configurable, but already specified in 38.321 without any configuration.

Observation 1A: According to 38.321, if no threshold for prioritization is configured, threshold-based prioritization is not applied, but RACH and emergency call are prioritized over sidelink.

### Question 1A: Do we need to specify something more than the existing specification to take into account the prioritization case when UE is under SL incapable RAN node?

- Option A1: No, the current specification is enough. i.e. when UE is under SL incapable RAN node, UE uses LTE V2X method for UL/SL prioritization, i.e., only emergency call and MSG1/MSG3 in RACH gets prioritized.

- Option A2: Yes (explain what we should additionally specify, if selected)

- Option A3: In case the threshold(s) are not configured by an SL incapable RAN node, the NR UL is always prioritized over LTE/NR SL TX

| Company | Preferred Option | Comment |
| --- | --- | --- |
| OPPO | A1 with comment | But besides “only emergency call and MSG1/MSG3 in RACH gets prioritized.”, the threshold based solution also works since the threshold(s) are also in pre-configuration. |
| HW | A3 | Firstly we still think for UEs under SL incapable RAN node, they can rely on pre-config UL and SL thresholds and follow NR UL/SL prioritization rule. However if the majority anyway persists the existence of such case that even pre-configuration does not provide such thresholds, then we would like to support Option A3, i.e., the NR UL is always prioritized over LTE/NR SL TX. This option is more aligned with the logic of NR UL/SL prioritization rule i.e. to avoid the impact to Uu as much as possible. Actually the correction for A3 is quite simple and straightforward as shown below. The transmission of the MAC PDU is prioritized over uplink transmissions of the MAC entity or the other MAC entity if the following conditions are met: 1> if the MAC entity is not able to perform this sidelink transmission simultaneously with all uplink transmissions at the time of the transmission, and 1> if uplink transmission is neither prioritized as specified in clause 5.4.2.2 nor prioritized by upper layer according to TS [24.386] [xx]; and 1> if sl-PrioritizationThres is configured and if the value of the highest priority of logical channel(s) and a MAC CE in the MAC PDU is lower than sl-PrioritizationThres. Via this change, it is for sure that NR SL will not be prioritized over Uu when threshold is not configured and thus will be dropped. Also note that the content of LTE SL vs. NR UL prioritization will be removed, as the case is confirmed by companies as not existing. So no need to consider that situation. However, if the majority assumes pre-configuration can avoid, then we may not further need to discuss this issue. |
| Interdigital | A3 | Agree with Huawei |
| Samsung | A3 with comment | Regardless of SL incapable RAN node, if threshold is configured then UE should determine SL/UL priority based on the threshold. Otherwise UL transmission should be prioritized. |
| Futurewei | A3 | We should follow the principle that Uu transmission is prioritized unless a clear rule exists to prioritize SL |
| Intel | A3 | We agree with Huawei’s view in that we can follow Uu rule in this case, although we are not sure how likely this scenario is (i.e. pre-configuration not providing prioritization thresholds) |
| MediaTek | A3 | Agree with HW. This issue may be solved by the threshold specified in pre-configuration. If not, NR UL should be prioritized over NR SL to minimize impact to NR UL. |
| Ericsson | A3 | Agree with Huawei |
| CATT | A3 with comment | We basically agree with HW. But we reckon current Option A3 is not precise and provide a change: - Option A3: In case the threshold(s) are not (pre-)configured by an SL incapable RAN node, the NR UL is always prioritized over LTE/NR SL TX. Because in LTE, thresSL-TxPrioritization is also configured in pre-configuration. In NR, we can also use this method (using the pre-configured threshold) if UE is under SL incapable RAN node. |
| Lenovo | A3 | Agree with Huawei that this scenario is a corner case. If we need to consider such corner case, then NR UL is always prioritized |
| Qualcomm | A1 with comment | Emergency call and MSG1/MSG3 in RACH get prioritized, and the NR priority rules may also apply if pre-configured. |
| Apple | A1 if a SL threshold is preconfigured in UE; A3 if no threshold is preconfigured | the SL-incapable RAN node cannot provide NW-configuration for threholods used for Uu/SL prioritization. In case there is a preconfiguration in UE, then A1 is proper. If there is no preconfiguration, A3 can be used. But I think the SL threshold can always be included in preconfiguration of NR V2X UE to make the solution simply as same as LTE. |
| Spreadtrum | A1 with comments | Emergency call and MSG1/MSG3 in RACH gets prioritized. NR UL-SL prioritization rule can be reused if threshold is pre-configured, otherwise, UL should always be prioritized. |
| Sharp | A3 | Agree with Huawei |
| vivo | A3 | Agree with the companies above. Pre-configuration can provide threshold parameters for prioritization decision. Otherwise NR Uu is prioritized. |
| Xiaomi | A1 | Existing spec is enough. No need to change. SL threshold should be default pre-configured. |
| Fujitsu | A1 | Agree with OPPO. |
| ASUSTeK | A3 |  |
| Nokia | A3 with comments | We are fine with specifying this case if majority agrees, but thresholds should be in pre-configuration as OPPO describes, so the issue should not occur |

## Issue 2: Missing case for SR prioritization

The related proposals are also available below:

| Company | Tdoc | Proposals |
| --- | --- | --- |
| Huawei | R2-2005492 | Proposal 10: When SL SR and UL data overlaps, the SL SR is prioritized only when priority value of the logical channel which triggers the SR is lower than sl-Prioritizationthres and the value of the highest priority of the logical channel(s) in the MAC PDU is higher than ul-PrioritizationThres. |

Regarding the prioritization case where the PUCCH resource for SR transmission occasion overlaps with any UL-SCH resource(s), in RAN2#108, some agreement were reached during IIOT discussion.

Intra-UE prioritization:

1: 	If PUCCH resource for an SR’s transmission occasion overlaps a UL-SCH resource, SR’s transmission is allowed (prioritized) based on a comparison of priority of the LCH that triggered the SR and a priority value for the UL-SCH resource (where the priority value is determined as in previous agreement), if the priority of the LCH that triggered the SR is higher.

2:	For SR-Data conflict with equal priority, UL-SCH (i.e. data) is prioritized.

Similar overlapping can be occurred in SL communication, i.e., the SR transmission occasion can be triggered by SL LCH, and the SL transmission may overlap with UL-SCH resource. For such overlapping case, similar principle with IIOT solution can be applied considering the two priority thresholds (i.e., sl-Prioritizationthres, ul-PrioritizationThres).

Observation 1B: RAN2 need to discuss how to handle the prioritization between the SL SR and UL data in case the pending SR triggered by SL LCH overlaps with any UL-SCH resource(s).

Rapporteur thinks that principle raised by R2-2005492 can be used in this overlapping case. Therefore, SL SR transmission will be prioritized only when priority value of the logical channel which triggers the SR is lower than sl-Prioritizationthres and the value of the highest priority of the logical channel(s) in the MAC PDU is higher than ul-PrioritizationThres.

### Question 1B: How to handle the prioritization between the SL SR and UL data in case the pending SR triggered by SL LCH overlaps with any UL-SCH resource(s)?

- Option B1: When SL SR and UL data overlaps, the SL SR is prioritized only when priority value of the logical channel which triggers the SR is lower than sl-Prioritizationthres and the value of the highest priority of the logical channel(s) in the MAC PDU is higher than ul-PrioritizationThres

- Option B2:  Other solution (Specify, if selected)

- Option B3: No solution is needed for this case

| Company | Preferred Option | Comment |
| --- | --- | --- |
| OPPO | B1 with comment | If there is MAC CE whose priority is always higher than MAC SDU except for CCCH, then UL is always prioritized. If there is only MAC CE whose priority is always lower than MAC SDU, then 2nd part of the condition is not applied. |
| HW | B1 | Follow the NR SL/UL data prioritization rule. |
| Interdigital | B1 | This rule is consistent with what we have specified so far for other cases. |
| Samsung | B1 | We think that NR SL/UL prioritization rule is applicable. |
| Futurewei | B1 | Consistent with NR SL/UL prioritization. |
| Intel | B1 | It seems logical to follow the NR SL/UL prioritization rule |
| MediaTek | B1 | It makes sense to follow the same way we prioritize UL SR and SL data, i.e., consider the priority of SR same as the priority value of logical channel which triggers the SR, and then follow the UL/SL data prioritization rule. |
| Ericsson | B1 |  |
| CATT | B1 | Agree with Rapporteur. |
| Lenovo | B1 | We would like to reuse NR UL/SL prioritization rule and think unified rule is better. |
| Qualcomm | B1 |  |
| Apple | B1 |  |
| Spreadtrum | B1 |  |
| Sharp | B1 |  |
| vivo | B1 | Align with NR SL/UL prioritization. |
| Xiaomi | B1 |  |
| Fujitsu | B1 | We can follow the NR SL/UL prioritization rule. |
| ASUSTeK | B1 | For prioritization between SR triggered by LCH and UL data, we are ok to follow the same rule as SL/UL data prioritization rule. |
| Nokia | B1 |  |

## Issue 3: Whether to define remaining PDB

The related proposals are also available below:

| Company | Tdoc | Proposals |
| --- | --- | --- |
| MediaTek | R2-2004751 | Observation 5: The term “remaining PDB” is used in MAC spec but not defined. Proposal 5-1: the remaining PDB of SL data can be the PDB of a QoS flow mapped to it minus the time since SL data generated until the resource reselection. Proposal 5-2: the remaining PDB of SL MAC CE can be the latency bound minus the time since SL MAC CE generated until the resource reselection. Proposal 5-3: Only the sidelink logical channel(s) with SL data available for transmission and/or the triggered sidelink MAC CE are taken into account for determining the remaining PDB for the PSSCH/PSCCH transmission. Proposal 5-4: The remaining PDB for the PSSCH/PSCCH transmission is the minimum value of the remaining PDB(s) of the sidelink logical channel(s) with SL data available and the latency bound of the triggered sidelink MAC CE for a destination. |
| Fujitsu | R2-2004889 | Proposal 1: Only the sidelink logical channel(s) with data available for transmission and/or the triggered sidelink MAC CE are considered for determining the remaining PDB for the PSSCH/PSCCH transmission. Proposal 2: The remaining PDB for the PSSCH/PSCCH transmission is the minimum value of the remaining PDB(s) of the sidelink logical channel(s) with data available and the latency bound of the triggered sidelink MAC CE for a destination. |

The term “remaining PDB” is already captured in MAC CR. However, some of contributions said that it is still not clear what is the meaning of “remaining PDB” in the MAC specification. They argue that current MAC CR only includes the same term as RAN1 agreements without defining it. In order to provide the remaining PDB for the PSSCH/PSCCH transmission to the physical layer and also to perform resource (re)selection in the MAC layer taking this remaining PDB into account.

Rapporteur thinks that anyhow UE can know remaining PDB internally, UE can determine remaining PDB considering the available data(s) from upper layer by UE implementation. In the meanwhile, R2-2004751, R2-2004889 propose to define the definition of remaining PDB on current running MAC CR. RAN2 can discuss whether to define remaining PDB in current MAC CR or not.

### Question 3A: Do you need to define remaining PDB in current MAC CR 38.321?

- Yes

- No (leave it to UE implementation)

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | No | It can be handled by UE implementation anyway. |
| HW | No | We support to leave it to UE implementation |
| Interdigital | No | It is fine to leave this to UE implementation, as was the case in LTE. |
| Samsung | No | We are fine to leave it to UE implementation. |
| Futurewei | No | This can be left to UE implementation. |
| Intel | No | Agree with the companies above |
| MediaTek | Yes | It does not make sense that we mention the concept of “remaining PDB” for UE to perform resource (re)selection, but we mention nothing in any spec about how to derive remaining PDB. In our view, the definition of remaining PDB is not super complex and thus we strongly suggest to clarify its definition in the spec. Besides, in general, we will say “how to do something is up to UE implementation”, but we will not say “how to define a mentioned concept is up to UE implementation”. |
| Ericsson | No with comment | We are fine to leave it to UE implementation. Another alternative to compromise is to rephrase “remaining PDB” as e.g. “latency requirement” to have less mandatory flavour. |
| CATT | No | We prefer to leave it to UE implementation. |
| Lenovo | Yes and No | Without going into defining the exact calculation/ formula, we can generally say that remaining PDB is the “remaining Packet Delay Budget after considering how long has the packet been waiting in the L2 buffers before getting scheduled for transmission” or something similar, so that implementations will not be very diverting. |
| Qualcomm | No | Leave to UE implementation |
| Apple | No strong vew | Remaning PDB is a very clear concept. But there is also no harm to clarify it. |
| Spreadtrum | No |  |
| Sharp | No | UE implementation is preferred. |
| vivo | No | It is fine to leave this to UE implementation. |
| Xiaomi | Yes | Assuming companies have common understanding on this term, it’s better to capture it in the spec. |
| Fujitsu | Yes | PDB is a PC5 QoS characteristics for a QoS flow, and a MAC PDU may include the data from more than one logical channels corresponding to multiple PC5 QoS flows and/or a MAC CE. Currently it is unclear about what is the remaining PDB for the PSSCH/PSCCH transmission in MAC. Therefore, we propose to define the remaining PDB the PSSCH/PSCCH transmission in MAC, i.e. the minimum value of the remaining PDB(s) of the sidelink logical channel(s) with data available and the latency bound of the triggered sidelink MAC CE. |
| ASUSTeK | No |  |
| Nokia | No, with comment | Agree with Ericsson |

### Question 3B: If yes in Q3A, how to define remaining PDB of SL data?

- Option B1: the remaining PDB of SL data can be the PDB of a QoS flow mapped to it minus the time since SL data generated until the resource reselection

- Option B2:  Others (Specify, if selected)

| Company | Preferred Option | Comment |
| --- | --- | --- |
| MediaTek | B1, but | The intention is correct, but the wording needs to be improved. We suggest to update it as “… since the SL data becomes available for transmission”. The reason is because “generated” is not a legacy description we use for the arriving data in MAC spec. |
| Lenovo |  | We offered one tentative definition: “remaining Packet Delay Budget after considering how long has the packet been waiting in the L2 buffers before getting scheduled for transmission” |
| Xiaomi | B1 |  |
| Fujitsu | B1 | The remaining PDB of a sidelink logical channel can be the PDB of a QoS flow mapped to it minus the processing time of the packet(s) of the QoS flow in the higher layers. |

### Question 3C: If yes in Q3A, how to define remaining PDB of SL MAC CE?

- Option C1: the remaining PDB of SL MAC CE can be the latency bound minus the time since SL MAC CE generated until the resource reselection

- Option C2:  Others (Specify, if selected)

| Company | Preferred Option | Comment |
| --- | --- | --- |
| MediaTek | C1, but | We think it should be “since the MAC CE is triggered”. This is because SL MAC CE, same as UL MAC CE, is not generated until sidelink resource becomes available for transmission. So, the duration from “SL CSI report is triggered” to “SL resource is available and thus SL CSI report MAC CE is generated” should be taken into account for remaining PDB. |
| Lenovo |  | Not necessary to also define this for MAC CE(s). However, if we do want to define this for e.g. CSI MAC CE then like MediaTek said remaining PDB is the time remaining in PDB since “SL CSI report is triggered” can be used. |
| Xiaomi | C1 |  |
| Fujitsu |  | The SL MAC CE is generated if the SL resource for new transmission is available, so the resource selection is before the SL MAC CE generation. So C1 is somewhat not correct. Since resource (re)selection using the remaining PDB may be triggered when a SL MAC CE is triggered, we think that the remaining PDB of SL MAC CE can simply be the latency bound of the MAC CE. |

## Issue 4: Maximum number of receiving sidelink process

The related proposals are also available below:

| Company | Tdoc | Proposals |
| --- | --- | --- |
| Huawei, HiSilicon | R2-2005492 | Proposal 11d: In NR V2X, the maximum number of receiving Sidelink processes associated with the Sidelink HARQ Entity is 64. |

In order to ensure that the RX UE has enough Sidelink processes to receive data from different TX UEs, the maximum number of receiving Sidelink processes associated with the Sidelink HARQ Entity for reception of the SL-SCH needs to be greater than the maximum number of transmitting Sidelink processes associated with the Sidelink HARQ Entity for transmission on SL-SCH. R2-2005492 proposes it can be 64, taking into account the overhead of HARQ buffer.

### Question 4A: Which option do you prefer for the maximum number of receiving Sidelink processes associated with the Sidelink HARQ entity in NR sidelink communication?

- Option A1: 64

- Option A2: More than 64 (Specify your preferred value, if selected)

- Option A3: Less than 64 (Specify your preferred value, if selected)

| Company | Preferred Option | Comment |
| --- | --- | --- |
| OPPO | A3 | Even though considering TX-UE cannot perform FDMed transmission at the same slot but RX-UE can perform FDMed reception at the same slot, the necessity of having increased number of Rx buffer depends on the ratio between unidirectional traffic (Rx-only) and bidirectional traffic (Tx and Rx) for a same UE. Considering the possible impact / cost UE, we tend to limit the value to be less than 64, e.g., 16. |
| HW | A1 |  |
| Interdigital | A3 | 16 should be sufficient and is already higher than LTE |
| Samsung | A3 | We share the view as OPPO and Interdigital. 16 should be fine. |
| Futurewei | ? | Should it be “minimum” number, if the intent is to ensure that the RX UE has enough Sidelink processes to receive data from different TX UEs? The “minimum” number of RX UE’s HARQ processes should be larger than the “maximum” number of TX UE’s HARQ processes. |
| Intel | A3 | No strong view, but we currently don’t envision the need for 64 |
| MediaTek | A3? | We are not sure whether we need to define the maximum number of receiving HARQ process or we just leave it to UE implementation. Besides, we wonder whether it is related to UE capability, e.g. 16, 32, or 64 depending on UE capability. We prefer 16 or 32 since 64 is a bit larger than our expectation. |
| CATT | A1 | Considering the topic for the maximum number of transmitting Sidelink processes associated with each Sidelink HARQ Entity is still proceeding in R2-2005720, We’d like to choose 64 as the maximum number. |
| Lenovo | A3 | We think RAN1 has also discussed this; we can tentatively keep it at 64. |
| Qualcomm | A3 | We share the view expressed by MediaTek regarding the need to define a maximum, and the relationship to UE capability. It would be helpful to understand how this interacts with the ongoing UE capability discussion in RAN1 on the number of HARQ processes. |
| Apple | A3 with comments | Why there is a need for upper bound? If this is a minimum number to support typical SL operation, 16 is a proper value. |
| Spreadtrum | A3 | We think 16 is enough. |
| Sharp | A3 | 16 is a proper value. |
| vivo | A3? | We think it can be left to UE implementation. The basic number of RX HARQ processes should be larger than the maximum number of TX HARQ processes. But it may not need to limit or define what number of TX UEs a RX UE can communicate with simultaneously. |
| Xiaomi | A1 | No strong view. Theoretically, the number of HARQ process in RX side should not be smaller than TX side. |
| Fujitsu | A3 | No strong view, but slightly prefer less than 64. |
| ASUSTeK | At least 16? | If the maximum number of transmitting sidleink process is 16, then the minimum number of receiving process should be 16. No strong view on maximum number of receiving process. |
| Nokia | (A1) | No strong view, but agree with Lenovo that we may wait for RAN1 |

## Issue 5: Latency issues for CSI reporting

The related proposals are also available below:

| Company | Tdoc | Proposals |
| --- | --- | --- |
| vivo | R2-2005297 | Proposal 8: CSI reporting PDB window or timer will start in the first symbol of the next slot after the end of CSI trigger reception. Proposal 9: UE will continue the MAC PDU retransmission which multiplexes a CSI report with HARQ feedback enabled data after the latency bound of the CSI report exceeds. Proposal 10: If proposal 9 is agreed, it is left to RX UE implementation on how to distinguish CSI reports in same CSI report window, e.g. associate received CSI report with the CSI trigger based on the location of the first repetition of the corresponding CSI report transmission. |

In last RAN1 #100b-e meeting, the following agreement was made regarding CSI PDB.

Agreements: - The latency bound of SL CSI report is signaled from CSI triggering UE to CSI reporting UE via PC5-RRC. - The CSI triggering UE determines the latency bound by its implementation.

For the RX UE side, R2-2005297 proposes that a clear PDB window definition is needed after the end of CSI trigger reception. On the other hands, it can be also UE implementation how to RX UE select reporting PDB window. RAN2 can decide whether to define a clear PDB window for RX UE or not.

### Question 5A: Do you need to define CSI reporting PDB window for the CSI reporting UE?

- Yes

- No (leave it to UE implementation)

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | No | We do not see necessity to further explore this feature at this late stage. |
| HW | Yes | We think the CSI reporting UE needs to maintain a timer or window to cancel the CSI reporting upon expiry of the timer or window. |
| Interdigital | Yes | The trigger for cancelling a CSI report is based on the expiry of the required latency for that report, so it makes sense to define a PDB window. |
| Samsung | No | We think it is fine to leave it up to UE. |
| Futurewei | Yes | Otherwise, this is not testable. |
| Intel | No | Agree with OPPO |
| MediaTek | Yes | It makes sense to use a timer to model the “delay bound” mentioned in RAN1 agreement. |
| Ericsson | Yes | Snice the latency bound is signalled form CSI triggering UE, then the CSI reporting UE should follow it. A clear procedure defined in the spec will be helpful. |
| CATT | No | We prefer to leave it to UE implementation. |
| Lenovo | Yes | Since it is clear that there is a “latency bound” associated with a CSI reporting. We can specify a timer - leaving this to UE implementation may not be sufficient since there’s an associated UE behaviour at timer expiry and timer stopping. Also, in future when multiple such reports (towards same and different destinations) are waiting for transmission, a specific UE behaviour alongside timer would need to be defined. |
| Qualcomm | No | Agree with OPPO |
| Apple | Yes | We agree that a PDB window is needed for this case. |
| Spreadtrum | No | Agree with OPPO |
| Sharp | Yes | We share the same view with Ericsson, a clear procedure is preferred. |
| vivo | Yes | Timer or window defined for CSI reporting will make UE behaviours cleaner, especially expiry behaviours. |
| Xiaomi | No | The requirement and definition are clear. We can leave it to UE implementation. |
| Fujitsu | No | Agree with OPPO |
| ASUSTek | No | Similar to remaining PDB for SL data, CSI reporting PDB can be leave to UE implementation without specifying. |
| Nokia | Yes | It should be already defined by the latency, but would be OK for us to specify |

### Question 5B: If yes in Q5A, how to define CSI reporting PDB window?

| Company |  |
| --- | --- |
| HW | The value of the PDB window is the same as the latency bound signalled from CSI triggering UE to CSI reporting UE via PC5-RRC. The UE shall:  Start the window upon the Sidelink CSI reporting is triggered;  Stop the window upon the Sidelink CSI reporting is transmitted;  Cancel the Sidelink CSI reporting upon window expiry. |
| Interdigital | We are ok with the definition from Huawei, whereby the window should start in the first slot after the reception of the CSI report trigger by the PHY layer. |
| Futurewei | Agree with Huawei. |
| MediaTek | Agree with Huawei. |
| Ericsson | Agree with Huawei |
| Lenovo | Agree with HW (It can be modelled as PDB window/ timer) |
| Apple | Agree with Huawei |
| Sharp | Agree with Huawei |
| vivo | We are fine with views from Huawei, and RAN2 can decide between two start points of window: Option 1: CSI reporting PDB window starts in the first symbol after the end of CSI trigger reception; Option 2: CSI reporting PDB window starts in the first symbol of the next slot after the end of CSI trigger reception; Either one is OK. UE needs an accurate window definition. |

## Issue 6: Remaining issues on Groupcast HARQ feedback

Regarding issues on without distance-based HARQ operation, the related proposals are also available below:

| Company | Tdoc | Proposals |
| --- | --- | --- |
| InterDigital Inc., Apple, Lenovo, Motorola Mobility | R2-2005325 | Proposal 4: TX UE enables HARQ feedback without distance-based operation when the LCHs transmitted on a grant have no communication range configured. Proposal 5: The UE does not multiplex logical channels configured with range and logical channels configured without range together (similar to HARQ enable/disable) |

In RAN2#109bis-e Error: Reference source not found, the following agreement was made:

14:	A TX UE can use distance HARQ feedback only when the TX UE’s location is available (as agreed in RAN1). When the TX UE’s location is not available, TX UE enables HARQ feedback without the distance-based operation.

When TX UE does not know its location, UE cannot use distance-based HARQ feedback. Another case where the UE cannot uses distance based HARQ feedback is when it transmit data from LCHs is configured without a range configuration (i.e., no sl-TransRange in the radio bearer configuration). RAN2 need to discuss how to UE operate when UE is not configured a range configuration. R2-2005325 proposes TX UE enables HARQ feedback without distance-based operation same as when TX location is unavailable.

And, also, the latest RAN1 e-meeting, they have a conclusion regarding the issues on groupcast HARQ as follows. They considered that groupcast option 1 and option 2 are both possible when range requirement is not provided, and concluded that option 1 is possible through L1 signalling.

Conclusion: - It is feasible from L1 signaling perspective to use Groupcast option 1 (i.e., NACK only feedback) when Zone ID or Communication range requirement is not provided, if RAN2 decides to support this operation. - No action in RAN1 unless RAN2 informs RAN1 about their decision (to support or not) - Note that if RAN2 decides to support it, RAN1 needs to further discuss

Rapporteur thinks that, for the point of CR correction, in current CR, TX UE can set range requirement and location information for SCI only “if configured”. UE already operates HARQ feedback without distance-based operation.

### Question 6A: What should TX UE do for groupcast HARQ feedback when range configuration is not configured for a sidelink logical channel or Zone_id is not configured by the network?

- Option A1: TX UE enables HARQ feedback without the distance-based operation (aligned with the in-principle agreed CR and same as when location information is not available).

- Option A2: TX UE disables HARQ feedback

- Option A3: Up to UE implementation

- Option A4: TX UE enables HARQ feedback without the distance-based operation but does not multiplex LCHs with and without range requirement together

| Company | Preferred Option | Comment |
| --- | --- | --- |
| OPPO | A1 |  |
| HW | A2 | This is a similar case as TX UE’s location is not available and in both cases, UE should disable HARQ feedback. Actually as cited above in the RAN1 agreement, current they did not agree to support without distance-based option 1 (NACK-only feedback) and it is up to RAN2 to make a final decision. But please note RAN1 needs further discussion on how to support it if RAN2 agrees to support it. However, this is the last meeting before completing the WI as well as freezing the spec and RAN1 meeting already closed. If RAN2 agrees to support this, how and when to deal with the impact on RAN1? This is quite likely to further result in the inter-operability issues with the conclusions made after June. Therefore, we think similar to the mixed retransmission issue, without distance-based option 1 should not be supported as this is the last meeting and there is significant impact on RAN1. |
| Interdigital | A4 | If a LCH with range requirement (say 50m) is multiplexed with a LCH without range requirement, the UE would set the range in SCI to be 50m, and the QoS of the second LCH would only be met to 50m, which is not intended. Therefore, option 4 should be the correct MAC behaviour. As for option A2, HARQ enable/disable should be controlled by the SLRB configuration only. |
| Samsung | A1 |  |
| Futurewei | A2 | Option A1 is not a good choice, given that RAN1 hasn’t been able to come up details for it. As it is the last meeting to finalize R16 functionalities, the issue can be handled as an exception case in MAC to just have the HARQ feedback disabled. |
| Intel | A1 | We think A1 is the most logical way forward. For A2, we think it should just be based on LCH configuration. As for A4, the case of “multiplexing LCH with and without range requirement together” does not need to be considered since if the upper layer does not provide a range requirement for a given LCH, it implies that the application does not care what range is used at the AS layer for the transmission of the corresponding data. So, that LCH can be multiplexed with other LCH(s) with any range requirement for AS layer perspective. |
| MediaTek | A1 | If TX UE cannot provide information for Rx UE to determine whether Rx UE is in communication range or not (e.g. lack of TX UE location information, zone id information, or range communication), Tx UE should enables HARQ feedback without the distance-based operation. |
| Ericsson | A1 | In our understanding, Groupcast HARQ option 1 is essentially about NACK only, and distance-based is something on top of it. So, even if distance-based FB is not feasible due to unavailable location, groupcast HARQ option 1 can be still adopted. |
| CATT | A1 | Agree with Ericsson. |
| Lenovo | A1 | Same understanding as Ericsson |
| Qualcomm | A1 | This is similar to the scenario when the location of the Tx UE is not available. For this, RAN2 which had been has agreed that the Tx UE may enable non-distance based HARQ. This The condition in this question is just another case of non-distance based HARQ, which is supported already by RAN2. In addition, it has stated in the LS from RAN1’s LS has stated that it is feasible from an L1 signaling perspective it is feasible to use Groupcast option 1 (i.e., NACK only feedback) when Zone ID or Communication range requirement is not provided. |
| Apple | A4 if LCH is not configured with range requirements, A1 if zone-config is not provided by NW | I think the question contains two cases and need to be discussed separately, First, if a LCH is configured without range requirement, then it will use groupcast HARQ feedback w/o distance-based mechanism, but at the same time other LCHs with range configuration can still use the location-based HARQ ACK . Those two LCHs cannot be mixed in the same MAC PDU. In this case, A4 is used. But if zone-config is not provided, then the TX UE cannot do any location-based HARQ, so A1 is used in this case |
| Spreadtrum | A1 | If Zone_ID or range configuration is not available, groupcast feedback option 1 can still be used. |
| Sharp | A1 |  |
| vivo | A1 |  |
| Xiaomi | A1 |  |
| Fujitsu | A1 |  |
| ASUSTeK | A1 |  |
| Nokia | A2 if LCH is not configured with range requirements A1 if zone-id is not provided. | Please note that the case only counts when no group information is available either. We should avoid unecessary retransmissions, hence if the LCH is not configured with range requirements we cannot control it as intended. If zone ID is not configured by the network though, we can agree with A1, as this is not UEs fault. |

# Conclusion and recommendation