3GPP TSG RAN WG2#110-e	R2-2006098

Online meeting, 1st - 12th June, 2020

Title:	LS Reply on DCP Open Issues						R1-2004994

Response to:	R1-2003260 (R2-2004175)

Release:	Rel-16

Study Item:	NR_UE_pow_sav-Core

Source:	RAN1

To:	RAN2

CC: 	RAN4

Contact Person:

#### Name:	Fang-Chen Cheng

E-mail Address:	fcc@catt.cn

1. Overall Description:

RAN1 would like to thank RAN2 on DCP open issues and informing RAN1 of the RAN2 agreements.

RAN1 discussed the issue on the collision of DCP and RAR when a DCP monitoring occasion overlaps with the ra-ResponseWindow or msgB-ResponseWindow. Regarding RAN2’s understanding on RAR impact with the collision of DCP and RAR when the corresponding search spaces are not quasi-collocated, RAN1 understanding is:

- RAR addressed to C-RNTI would not be impacted if the DCI scheduling RAR addressed to C-RNTI is configured on type-3 CSS with lower index than that of DCP, according to TS38.213 [1];

- RAR addressed to C-RNTI would not be prioritized over DCP if it is configured on USS or on type-3 CSS with higher index than that of DCP.

RAN1 could not reach a consensus on whether to always prioritize RAR addressed to C-RNTI over DCP by RAN1 specification change or to leave it to be handled by network implementation.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #102	24th - 28th  August, 2020     e-Meeting

TSG-RAN WG1 Meeting #103	12th -16th October, 2020     China

# Reference

- TS38.213v16.1.0, “Physical layer procedures for control”