3GPP TSG-RAN WG2 Meeting #111-e	R2-2006582

Electronic, 17th – 28th Aug, 2020

Agenda item:	8.6.2

Source:	Huawei, HiSilicon

Title:	Common aspects between RACH and CG-based scheme

Document for:	Discussion

- Introduction

A WI on small data transmission from RRC_INACTIVE was approved [1]. Based on the WI, RAN2 needs to specify the general procedures for both RACH-based schemes and CG-based schemes.

- For the RRC_INACTIVE state: - UL small data transmissions for RACH-based schemes (i.e. 2-step and 4-step RACH): - General procedure to enable UP data transmission for small data packets from INACTIVE state (e.g. using MSGA or MSG3) [RAN2] - Enable flexible payload sizes larger than the Rel-16 CCCH message size that is possible currently for INACTIVE state for MSGA and MSG3 to support UP data transmission in UL (actual payload size can be up to network configuration) [RAN2] - Context fetch and data forwarding (with and without anchor relocation) in INACTIVE state for RACH-based solutions [RAN2, RAN3] Note 1: The security aspects of the above solutions should be checked with SA3 - Transmission of UL data on pre-configured PUSCH resources (i.e. reusing the configured grant type 1) – when TA is valid - General procedure for small data transmission over configured grant type 1 resources from INACTIVE state [RAN2] - Configuration of the configured grant type1 resources for small data transmission in UL for INACTIVE state [RAN2]

In this paper, we discuss the common aspects for small data transmission (SMT) between random access based schemes and CG based scheme, and provide our considerations.

- Discussion

2.1 Small data packet size and use case

Based on the WI [1], the following services are considered for small data transmission.

- Smartphone applications: - Traffic from Instant Messaging services (whatsapp, QQ, wechat etc) - Heart-beat/keep-alive traffic from IM/email clients and other apps - Push notifications from various applications - Non-smartphone applications: - Traffic from wearables (periodic positioning information etc) - sensors (Industrial Wireless Sensor Networks transmitting temperature, pressure readings periodically or in an event triggered manner etc) - smart meters and smart meter networks sending periodic meter readings

For NR small data, it can support more services and subsequent transmissions. RAN2 needs to decide the packet sizes for a single transmission and occurrences of subsequent transmission firstly. We provide the typical traffic pattern for Heart-beat and for Instant messaging service in the following, respectively. The other services, e.g., traffic from wearables, usually consist heart-beat traffic and IM traffic.

- Heart-beat traffic: Heart-beat traffic is typical TCP-IP traffic, which consists of a

For Heart-Beat traffic, the IP packet size is typical around 100 bytes and a Heart-Beat message arrives every 5 minutes or in the order of seconds;

Observation 1: The IP packet size should be supported for Heart-beat traffic is around 100 bytes and a Heart-beat message arrives every 5 minutes;

- Instant Message traffic: For Instant Messaging services, e.g. WeChat, the procedures for Instant Messaging between Client (i.e. UE) and Server was observed as the followings. There is four steps between Client and Server.

- The Client sends the Instant message to the Server;

- The Server responses TCP ACK for Instant message to the Client;

- The Server sends the transmission status notification for the Instant message to the Client;

- The Client responses the TCP ACK for the transmission status notification to the Server.

Figure 1: Typical Traffic for Instant Message between UE and server

As we observed, the time gap between every two steps in figure 1 is about 200 ms ~ 300 ms. The packet size for the Instant message is different with different content (in step 1), for which we provide a table with different content and its packet size as showed in Table 1. The size for TCP ACK is around 56 bytes (in step 2 and step 4), and the size is around 210 bytes for transmission status notification (in step 3).

| Case | Content in Instant message | Packet size for Instant message |
| --- | --- | --- |
| 1 | “1234567890” | 201 bytes |
| 2 | One Emoji | 201 bytes |
| 3 | 24 Chinese characters | 265 bytes |
| 4 | 120 Chinese characters | 379 bytes |
| 5 | 240 Chinese characters | 395 bytes |

Table 1: Typical packet size for instant message

Considering that 100 Chinese characters in WeChat to be transmitted is rare, we think the largest packet size is 300 bytes for Instant messaging services. While for transmitting of contents in other formats, e.g., photos, videos, the size of the content varies greatly with the length, definition and other factors, while generally, the size is much larger than the contents in plain text. Hence, we think they are out of the scope of the discussion in small data transmission in RRC_INACTIVE and more suitable to be transmitted in RRC_CONNECTED.

Observation 2: The largest packet size should be supported for Instant Messaging services is about 300 bytes (in step 1), the packet size for TCP ACK is about 56 bytes (in step 2 and step 4), the packet size for transmission status notification is about 210 bytes (in step 3) and the message is arrived between every two steps within 200 ms ~ 300 ms;

2.1.1 Small data packet size

Based on the above observations, we think the largest packet size is 300 bytes for small data transmission based on the traffic patterns and sizes of Instant message and Heart-beat traffic.

Proposal 1: Largest packet size for small data transmission is 300 bytes.

Given the above maximum TBS that small data transmission should support, we think that RAN1 needs to analyse the performance and coverage for different schemes i.e., RACH-based/CG-based for a given TBS under the maximum TBS.

Since the packet size impacts the UL coverage for both RA-based schemes and CG-based scheme, RAN2 should inform RAN1 on decided largest packet size and ask they whether there is any problem to transmit the small data with such packet size in a single uplink transmission for both CG-based scheme and RA-based scheme.

Proposal 2: Send LS to RAN1 on largest packet size of 300 bytes for small data.

2.1.2 Subsequent transmission use cases

Based on the WI, i.e. “Transmission of small data in UL, subsequent transmission of small data in UL and DL and the state transition decisions should be under network control”, we need to consider the subsequent transmission for small data transmission. It is also essential to understand the traffic pattern based on the supported applications for small data in the WID. In order to have a common understanding, we consider the following cases for determining potential subsequent transmission.

| case | UL data | DL data | Subsequent transmission |
| --- | --- | --- | --- |
| 1 | a single transmission for UL user data | No DL user data | - No subsequent DL and UL transmission |
| 2 | a single transmission for UL user data | a single transmission for DL user data | - A single DL subsequent transmission - No subsequent UL transmission |
| 3 | a single transmission for UL user data | Multiple transmissions for DL user data | - Multiple subsequent DL transmissions - No subsequent UL transmission |
| 4 | Multiple transmissions for UL user data | No DL user data | - Multiple subsequent UL transmissions - No subsequent UL transmission |
| 5 | Multiple transmissions for UL user data | a single transmission for DL user data | - Multiple subsequent UL transmissions and a single DL subsequent transmission |
| 6 | Multiple transmissions for UL user data | Multiple transmissions for DL user data | - Multiple subsequent UL and DL transmissions |

We then analyze whether the above cases need to be supported. For case 1, we think it can be supported, e.g. for UDP-based service similar to LTE EDT; For case 2, we think it can be supported, e.g. for TCP-based service similar to LTE EDT; For case 3, we think it can be supported if the downlink data is larger or for the case of IM traffic step 1~3, where there is a single UL data and two DL data; For case 4, we think it can be supported if the UDP-based uplink data is larger. For case 5, we think it can be supported if the TCP-based uplink data is larger. For case 6, we think it can be supported for Traffic from Instant Messaging services.

Proposal 3: The following case(s) should be supported for subsequent transmission.

- Single transmission of UL user data and no DL user data

- Single transmission of UL user data and single transmission of DL user data

- Single transmission of UL user data and multiple transmissions of DL user data

- Multiple transmissions of UL user data and no DL user data

- Multiple transmissions of UL user data and single transmission of DL user data

- Multiple transmissions of UL user data and multiple transmissions of DL user data

2.2 User Plane-SMT vs Control Plane-SMT

In LTE, both user plane (UP) and control plane (CP) solutions are supported for MO-EDT and PUR, as follows

In MO-EDT/PUR for Control Plane CIoT EPS optimisation,

- Uplink user data are transmitted in a NAS message concatenated in UL RRCEarlyDataRequest message on CCCH;

- Downlink user data are optionally transmitted in a NAS message piggybacked in DL RRCEarlyDataComplete message on CCCH;

In MO-EDT/PUR for User Plane CIoT EPS optimisation,

- Uplink user data are transmitted on DTCH multiplexed with UL RRCConnectionResumeRequest message on CCCH;

- Downlink user data are optionally transmitted on DTCH multiplexed with DL RRCConnectionRelease message on DCCH;

For NR small data transmission over NR RRC_INACTIVE, we first need to discuss whether to support for CP solution or UP solution. In RRC_INACTIVE, the UE AS context is kept in last serving gNB and is identified by I-RNTI. Based on the WI, one of objectives is to specify context fetch and data forwarding (with and without anchor relocation) when the UE moves to a new cell. We think UP solution shall be preferred because the last serving gNB needs to identify the UE context based on the contents (e.g. I-RNTI and short resume MAC-I) in uplink RRC message of UP solution provided by UE. While for CP solution, this is not possible because there is no UE context ID provided. Instead S-TMSI is included in uplink RRC message for CP solution. For CP solution, we think it has more CN impacts and SA2 discussion is involved. Furthermore, CP solution is more suitable to be supported in RRC_IDLE when RRC context is not kept.

Proposal 4: User plane solution is supported for R17 small data transmission in RRC_INACTIVE.

2.2.1 Uplink RRC message

Based on the discussion above, we need to further discuss on the PDU structure for the uplink user data and downlink use data. For the uplink transmission in small data, there are two possible solutions.

- Option 1: Similar to the UP solution of LTE MO-EDT/PUR, uplink user data are transmitted on DTCH multiplexed with uplink RRC message on CCCH,

- Option 2: Uplink user data are transmitted on DTCH without multiplexing with uplink RRC message.

Option 1 is supported because the UE may move to a new gNB from the last serving gNB where the UE receives RRCRelease with suspend configuration. In that case the content of uplink RRC message is used to identify the UE context by the Last serving gNB when the new gNB requests the Last serving gNB to provide the UE context. This option can be also applicable to the case where the serving gNB is not changed if contention-based resource is used for small data. We think both CG-based scheme and RA-based scheme can be applicable for this option.

Proposal 5: For uplink small data transmission, uplink user data are transmitted on DTCH multiplexed with RRC message on CCCH.

For option 2, it can be considered to improve resource efficiency for CG-based scheme and RA based scheme if small data is transmitted on the serving cell where the UE received RRCRelease with suspendConfig. This option can be applicable for both CG and RACH-based scheme.

Proposal 6：For small data transmission on serving cell where the UE receives RRCRelease with suspendConfig, RAN2 should study whether to support the transmission of uplink data without RRC message.

In legacy releases [2], the content of RRCResumeRequest1 is the followings.

RRCResumeRequest1 ::= SEQUENCE { rrcResumeRequest1 RRCResumeRequest1-IEs } RRCResumeRequest1-IEs ::= SEQUENCE { resumeIdentity I-RNTI-Value, resumeMAC-I BIT STRING (SIZE (16)), resumeCause ResumeCause, spare BIT STRING (SIZE (1)) }

The reason why in Release-15, RRCResumeRequest1 was introduced was because of the consideration for the coverage. A truncated version of RRCResumerequest1 with shortened I-RNTI was introduced in case of bad coverage. In this case, there are only 48 bits in the RRC message. When to use the full resume message or the shortened one is indicated by the field useFullResumeID in SIB1.

Since for small data transmission, the target is to multiplex uplink data with RRC message, in case of coverage issue within the cell that the UE has to transmit a truncated version of the RRC message to increase the uplink coverage, there is no need to configure small data transmission anymore. Hence, when SIB1 does not broadcast the field useFullResumeID, there is no need to perform small data transmission.

Proposal 7: When SIB1 does not broadcast the field useFullResumeID, network does not configure small data transmission.

In LTE UP-EDT, uplink user data are multiplexed with RRCConnectionResume message containing the resume identity and resume cause, and the following cause values are supported by taking all the resume procedures into account.

ResumeCause ::=              ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1

}

However, for CP-EDT where the uplink user data are concatenated into the RRCEarlyDataRequest messge, and the establishment causes are reconsidered for the purpose CP-EDT only that is different from that in the RRCConnectionResume.

establishmentCause-r15           ENUMERATED {   mo-Data-r15, delayTolerantAccess-r15},

When it comes to NR, according to the CT1 requirement, the resume cases have been extended to up to 11 values as shown below, which would always consume 4bits in each RRC resume procedure. However, regarding the small data transmission procedure in Rel-16, it seems that only few resume causes are indeed necessary in the context of uplink data transmission only, e.g. mo-Data and highPriorityAccess, which is similar to CP-EDT causes. As a result, compared with the legacy resume causes in NR, around 3 bits can be saved thereby more user data can be accommodated.

ResumeCause ::=             ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, rna-Update, mps-PriorityAccess,

mcs-PriorityAccess, spare1, spare2, spare3, spare4, spare5 }

Proposal 7bis: For the uplink RRC message, I-RNTI and resumeMAC-I should be included as in RRCResumeReqeust1 while RAN2 should consider to create a new ResumeCause with reduced signaling cost.

2.2.2 Downlink RRC message

Then, for small data transmission in downlink, we think the following cases need to be considered.

- DL message can include RRC message, e.g., in the case when the network wants to terminate the small data transmission and keeps UE in RRC_INACTIVE with RRCRelease with suspendConfig.

- DL message can also be transmitted without RRC message, e.g., subsequent uplink transmission or subsequent downlink transmission.

Proposal 8: For the downlink small data transmission, downlink user data are optionally transmitted on DTCH multiplexed with RRC message on DCCH or without RRC message.

In legacy [2], the content of RRCRelease is the followings.

RRCRelease-IEs ::= SEQUENCE { redirectedCarrierInfo RedirectedCarrierInfo OPTIONAL, -- Need N cellReselectionPriorities CellReselectionPriorities OPTIONAL, -- Need R suspendConfig SuspendConfig OPTIONAL, -- Need R deprioritisationReq SEQUENCE { deprioritisationType ENUMERATED {frequency, nr}, deprioritisationTimer ENUMERATED {min5, min10, min15, min30} } OPTIONAL, -- Need N lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension RRCRelease-v1540-IEs OPTIONAL } xxxx SuspendConfig ::= SEQUENCE { fullI-RNTI I-RNTI-Value, shortI-RNTI ShortI-RNTI-Value, ran-PagingCycle PagingCycle, ran-NotificationAreaInfo RAN-NotificationAreaInfo OPTIONAL, -- Need M t380 PeriodicRNAU-TimerValue OPTIONAL, -- Need R nextHopChainingCount NextHopChainingCount, ... }

We think the legacy RRCRelease message can be reused for response to the uplink RRC message. I-RNTI, ran-PagingCycle, ran-NotificaitonAreaInfo, t380, nextHopChainingCount are included in RRCrelease for small data transmission.

Proposal 9: Legacy RRCRelease message can be reused as baseline.

After small data transmission in the uplink, we think the following cases are possible if the downlink transmission includes RRC message.

- If the network wants to send the UE goes to RRC_CONECTED and resume the connection, network shall send RRCResume.

- If the network fails to retrieve the UE context, RRCSetup is sent for falling back to RRC connection establishment,

- If the network wants to keep the UE in RRC_INACTIVE and terminate the transmission, RRCRelease with suspend configuration is sent;

- If network wants to send the UE to IDLE and terminate the small data transmission, RRCRelease without suspend configuration is sent and the UE goes to RRC_IDLE.

- If the network experiences high traffic load and does not provide access to the UE, RRCReject can be sent and UE discards stored key and suspends SRB1.

We think all the above scenarios are possible for small data. For small data, after the uplink RRC message is transmitted, the network can respond to the UE with RRCResume or RRCSetup or RRCRelease with suspend configuration or RRCRelease without suspend configuration or RRCReject message as the downlink RRC response depending on the network situation.

Proposal 10: The downlink RRC message in response to small data transmission can be RRCResume, RRCSetup or RRCRelease with suspendConfig or RRCRelease without suspendConfig or RRCReject message.

However the small data transmission procedure ends when RRCRelease message is received. There is no uplink transmission after RRCRelease is received. However for other parameters (e.g. parameters in legacy RRCResumeComplete as showed in following), they shall be transmitted before RRCRelease message is received.

RRCResumeComplete-IEs ::= SEQUENCE { dedicatedNAS-Message DedicatedNAS-Message OPTIONAL, selectedPLMN-Identity INTEGER (1..maxPLMN) OPTIONAL, uplinkTxDirectCurrentList UplinkTxDirectCurrentList OPTIONAL, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension RRCResumeComplete-v1610-IEs OPTIONAL }

Proposal 11: RAN2 to consider how to transmit other new parameters (e.g. parameters in RRCResumeComplete) in RRCRelease.

2.2.3 Security aspects

In R15 RRC_INACTIVE [2], the RRCResumeReqeust and RRCResumeReqeust1 are not ciphered but integrity protected. RRCRelease message is integrity protected and ciphered using the newly derived keys.

For security of small user data, we think that the uplink RRC message and downlink RRC message, i.e. RRCRelease can be handled as legacy if it is included while the user data in uplink and downlink need to be ciphered for security. The keys for user data in uplink and downlink are derived using NextHopChainingCount provided in the RRCRelease message of the previous RRC connection similar to LTE MO-EDT for User Plane CIoT EPS optimisation.

Proposal 12: User data in uplink and downlink are ciphered. The keys are derived using the NextHopChainingCount provided in the RRCRelease message of the previous RRC connection.

Proposal 13: Uplink RRC message for small data is not ciphered but integrity protected with the old key while RRCRelease message is integrity protected and ciphered using the newly derived keys.

2.3 Scheme selection

2.3.1 SMT vs non-SMT

Provided that SMT resources are configured and available upon uplink data arrival at the UE side, one general issue is that the UE has to decide whether to initiate SMT or the legacy procedures, i.e., transfer to RRC_CONNECTED in a well-designed controlled manner. Recall that the motivation of SMT is targeted at reduced UE power consumption, it can be seen that the available data volume is an important factor that should be taken into account. When the UE has more data to transmit regardless of one-shot or subsequent transmission, it is beneficial to transit into the RRC_CONNECTED with link budget adaptation. Otherwise, SMT can show the advantage for the small data. We think date volume can be used as a baseline and further improvement can be discussed at the later stage once we have a good understanding on the SMT procedures.

Proposal 14: Whether to initiate SMT or legacy procedure to transit to the RRC_CONNECTED should be specified in a controlled manner, and data volume can be considered as a baseline for the selection.

2.3.1 CG vs RA for SMT

If the UE has chosen to transmit UL data with small data transmission, based on WI, both RA-based schemes and CG-based scheme are supported for SMT. For RA-based schemes, it can be used regardless of that TA is valid. For the case when TA is valid, the UE needs to determine which scheme is used when both schemes are enable. If CG resource is available, CG -based scheme should be used due to CG has less procedures to save power and was configured to tailor for UE-specific traffic requirement. If the CG resource is not available, RA-based scheme is used.

Proposal 15: If CG resource is available, CG -based scheme is used. Otherwise RA-based scheme is used.

2.4 BWP for small data transmission

In R15 NR, UEs in RRC_CONNECTED can be allowed to perform RA and transmit CG on initial BWP and dedicated BWPs.

For UEs in RRC_INACTIVE, the UE only has valid RA configuration on initial BWP to perform RA. The question comes which BWP can be used for small data transmission over RRC_INACTIVE, especially for CG-based scheme. We think only initial BWP is used for small data transmission over RRC_INACTIVE regardless of CG-based scheme and RA-based scheme.

Proposal 16: Only initial BWP is used for small data transmission over RRC_INACTIVE.

2.6 Procedures for Subsequent transmission

2.6.1 Subsequent uplink transmission

Next, we discuss procedures for subsequent uplink transmission to be supported after the first transmission is transmitted via CG or Msg3 in 4-step RA type or MsgA in 2-step RA type. We propose a solution for subsequent uplink transmission, i.e. the uplink subsequent transmission is transmitted using CG for CG-based schemes or MsgA grant for 2-step RA-based scheme or RAR grant for 4-step RA-based scheme as showed in Figure 2.

Furthermore, the UE is expected to receive a response for uplink data transmission. The response is different between RA-based schemes and CG-based scheme. For RA-based schemes, the response is Msg4 for 4-step RA type or MSGB for 2-step RA type. In this case the UE needs to send HARQ feedback for msg4 or msgB. For CG-based scheme, the response can be a L1 feedback (HARQ feedback similar to NR-U downlink feedback information or indication of fallback to legacy procedure similar to fallback indication in LTE EDT). The procedure terminates when RRCRelease message is received.

Figure 3: subsequent transmission for small data

Proposal 17: Subsequent uplink transmissions are transmitted using CG for CG-based scheme or MsgA UL grant for 2-step RA-based scheme or RAR UL grant for 4-step RA-based scheme.

2.6.2 Assistance information

In LTE EDT, assistance information report, i.e. AS RAI (Release Assistance information), can be triggered for the following conditions.

If the UE is a BL UE or UE in enhanced coverage or an NB-IoT UE, a Downlink Channel Quality Report (DCQR) shall be triggered if any of the following events occur: - DCQR Command MAC control element is received, in which case the DCQR is referred below to as "Regular DCQR"; - for BL UE or UE in enhanced coverage, transmission of DCQR in Msg3 is configured by upper layers in mpdcch-CQI-Reporting, in which case DCQR is referred below to as "Msg3 DCQR". If any type of DCQR has been triggered: - start performing DL channel quality measurements according to TS 36.133 [9]. If "Regular DCQR" has been triggered: - if an uplink grant has been received on the PDCCH for MAC entity's C-RNTI: - instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19; - cancel the triggered "Regular DCQR". If "Msg3 DCQR" has been triggered: - if an uplink grant has been received on the PDCCH for MAC entity's RA-RNTI: - instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19;

The AS RAI MAC CE has the following information.

Table 6.1.3.19-1: Values for AS RAI Codepoint/Index; Value. 00; No RAI information. 01; No subsequent DL and UL data transmission is expected. 10; A single subsequent DL transmission is expected. 11; Reserved.

For small data transmission, we think AS RAI can also be reported to network similar to LTE EDT. Based on the AS RAI, the network can determine when the RRCRelease message should be sent, e.g. the network can transmit the RRCRelease message immediately if no subsequent DL and UL data transmission is expected. However considering more subsequent transmission use cases would be supported, additional information should be included in AS RAI, e.g., whether subsequent uplink transmission is expected, whether there is no subsequent DL and UL data transmission, whether a single subsequent DL transmission is expected, or whether multiple subsequent DL transmissions are expected.

Proposal 18: Assistance information for subsequent UL and DL transmission can be sent to the gNB if it is enable by network.

2.7 Failure handling

In Rel 15 NR [2], T319 is used to control time to perform RRC resume procedure. The timer is started upon transmission of RRCResumeRequest or RRCResumeRequest1. The timer is stopped upon reception of RRCResume, RRCSetup, RRCRelease, RRCRelease with suspendConfig or RRCReject message, cell re-selection and upon abortion of connection establishment by upper layers. The UE goes to RRC_IDLE when T319 expires as the following.

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T319 | Upon transmission of RRCResumeRequest or RRCResumeRequest1. | Upon reception of RRCResume, RRCSetup, RRCRelease, RRCRelease with suspendConfig or RRCReject message, cell re-selection and upon abortion of connection establishment by upper layers. | Perform the actions as specified in 5.3.13.5. |

We think the Resume failure can be reused for small data transmission. However the value of this timer may need to be extended to cover the subsequent uplink and downlink transmission.

Proposal 19: The value of T319 needs to be extended.

2.8 Lossless data transmission

Here we discuss whether to support data less transmission in following case.

- Case 1: cell reselection occurs during small data transmission procedure

- Case 2: RRC setup is triggered after the new gNB fails to retrieve UE context.

For case 1, in Rel-15 NR, the UE goes to RRC_IDLE when cell re-selection occurs during RRC resume procedure. After going to RRC_IDLE, the UE discards Inactive AS context and releases the suspendConfig.

5.3.13.6 Cell re-selection or cell selection while T390, T319 or T302 is running (UE in RRC_INACTIVE) The UE shall: 1> if cell reselection occurs while T319 or T302 is running: 2> perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure'; ====OMITTED====== 5.3.11 UE actions upon going to RRC_IDLE The UE shall: ====OMITTED====== 1> discard the UE Inactive AS context, if any; 1> release the suspendConfig, if configured;

For small data, in that case there is user data which has not been acknowledged by network before going to RRC_IDLE. Considering data lossless transmission for user data, a different story may be needed.

For case 2, after the new gNB receives uplink data and RRC resume request, the new gNB requests the last serving gNB to provide the UE context. In case the new gNB fails to retrieve the UE context. The uplink data will not be decoded successfully by the new gNB due to not have the key for deciphering. The new gNB has to discard this uplink data.

Proposal 20: Lossless data transmission should be addressed when cell reselection occurs during small data transmission or when RRC setup is received after uplink small data transmission.

2.9 Measurement in RRC_INACTIVE

In Rel-15 RRC_INACTIVE, there is no measurement gap configured for inter-frequency measurement. Although the UE kept the UE context, the RRC configuration is allowed to be used only when RRCResume is received. We think this is reused for small data over RRC_INACTIVE.

Proposal 21: Measurement gap is not supported for small data over RRC_INACTIVE.

- Conclusion

Based on the above discussion, we make the following observations and recommend RAN2 to discuss and adopt the following proposals:

Observation 1: The IP packet size should be supported for Heart-beat traffic is around 100 bytes and a Heart-beat message arrives every 5 minutes;

Observation 2: The largest packet size should be supported for Instant Messaging services is about 300 bytes (in step 1), the packet size for TCP ACK is about 56 bytes (in step 2 and step 4), the packet size for transmission status notification is about 210 bytes (in step 3) and the message is arrived between every two steps within 200 ms ~ 300 ms;

Proposal 1: Largest packet size for small data transmission is 300 bytes.

Proposal 2: Send LS to RAN1 on largest packet size of 300 bytes for small data.

Proposal 3: The following case(s) should be supported for small data transmission.

- Single transmission of UL user data and no DL user data

- Single transmission of UL user data and single transmission of DL user data

- Single transmission of UL user data and multiple transmissions of DL user data

- Multiple transmissions of UL user data and no DL user data

- Multiple transmissions of UL user data and single transmission of DL user data

- Multiple transmissions of UL user data and multiple transmissions of DL user data

Proposal 4: User plane solution is supported for R17 small data transmission in RRC_INACTIVE.

Proposal 5: For uplink small data transmission, uplink user data are transmitted on DTCH multiplexed with RRC message on CCCH.

Proposal 6：For small data transmission on serving cell where the UE receives RRCRelease with suspendConfig, RAN2 should study whether to support the transmission of uplink data without RRC message.

Proposal 7: When SIB1 does not broadcast the field useFullResumeID, network does not configure small data transmission.

Proposal 7bis: For the uplink RRC message, I-RNTI and resumeMAC-I should be included as in RRCResumeReqeust1 while RAN2 should consider to adopt a new ResumeCause with reduced signaling cost.

Proposal 8: For the downlink small data transmission, downlink user data are optionally transmitted on DTCH multiplexed with RRC message on DCCH or without RRC message.

Proposal 9: Legacy RRCRelease message can be reused as baseline.

Proposal 10: The downlink RRC message in response to small data transmission can be RRCResume, RRCSetup or RRCRelease with suspendConfig or RRCRelease without suspendConfig or RRCReject message.

Proposal 11: RAN2 to consider whether to include other new parameters (e.g. parameters in RRCResumeComplete) in RRCRelease.

Proposal 12: User data in uplink and downlink are ciphered. The keys are derived using the NextHopChainingCount provided in the RRCRelease message of the previous RRC connection.

Proposal 13: Uplink RRC message for small data is not ciphered but integrity protected with the old key while RRCRelease message is integrity protected and ciphered using the newly derived keys.

Proposal 14: Whether to initiate SMT or legacy procedure to transit to the RRC_CONNECTED should be specified in a controlled manner, and data volume can be considered as a baseline for the selection.

Proposal 15: If CG resource is available, CG-based scheme is used. Otherwise RA-based scheme is used.

Proposal 16: Only initial BWP is used for small data transmission over RRC_INACTIVE.

Proposal 17: Subsequent uplink transmissions are transmitted using CG for CG-based scheme or MsgA UL grant for 2-step RA-based scheme or RAR UL grant for 4-step RA-based scheme.

Proposal 18: Assistance information for subsequent UL and DL transmission can be sent to the gNB if it is enable by network.

Proposal 19: The value of T319 needs to be extended.

Proposal 20: Lossless data transmission should be addressed when cell reselection occurs during small data transmission or when RRC setup is received after uplink small data transmission.

Proposal 21: Measurement gap is not supported for small data over RRC_INACTIVE.

- Reference

- RP-193252 Work Item on NR smalldata transmissions in INACTIVE state

- TS 38.331, Radio Resource Control Protocol, 3GPP, v16.0.1