3GPP TSG-RAN WG2 Meeting #111-e	R2-2006660

Online, 17–28 August 2020

# Agenda item:	8.12.2.1

# Source:	Samsung

# Title:	Capability and initial access of RedCap UEs

# Document for:	Discussion and Agreement

# 1	Introduction

RAN approved a new SI: Study on support of reduced capability NR devices for Rel-17 [1], and responsible WGs (RAN1 & RAN2) should first discuss how to define reduced capability (RedCap) NR devices for Rel-17.

The contribution discusses basic principle of RedCap UEs' capabilities, and initial access of RedCap UEs with such reduced capabilities.

# 2	Discussion

In NR, the concept of UE category is not used anymore, but UE indicates all its capabilities separately (e.g. maxNumberMIMO-LayersPDSCH, supportedModulationOrderDL, etc.), and its maximum data rate can be derived from those capablities.

From the WID [1], the following features can be potentially considered for UE complexity reduction:

- Reduced number of UE RX/TX antennas - UE Bandwidth reduction Note: Rel-15 SSB bandwidth should be reused and L1 changes minimized - Half-Duplex-FDD - Relaxed UE processing time - Relaxed UE processing capability

From the list, each listed item has completely different characteristic, so it would be very difficult to merge them into a single capability (like UE category concept). Therefore, to have a separate capability for each reduced feature can be considered as a baseline.

Proposal 1:	As a baseline, each reduced feature agreed by RAN1 and RAN2 would have a separate capability.

It should be noted that some of features listed above are needed for the Random Access (RA) procedure to access to the cell (i.e. before reporting its capability). Hence, the above listed features can be categorized into two groups: features that only require a capability, and features that require both capability and special handling of RA procedure. Assuming a RedCap UE camps on the supported gNB, and performs the RA procedure, the following table summarizes whether the UE with such a feature is performing RA procedure in advance:

| Feature | gNB needs to know whether such UE performs RA in advance | Remarks |
| --- | --- | --- |
| Reduced number of UE RX/TX antennas | No, but it depends on RAN1 agreements of other SI like Coverage Extension | As the RedCap UE already camps on the cell (assuming it receives DL SSB correctly), it would be able to perform the RA procedure using the legacy RA resources. However, it should be noted that the UE was able to camp on the cell due to some enhancements on DL transmissions for such UEs in Rel-17 (maybe, part of the Rel-17 SI: coverage extension), and in that case, the RA procedure should also be enhanced for better coverage [2]. Then, separate RA resources should be configured for such UEs. Since RAN1 is discussing the impacts by having reduced number of antennas seriously, RAN2 should wait until RAN1 concludes the issue. Anyway, later, UE should indicate its capability to the network to be configured with the relevant configurations by network. |
| UE Bandwidth reduction | No | Since Rel-15 SSB bandwidth would be reused, UE should be able to perform the RA procedure using legacy RA resources (i.e. no updates would be needed for the feature). Then, UE indicates its capability to the network after completion of RA procedure so that it can be configured with the proper configurations (e.g. BWP details) by network. |
| Half-Duplex-FDD | No | The feature exists in LTE, and UE indicates its capability later. So perhaps, to introduce a new capability (per band) would be enough. |
| Relaxed UE processing time/capability | Yes, but maybe No (see remarks) | During the RA procedure, network should know these UEs' access in advance, because these UEs would require longer K2 value (i.e. between RAR and UL grant for Msg3). Hence, one of the easiest ways would be to configure a separate RA resource for RedCap UEs. Then, with the additional RA resource, network would provide longer K2 value. Another alternative would be to not introduce such additional RA resource but network to provide longer K2 values for all the UEs using the legacy RA resource. It would result degrading the performance of legacy UEs a bit at the initial access, but it may be tolerated as it only occurs at the initial access stage. Later, UE indicates its capability to the network to be configured with the relevant configurations by network. |

From the table above, it can be observed that separate RA resources for RedCap UEs may or may not be required depending on the conclusion of 'Reduced number of UE RX/TX antennas' and 'Relaxed UE processing time/capability'. To make some progress for the SI in RAN2, to narrow the discussion scope down (e.g. by excluding the coverage extension aspects) for the time being in RAN2 would be beneficial. After RAN1 makes some progress on these aspect, RAN2 can discuss the technical detail to support such scenarios. If then, RAN2 does not have to consider separate RA resources for the UEs with 'Reduced number of UE RX/TX antennas' (again, for the time being).

Also UEs with 'Relaxed UE processing time' would not require separate RA resources either, if RAN2 agreed that network always provide longer K2 values in RAR for all the UEs, as stated in the table above. As this approach would result degrading the performance of legacy UEs at the initial access, it would be beneficial for UE to indicate its capability as early as possible e.g. in Msg3 after RAR reception. That is, if network receives UE's capability from Msg3, network would be able to schedule those UE with a longer K2 value, while legacy UEs can be scheduled with normal K2 values.

Proposal 2:	For UEs with 'Reduced number of UE RX/TX antennas', RAN2 does not consider 'coverage extension' items for the time being until RAN1 concludes.

Proposal 3:	For both legacy UEs and UEs with 'Relaxed UE processing time/capability', network would provide longer K2 values during initial access, assuming the RA resources are shared between them.

Proposal 4:	Until RAN1 provides input to RAN2, separate RA resources for RedCap UEs are not considered from RAN2 discussion. RAN2 assumes RAN1 will discuss whether separate RA resources should be used, and provide input to RAN2. This will be re-visited depending on their input.

Proposal 5:	RAN2 to discuss whether UE needs to indicate its capability (especially for UE with 'Relaxed UE processing time/capability' before the normal capability signalling.

# 3	Conclusion

Proposal 1:	As a baseline, each reduced feature agreed by RAN1 and RAN2 would have a separate capability.

Proposal 2:	For UEs with 'Reduced number of UE RX/TX antennas', RAN2 does not consider 'coverage extension' items for the time being until RAN1 concludes.

Proposal 3:	For both legacy UEs and UEs with 'Relaxed UE processing time/capability', network would provide longer K2 values during initial access, assuming the RA resources are shared between them.

Proposal 4:	Until RAN1 provides input to RAN2, separate RA resources for RedCap UEs are not considered from RAN2 discussion. RAN2 assumes RAN1 will discuss whether separate RA resources should be used, and provide input to RAN2. This will be re-visited depending on their input.

Proposal 5:	RAN2 to discuss whether UE needs to indicate its capability (especially for UE with 'Relaxed UE processing time/capability' before the normal capability signalling.

# 4	References

[1]	RP-201386	Revised SID on Study on support of reduced capability NR devices	Ericsson

[2]	R1-2003912	Coverage recovery for low capability device	Samsung