3GPP TSG-RAN WG2 Meeting #111-e		R2-2006685

Electronic, 17th Aug. – 28th Aug. 2020

Source:	vivo, Samsung, LG Electronics Inc., MediaTek Inc., Panasonic

Title:	Discussion on how to restructure the RRM relaxation

Agenda Item:	6.9.3

Document for:	Discussion and Decision

- Introduction

During the email discussion: [Post110-e][505][PowSav] RRM Open Issues (Vivo) -- 38.304 CR[1], some companies identified some issues according to current structure of RRM relaxation procedure in TS 38.304. In order to finalize the CR by the deadline, we haven’t discuss how to re-structure the relaxation procedure to resolve any potential problems. Most companies (who provided comments on this point) agreed that we can continue this discussion in August meeting.

This paper will try to identify the potential issues on the current procedure structure for RRM relaxation, and provide solution as well as the corresponding Text Proposal.

- Discussion

  - Problems in current procedure structure

In order to identify any potential problem on the current procedure structure, we first provide the intended mapping for the current specification and the use cases summarized in [2] as below. We can analyze what problem exists in current structure describing all the “IF conditions” and later the “expected actions” based on this mapping.

- On the left column of the following table: we copy the current specification and split into several sub-section.

- On the right column of the following table: we provide intended mapped use case, and the potential problem.

Table 1. Analysis of current procedure structure

| Current specification | Comments and analysis Note: the following use case X is listed in [2]. |
| --- | --- |
| #### 5.2.4.9 Relaxed measurement ##### 5.2.4.9.0 Relaxed measurement rules When the UE is required to perform measurements of intra-frequency or NR inter-frequencies or inter-RAT frequency cells according to the measurement rules in clause 5.2.4.2: | - |
| - if lowMobilityEvaluation is configured and cellEdgeEvaluation is not configured; and, - if the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and, - if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP: - the UE may choose to perform relaxed measurements for intra-frequency, NR inter-frequency, or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.8, 4.2.2.9, and 4.2.2.10 in TS 38.133 [8]; | Use case E and I (when only low-mobility is configured) Issue 1-1: Use case A-2 will also fall into this condition, but the intended behaviour for A-2 should be the legacy. Issue 2-1: Use case A-1 will also fall into this condition, but the intended behaviour for A-1 should be stopping measurement up to 1hour, which is captured as below. i.e. the condition for this part is the subset of the condition for A-1. |
| - if cellEdgeEvaluation is configured and lowMobilityEvaluation is not configured; and, - if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled: - the UE may choose to perform relaxed measurements for intra-frequency, NR inter-frequency, or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.8, 4.2.2.9, and 4.2.2.10 in TS 38.133 [8]; | Use case F and J (when only cell-edge is configured) Issue 1-2: Use case B-1 will also fall into this condition, but the intended behaviour for B-1 should be the legacy. |
| - if both lowMobilityEvaluation and cellEdgeEvaluation are configured; and, - if combineRelaxedMeasCondition is not configured: - if the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell, and, the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; or, - if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled: - the UE may choose to perform relaxed measurements for intra-frequency, NR inter-frequency, or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.8, 4.2.2.9, and 4.2.2.10 in TS 38.133 [8]; | Use case E, F, I, and J (when both conditions with OR is configure) Issue 1-3: Use case B-2 will also fall into this condition, but the intended behaviour for B-2 should be the legacy. |
| - if both lowMobilityEvaluation and cellEdgeEvaluation are configured; and, - if the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and, - if less than 1 hour has passed since measurements for cell (re-)selection were last performed; and, - if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; and, - if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled: - the UE may choose not to perform measurement for measurements of intra-frequency, NR inter-frequencies of equal or lower priority, or inter-RAT frequency cells of equal or lower priority; - if highPriorityMeasRelax is configured with value true: - the UE may choose not to perform measurement for measurements of NR inter-frequencies or inter-RAT frequency cells of higher priority; | Use case C-1, G-1 and K Issue 2-2: Use case G-2 will also fall into this condition, but the intended behaviour for G-2 should be stopping measurement up to Thigher_priority_search. which is captured as below. i.e. the condition for this part is the subset of the condition for G-2. |
| - if lowMobilityEvaluation is configured and cellEdgeEvaluation is not configured; and, - if the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ; and, - if the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and, - if less than 1 hour have passed since measurements for cell (re-)selection were last performed; and, - if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; and, - if highPriorityMeasRelax is configured with value true: - the UE may choose not to perform measurement for measurements of NR inter-frequencies or inter-RAT frequency cells of higher priority; | Use case A-1 |
| - if both lowMobilityEvaluation and cellEdgeEvaluation are configured; and, - if the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ; and, - if the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and, - if less than Thigher_priority_search (see clause 4.2.2.7 in TS 38.133 [8]) has passed since measurements for cell (re-)selection were last performed; and, - if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; and, - if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled; and, - if highPriorityMeasRelax is not configured: - the UE may choose not to perform measurement for measurements of NR inter-frequencies or inter-RAT frequency cells of higher priority. | Use case G-2 |

Observation 1: There is some technique issue (e.g. issue 1 or issue 2 in above table) according to the current procedure structure.

Regarding the other issues discussed in the email discussion, someone maybe have misunderstanding, e.g. when condition is “A or B”, someone may misunderstand “A and B” will also fall into this condition. But strictly speaking, we think this is no technique issue. As commented by some company during the email discussion [Post110-e][505][PowSav] RRM Open Issues (Vivo) -- 38.304 CR, it is obvious that when a UE meets the most favorable requirements for relaxing (e.g. both cell center and low mobility) it also meets the less demanding criteria associated with “less” relaxation. But since all the spec is written in the sense of allowing the UE to relax, it seems obvious that when it is allowed to both relax a bit and relax more, it will choose to relax more. Thus, in the below, we will mainly focus on the above identified issues to re-structure the specification. Whether to polish the specification to avoid any potential misunderstanding is the best effort.

  - Principle to capture the conditions of RRM relaxation

After analyzing the above issues, we think the main reason is that we only use “IF and IF….” structure to list all the conditions for all use cases, but the conditions for some use cases have containment or overlapping. As mentioned by some companies during the email discussion, there are several different branches with the same conditions, e.g. “if both lowMobilityEvalutation and cellEdgeEvalutation are configured”. Thus, in order to solve the above potential problem, I think we should use the “IF/ELSE” statements to reflect the mutex and uniqueness of all conditions.

Observation 2: “IF/ELSE” statements should be used to reflect the mutex and uniqueness of all conditions for all use cases.

Moreover, we should also follow the following principle to re-structure the RRM relaxation procedure part:

- Current procedure should correctly capture the RAN2/RAN4 conclusion, and nothing should be missing.

- For not relaxed measurement case (i.e. legacy behaviour cases), it should not be captured in this section. The UE behaviour should follow the legacy procedure in TS 38.304 and TS 38.133.

- For relaxed measurement cases, it should be captured only once (i.e.in one branch) in the specification, i.e. in order to avoid double captured problem.

- The principle for the UE behaviour in idle/inactive mode should be clear and unique based on TS 38.304 and TS 38.133.

Proposal 1: RAN2 to discuss the re-structure of RRM relaxation procedure based on the above principle.

  - Text proposal

Based on the above discussion, the corresponding text proposal on TS 38.304 is attached in Annex (Note: we also provide the clean version at the end for companies easy checking).

Proposal 2: RAN2 to consider the following TP in Annex about to re-structure of RRM relaxation procedure in TS 38.304.

- Conclusion

In this contribution, we discuss the re-structure of the RRM relaxation procedure in TS 38.304. Based on the discussion, we have the following observations and proposals:

Observation 1: There is some technique issue (e.g. issue 1 or issue 2 in above table) according to the current procedure structure.

Observation 2: “IF/ELSE” statements should be used to reflect the mutex and uniqueness of all conditions for all use cases.

Proposal 1: RAN2 to discuss the re-structure of RRM relaxation procedure based on the above principle.

Proposal 2: RAN2 to consider the following TP in Annex about to re-structure of RRM relaxation procedure in TS 38.304.

- References

- R2-2005949, CR for UE Power Saving in NR, vivo

- R2-2005867, Summary of RRM relaxation behaviors, CATT

# Annex – Text proposal

Start of change

#### 5.2.4.9	Relaxed measurement

##### 5.2.4.9.0	Relaxed measurement rules

When the UE is required to perform measurements of intra-frequency or NR inter-frequencies or inter-RAT frequency cells according to the measurement rules in clause 5.2.4.2:

-	if lowMobilityEvaluation is configured and cellEdgeEvaluation is not configured; and,

-	if the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and,

-	if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP:

-	the UE may choose to perform relaxed measurements for intra-frequency according to relaxation methods in clauses 4.2.2.8 in TS 38.133 [8];

-	if the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

-	if less than 1 hour have passed since measurements for cell (re-)selection were last performed; and,

-	if highPriorityMeasRelax is configured with value true:

-	the UE may choose not to perform measurement for measurements of NR inter-frequencies cells of higher priority, or inter-RAT frequency cells of higher priority;

-	else (i.e. the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ):

-	the UE may choose to perform relaxed measurements for NR inter-frequency or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.9, and 4.2.2.10 in TS 38.133 [8];

-	if cellEdgeEvaluation is configured and lowMobilityEvaluation is not configured; and,

-	if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled:

-	the UE may choose to perform relaxed measurements for intra-frequency cells according to relaxation methods in clauses 4.2.2.8 in TS 38.133 [8];

-	if the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

-	the UE may choose to perform relaxed measurements for NR inter-frequency or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.9, and 4.2.2.10 in TS 38.133 [8];

-	if both lowMobilityEvaluation and cellEdgeEvaluation are configured:

-	if the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and,

-	if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; and,

-	if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled:

-	if less than Thigher_priority_search (see clause 4.2.2.7 in TS 38.133 [8]) has passed since measurements for cell (re-)selection were last performed:

-	if highPriorityMeasRelax is not configured; and,

-	if the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

-	the UE may choose not to perform measurement for measurements of NR inter-frequencies cells of higher priority, or inter-RAT frequency cells of higher priority;

-	else, if less than 1 hour has passed since measurements for cell (re-)selection were last performed:

-	the UE may choose not to perform measurement for measurements of intra-frequency cells, NR inter-frequencies cells of equal or lower priority, or inter-RAT frequency cells of equal or lower priority;

-	if highPriorityMeasRelax is configured with value true:

-	the UE may choose not to perform measurement for measurements of NR inter-frequencies cells of higher priority, or inter-RAT frequency cells of higher priority;

-	else:

-	if the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell, and, the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; or,

-	if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled:

-	if combineRelaxedMeasCondition is not configured:

-	the UE may choose to perform relaxed measurements for intra-frequency, NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of equal or lower priority according to relaxation methods in clauses 4.2.2.8, 4.2.2.9, and 4.2.2.10 in TS 38.133 [8];

-	if the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

-	the UE may choose to perform relaxed measurement for NR inter-frequencies cells of higher priority, or inter-RAT frequency cells of higher priority according to relaxation methods in clauses 4.2.2.8, 4.2.2.9, and 4.2.2.10 in TS 38.133 [8];

The above relaxed measurements and no measurement are not applicable for frequencies that are included in VarMeasIdleConfig, if configured and for which the UE supports dual connectivity or carrier aggregation between those frequencies and the frequency of the current serving cell.

End of change

# Annex – Text proposal – Clean version

#### 5.2.4.9	Relaxed measurement

##### 5.2.4.9.0	Relaxed measurement rules

When the UE is required to perform measurements of intra-frequency or NR inter-frequencies or inter-RAT frequency cells according to the measurement rules in clause 5.2.4.2:

-	if lowMobilityEvaluation is configured and cellEdgeEvaluation is not configured; and,

-	if the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and,

-	if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP:

-	the UE may choose to perform relaxed measurements for intra-frequency according to relaxation methods in clauses 4.2.2.8 in TS 38.133 [8];

-	if the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

-	if less than 1 hour have passed since measurements for cell (re-)selection were last performed; and,

-	if highPriorityMeasRelax is configured with value true:

-	the UE may choose not to perform measurement for measurements of NR inter-frequencies cells of higher priority, or inter-RAT frequency cells of higher priority;

-	else (i.e. the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ):

-	the UE may choose to perform relaxed measurements for NR inter-frequency or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.9, and 4.2.2.10 in TS 38.133 [8];

-	if cellEdgeEvaluation is configured and lowMobilityEvaluation is not configured; and,

-	if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled:

-	the UE may choose to perform relaxed measurements for intra-frequency cells according to relaxation methods in clauses 4.2.2.8 in TS 38.133 [8];

-	if the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

-	the UE may choose to perform relaxed measurements for NR inter-frequency or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.9, and 4.2.2.10 in TS 38.133 [8];

-	if both lowMobilityEvaluation and cellEdgeEvaluation are configured:

-	if the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and,

-	if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; and,

-	if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled:

-	if less than Thigher_priority_search (see clause 4.2.2.7 in TS 38.133 [8]) has passed since measurements for cell (re-)selection were last performed:

-	if highPriorityMeasRelax is not configured; and,

-	if the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

-	the UE may choose not to perform measurement for measurements of NR inter-frequencies cells of higher priority, or inter-RAT frequency cells of higher priority;

-	else, if less than 1 hour has passed since measurements for cell (re-)selection were last performed:

-	the UE may choose not to perform measurement for measurements of intra-frequency cells, NR inter-frequencies cells of equal or lower priority, or inter-RAT frequency cells of equal or lower priority;

-	if highPriorityMeasRelax is configured with value true:

-	the UE may choose not to perform measurement for measurements of NR inter-frequencies cells of higher priority, or inter-RAT frequency cells of higher priority;

-	else:

-	if the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell, and, the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; or,

-	if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled:

-	if combineRelaxedMeasCondition is not configured:

-	the UE may choose to perform relaxed measurements for intra-frequency, NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of equal or lower priority according to relaxation methods in clauses 4.2.2.8, 4.2.2.9, and 4.2.2.10 in TS 38.133 [8];

-	if the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

-	the UE may choose to perform relaxed measurement for NR inter-frequencies cells of higher priority, or inter-RAT frequency cells of higher priority according to relaxation methods in clauses 4.2.2.8, 4.2.2.9, and 4.2.2.10 in TS 38.133 [8];

The above relaxed measurements and no measurement are not applicable for frequencies that are included in VarMeasIdleConfig, if configured and for which the UE supports dual connectivity or carrier aggregation between those frequencies and the frequency of the current serving cell.