3GPP TSG-RAN WG2 Meeting#111-electronic     	                          R2-2006694

E-Meeting, 17th – 28th August 2020

Source: 	vivo

Title:  	DRX enhancement for RedCap UEs

Agenda Item:	8.12.3

Document for:	Discussion and Decision

Introduction

In RAN#86 Plenary, a new SID on the support of reduced capability NR devices has been approved [1]. The objectives regarding UE power saving and battery lifetime enhancement are listed below:

Study UE power saving and battery lifetime enhancement for reduced capability UEs in applicable use cases (e.g. delay tolerant) [RAN2, RAN1]: - Reduced PDCCH monitoring by smaller numbers of blind decodes and CCE limits [RAN1]. - Extended DRX for RRC Inactive and/or Idle [RAN2]. - RRM relaxation for stationary devices [RAN2].

In this contribution, we would like to provide our considerations on the extended DRX for RedCap UEs in RRC IDLE or INACTIVE.

Discussion

Existing DRX mechanism for UE in RRC IDLE or INACTIVE

DRX mechanism is introduced to control the activity of PDCCH monitoring, so that power saving can be achieved. The existing DRX mechanism for UE in NR RRC IDLE or INACTIVE is depicted as follows:

Figure 1: DRX mechanism for UE in NR RRC IDLE or INACTIVE

In NR, a UE in RRC IDLE or INACTIVE can determine the DRX cycle based on the default DRX parameter broadcasted in SIB1. If the UE wants to use UE specific DRX parameter, the UE shall include its preferred values in the registration request message sent to AMF. Then the AMF shall determine the accepted DRX parameter and respond to the UE with registration accept message. Moreover, for RRC INACTIVE UE, it may receive the UE-specific DRX parameter by the RRC Release message. After that, the UE can determine the DRX cycle based on all the received DRX parameters. The maximum DRX cycle length for the in RRC IDLE or INACTIVE is currently 2.56 seconds (i.e. 256 radio frames).

Motivation for extended DRX

Generally, Rel-15/16 NR is designed for moderate and high complexity devices, e.g. eMBB, URLLC, correspondingly, the requirements on UE PDCCH monitoring frequency is also moderate and high to shorten service latency and provide better scheduling flexibility, etc. For Rel-17 RedCap UEs, the power consumption feature becomes more and more critical. For wearable devices (e.g. smart watches, eHealth rings), the battery life of the device should be multiple days or even up to 1-2 weeks. For industrial wireless sensors, the battery life should last at least a few years. To achieve better UE power saving and significantly increase battery lifetime, extending the DRX cycle for RedCap UEs in RRC IDLE or INACTIVE is beneficial.

Considering that extended DRX with cycle up to 10.24s for CM-CONNECTED with RRC INACTIVE state has been introduced for 5GS in Rel-16, and the latency requirement of services for various RedCap UEs is not delay-tolerant (e.g. the transfer interval of industrial control application should be less than 100ms in most cases), we think that the maximum value of DRX cycle for RedCap UEs in RRC IDLE or INACTIVE should be 10.24s, which also helps to limit the impact on both UE and NW. The use case requiring DRX cycle longer than 10.24s needs to be further studied and identified. Thus, we propose that:

Proposal: Extend the DRX cycle up to 10.24s for RedCap UEs in RRC IDLE or INACTIVE.

Conclusions

In this contribution, we have provided our considerations on the extended DRX for RedCap UEs in RRC IDLE or INACTIVE. And we have the following proposal:

Proposal: Extend the DRX cycle up to 10.24s for RedCap UEs in RRC IDLE or INACTIVE.

- 4. Reference

[1] 3GPP RP-193238, “New study item: support of reduced capability NR devices”, Ericsson, RAN#86, Sitges, December 2019.