3GPP TSG-RAN WG2 #111e	Tdoc R2-2006700

Electronic meeting, August 17th – 28th, 2020

Agenda Item:	8.5.3

Source:	Ericsson

Title:	Uplink enhancements for URLLC in unlicensed controlled environments

Document for:	Discussion

# 1	Introduction

The work item on Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR [1] specifies the below as one objective:

Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]:

  - Specify support for UE-initiated COT for FBE with minimum specification effort

  - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

In this contribution, we mainly address how features related to UL configured-grant introduced in NR-U and URLLC can be harmonized to be applicable in particular in unlicensed controlled environments. For the other sub-objective on UE-initiated COT for FBE, we believe RAN2 impacts are RRC signalling to support features introduced in RAN1 (more details can be seen here [2]) and RAN2 can start the work in the later stage of the work item.

# 2	Background

In the following we briefly summarize the configurable features introduced in NR-U and URLLC and highlight their coupling and restrictions in configuration.

## 2.1	UE autonomous retransmissions based on cg-RetransmissionTimer (NR-U)

Given the possibility of LBT-failure occurrence, autonomous retransmissions on CG were introduced for a UE to be able to retransmit a TB using a later CG resource without waiting for HARQ A/N from the gNB. The cg-RetransmissionTimer therein determines the UE when to retransmit the TB. In general UE assumes “NACK” for a HARQ process, thus retransmits at timer expiry, unless a positive downlink feedback (DFI) for that HARQ process is received from gNB or a DCI carrying a grant for the same HARQ process associated with the TB. DFI was introduced for the gNB to provide explicit HARQ A/N. In addition, the UE autonomously selects HARQ process IDs in this mode of operation, and indicates them in uplink CG-UCI to the gNB. This is required since UE may autonomously select based on cg-RetransmissionTimer to retransmit on the sameHARQ process (as soon as possible after LBT success) or transmit new data on another HARQ process, and in order to differentiate HARQ processes received, gNB requires this information in UL.

cg-RetransmissionTimer: allows UE autonomous retransmission on CG when expired.

- RRC: Indicates the initial value of the configured retransmission timer (see TS 38.321 [3]) in multiples of periodicity. The value of cg-RetransmissionTimer is always less than the value of configuredGrantTimer. This field is always configured for operation with shared spectrum channel access together with harq-ProcID-Offset. This field is not configured for operation in licensed spectrum or simultaneously with harq-ProcID-Offset2.

- MAC: For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation select an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. The UE shall prioritize retransmissions before initial transmissions. The UE shall toggle the NDI in the CG-UCI for new transmissions and not toggle the NDI in the CG-UCI in retransmissions.

downlink feedback information (DFI): included in DCI to indicate acknowledgements for HARQ processes in order to be able to stop the cg-RetransmissionTimer. RRC-parameter cg-minDFI-Delay is introduced to control which HARQ processes are considered acknowledged, depending on time passed since transmission.

CG-uplink control information (UCI): had been enhanced to include the HARQ process ID (and RV and NDI) of the currently transmitting HARQ process, in order to allow UE selection of different HARQ processes for transmission or retransmission.

## 2.2	Multiple configured grant configurations (URLLC)

To better support TSC communication with multiple parallel periodical traffic flows, multiple CG configurations per BWP were introduced. In order to control the HARQ process IDs used per configuration, the following parameters were introduced – separately in URLLC and NR-U work items:

harq-ProcID-Offset: defines the range of HARQ process IDs the UE can choose from for a CG configuration.

- RRC: For operation with shared spectrum channel access, this configures the range of HARQ process IDs which can be used for this configured grant where the UE can select a HARQ process ID within [harq-procID-offset, .., (harq-procID-offset + nrofHARQ-Processes – 1)].

harq-ProcID-Offset2: defines the range of HARQ process IDs per CG-configuration to determine the HARQ process ID per symbol, according to the formula.

- RRC: Indicates the offset used in deriving the HARQ process IDs, see TS 38.321 [3], clause 5.4.1. This field is not configured for operation with shared spectrum channel access

- MAC: For configured uplink grants with harq-ProcID-Offset2, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation: HARQ Process ID = [floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

## 2.3	LCH-based intra-UE prioritization (URLLC)

In IIOT work item, intra-UE prioritization was introduced in MAC, in order to select/prioritize between overlapping grants depending on which logical channels (LCHs) are to be multiplexed on those grants. With lch-BasedPrioritization, the overlapping grants are considered prioritized or de-prioritized, and if de-prioritized, if autonomousTx is configured for a CG, an autonomous retransmission of the previously de-prioritized data is conducted on the next CG occasion with same HARQ process ID.

lch-BasedPrioritization (per MAC entity): controls whether LCH-based prioritized is used.

- RRC: If this field is present, the corresponding MAC entity of the UE is configured with prioritization between overlapping grants and between scheduling request and overlapping grants based on LCH priority, see TS 38.321 [3].

autonomousTx (per CG) cond lch-basedPrioritization: controls whether UE retransmits deprioritized data on next CG occasion.

- RRC: If this field is present, the Configured Grant configuration is configured with autonomous transmission, see TS 38.321 [3].

# 3	Discussion

From the above background description several aspects for harmonization of features and configurations for operation in licensed and shared spectrum become obvious.

First of all, we observe the following feature package in current specification:

- cg-RetransmissionTimer, UE-chooses HARQ ID, harq-ProcIdOffset, DFI, CG-UCI; conditioned to always and only on shared spectrum

With this conditioning, there is no possibility to use the shared spectrum with legacy CG-operation in which UE selects HARQ IDs according to formula and gNB controls retransmission behaviour. Supporting the feature set a) seems to be a burden on the UE implementation that is not negligible. It is noteworthy that those enhancements would only be needed in the worst-case scenario where significant interference is observed. This restriction completely ignores the possibility of operating NR-U in a controlled environment, where the LBT failures are negligible or even not present. This has been already considered in Rel-16 NR-U to motivate FBE channel access mechanism. FBE is mainly to be used in controlled environments where it can be guaranteed that external interference will not occur and LBT failure can be minimized if not completely avoided (see also [3]). Technically, allowing autonomous retransmission is not fundamentally needed in this scenario. In fact, if CG is configured exactly as defined in Rel-15, the system will operate well enough especially in an environment where external interference is not expected to occur. So, in these controlled environments, similar spectral properties, i.e. always LBT-success, may be established, and for URLLC traffic, mandatory usage of UE-triggered autonomous retransmissions are unnecessary and even come with latency disadvantages.

Furthermore, we think 3GPP specifications should enable a set of configuration options to choose from. It is then an implementation choice to use certain configurations in certain scenarios. We therefore propose to remove the coupling of the above configuration package a) to shared spectrum:

Remove the condition of NR-U CG features <cgRetxTimer, UE choosing HARQ ID, harq-ProcIDOffset, DFI, CG-UCI> to be coupled with shared spectrum.

This proposal means that, e.g., network does not have to configure cg-RetransmissionTimer for operation in shared spectrum or network can configure UE to follow the HARQ process ID formula in shared spectrum, also not using DFI, nor UCI.

Secondly, we observe that two harq-ProcID-Offset were introduced in Rel-16, in NR-U and URLLC/IIOT respectively. Generally, the two fields serve the same purpose, i.e. to determine the range of HARQ process IDs per CG-configuration, which is necessary in case of multiple CG configurations. In the specification, however, the two fields are used to differentiate UE-behaviour regarding following the formula to determine the HARQ process ID or choosing the HARQ process IDs themselves, as it becomes obvious from the following table:

Table 1: HARQ process ID determination.

|  | harqProcOffset2 configured | harqProcOffset2 not configured |
| --- | --- | --- |
| harq-ProcID-Offset configured | Not allowed acc. to RRC restriction for cg-RetransmissionTimer | UE selects (NR-U way) |
| harqProcID-Offset not configured | Formula with offset (IIOT way) | Formula w/o offset (legacy) |

The harq-ProcID-Offset is always configured together with the cg-RetransmissionTimer, so that conditioning of whether the UE chooses the HARQ proc ID or whether it’s selected according to the formula, can actually be set depending on the configuration of cg-RetransmissionTimer. A single harmonized harqProcOffset could then be used to define the range of HARQ IDs to choose/consider in formula, since the two parameters cannot be simultaneously configured and the purposes are exactly the same. Consequently, RRC specification would be much clearer.

Harmonize harq-ProcID-Offset and harq-ProcID-Offset2 usage.

# 4	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Remove the condition of NR-U CG features <cgRetxTimer, UE choosing HARQ ID, harq-ProcIDOffset, DFI, CG-UCI> to be coupled with shared spectrum.

Proposal 2	Harmonize harq-ProcID-Offset and harq-ProcID-Offset2 usage.

# 5	References

RP-201310, Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, 3GPP RAN#88e, June 29 - July 3, 2020.

R1-2005515, Enhancements for IIoT/URLLC on Unlicensed Band, Ericsson

R1-2003846, Configured Grant Enhancements, RAN1#101-e, Ericsson