3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2006790

Online, August 17th - 28th, 2020

Agenda Item:	8.9.2

Source:	OPPO

Title:	Paging enhancement for power saving

Document for:	Discussion, Decision

Introduction

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1]

  - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1]

- NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting

  - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1]

- NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4]

  - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1]

- NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized.

  - Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4]

- NOTE: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS

In RAN#86 meeting, a new WID RP-193173 for UE power saving enhancement was approved. In the WID, the object of UE power saving includes the following.

One of the objectives is paging enhancement for idle/inactive-mode UE to reduce unnecessary UE paging receptions. In this paper, we discuss on this aspect.

Discussion

In NR, a UE may use DRX in idle/inactive-mode for paging reception for the purpose of power saving. The UE monitors one PO (paging occasion) per DRX cycle. A PO consists ‘S’ consecutive PDCCH monitoring occasions, where ‘S’ is the number of SSBs transmitted by network. The UE assumes the same paging message is repeated in all the ‘S’ beams and the selection of the beam(s) for paging reception is up to UE implementation.

Using DRX for paging reception is beneficial for UE power saving.

UE determines its PF (paging frame) and PO based on UE_ID, the DRX cycle, the PF number in a DRX cycle and the PO number in a PF. For paging message reception in the coming PO, the UE needs to follow the below steps in order:

- SSB based synchronization before the PO for reception paging PDCCH and PDSCH

- P-RNTI scrambled PDCCH monitoring in the PO

- If a P-RNTI scrambled PDCCH is received, decode the PDCCH

- Receive PDSCH indicated by the PDCCH

- Decode PDSCH and process the paging message on PDSCH to check whether the UE identity is included in the pagingRecordList in paging message.

A single PO can be shared by multiple UEs, thus if network wants to page only one UE in the PO, all the other UEs in the same PO need to monitor the paging PDCCH and the corresponding paging PDSCH. After finishing all the steps above, those UEs can realize thatthe paging message is not intended for them, which results in false alarm paging for those UEs. Obviously, false alarm paging causes unnecessary paging reception, which increases UE power consumption.

False alarm paging causes unnecessary paging reception, which increases UE power consumption.

The impact of false alarm paging on UE power consumption depends on the false alarm probability, which further depends on the number of UEs sharing the same PO and the paging rate of these UEs. The more UEs share the same PO, the higher the false alarm probability will be.

The false alarm probability depends on the number of UEs which share the same PO and the paging rate of these UEs.

In order to reduce the false alarm probability, one straightforward way is to further divide the UEs associated with the same PO into several groups, and use group-based paging. More specifically, following options can be considered.

- Option 1: Indicate UE group information in the paging DCI

- Option 2: Configure separate time/frequency resources for paging PDCCH reception for different UE groups

For Option 1, the UE still needs to perform step 1 to 3. If UE detects that the UE group which the UE belongs to is not paged based on the DCI indication, the UE may skip the following step 4 and 5. This option can reduce unnecessary paging PDSCH receptions.

For Option 2, the UE needs to perform step 1and 2, if the UE group which the UE belongs to is not paged, the UE will not detect paging PDCCH, then the UE may skip the following step 3 to 5. Furthermore, in the massive UE scene, option 2 could increasing the paging capacity without increase on false alarm probability.

UE group-based paging can reduce unnecessary paging reception.

Another way is to introduce a “wake-up” signal before the PO to indicate whether UE needs to further monitor PO. In Rel-15 LTE, sequence-based wake-up signal, i.e. WUS, was introduced for idle mode paging for NB-IoT and eMTC UE. WUS configuration is provided in system information, and UE which supports WUS shall monitor WUS located before its PO. If the UE detects WUS, the UE will monitor the following PO(s). If the UE does not detect WUS, the UE is not required to monitor the following PO(s). In Rel-16 NB-IoT and eMTC, WUS was further enhanced and GWUS (group wake up signal) has been introduced. When GWUS is used, multiple WUS groups distributed over multiple WUS resources are configured by network. A UE supporting GWUS selects one WUS group based on its UE paging probability information and /or its UE ID. A common WUS group may be used to wake up all UEs monitoring the same WUS resource.

The power saving gain is significant for NB-IoT and eMTC UE because WUS could reduce unnecessary paging receptions with large repetitions which may last for a long period.

WUS-based paging for NB-IoT and eMTC UEs is useful for power saving since WUS could reduce unnecessary paging receptions with large repetitions which may last for a long period.

In R16 NR power saving WI, PDCCH-based WUS has been introduced for C-DRX, which is used to indicate UE whether to start drx-onDurationTimer to monitor PDCCH in the next Long DRX cycle. The power saving gain could be expected since UE does not need to monitor PDCCH during drx-OnDuration which will consume UE power if network does not expect to schedule the UE.

WUS for C-DRX is useful for power saving since it could reduce unnecessary PDCCH monitor which will consume UE power.

In NR, for idle/inactive-mode paging, the similar mechanism could be considered. The UE in idle/inactive-mode needs to wake up for synchronization for paging reception before its PO. Considering PDSCH reception may be more sensitive to synchronization requirement than PDCCH reception, the UE with poor channel quality may need to use multiple SS bursts for synchronization for both paging PDCCH and paging PDSCH reception before the PO. If WUS is used for idle/inactive-mode paging, in case network does not intend to page the UE, the UE will know it upon WUS reception and then can go to sleep immediately so that the unnecessary synchronization for paging PDSCH reception could be avoided. The power saving gain of WUS should be evaluated by RAN1.

WUS-based paging for NR UEs may reduce unnecessary synchronization and paging reception.

Comparison of candidate paging enhancements is given in the table below.

Table 1 summary of candidate paging enhancements

| Candidate paging enhancements | Advantage | RAN1 impacts | RAN2 impacts |
| --- | --- | --- | --- |
| UE group-based paging with UE group indication in paging DCI | May reduce unnecessary paging PDSCH reception | PDCCH DCI design | UE grouping method UE group-based paging procedure |
| UE group-based paging with separate paging time/frequency resource configuration for different UE groups | May reduce unnecessary paging PDCCH and PDSCH reception | N/A | UE grouping method Paging resource configuration UE group-based paging procedure |
| WUS-based paging | May reduce unnecessary synchronization and paging PDCCH&PDSCH reception | WUS design WUS based paging procedure | UE grouping method WUS-based paging procedure |

RAN2 discuss the following candidate paging enhancements.

- Option 1: UE group-based paging with UE group indication in paging DCI

- Option 2: UE group-based paging with separate paging time/frequency resource configuration for different UE groups

- Option 3: WUS-based paging

Conclusion

Based on the discussion we make the following observation:

- Using DRX for paging reception is beneficial for UE power saving.

False alarm paging causes unnecessary paging reception, which increases UE power consumption.

The false alarm probability depends on the number of UEs which share the same PO and the paging rate of these UEs.

UE group-based paging can reduce unnecessary paging reception.

WUS-based paging for NB-IoT and eMTC UEs is useful for power saving since WUS could reduce unnecessary paging receptions with large repetitions which may last for a long period.

WUS for C-DRX is useful for power saving since it could reduce unnecessary PDCCH monitor which will consume UE power.

WUS-based paging for NR UEs may reduce unnecessary synchronization and paging reception.

And we give the following proposals:

- RAN2 discuss the following candidate paging enhancements.

- Option 1: UE group-based paging with UE group indication in paging DCI

- Option 2: UE group-based paging with separate paging time/frequency resource configuration for different UE groups

- Option 3: WUS-based paging